| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup for the AAI_DSx MAC Control Message Supporting E-MBS Operation in IEEE P802.16m (16.2.3.45) |  |
| Date Submitted | 2010-05-11 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D5,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #31a: Announcement, IEEE 802.16-10/0032 |  |
| Abstract | Cleanup for the AAI_DSx message to support E-MBS on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Cleanup for the AAI_DSx MAC Control Message Supporting E-MBS Operation in IEEE P802.16m (16.2.3.45)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

This contribution clarifies following AAI_DSx messages in the IEEE 802.16m Amendment Draft Standard [1] to support E-MBS clearly.

- AAI_DSA-REQ

- AAI_DSA-RSP

- AAI_DSC-REQ

- AAI_DSC-REQ

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Modify the Table 740 in 16.2.3.45.1 of 802.16m/D5 as follows]

Table 740—AAI_DSA-REQ

| M/O |  |  | Attributes/Array of attributes |  |  | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ..... |  |  | ……….. |  |  | .... | ……… | ……….. |
| O |  |  | MBS Service TLV |  |  | 8 | 0: No available MBS 1: MBS in Serving BS Only 2: MBS in a multi-BS Zone supporting macro-diversity 3: MBS in a multi-BS Zone not supporting macro-diversity 4-255: Reserved |  |
| O |  |  | MBS Zone Identifier Assignment parameter |  |  | 8 | MBS zone identifier (bit 6 through 0 are the MBS Zone Identifier, bit 7 is set to 0) |  |
| ..... |  |  | ………. |  |  | ..... | …………. | ……….. |
| O |  |  | E-MBS Service |  |  | 12 | Indicates whether the MBS service is being requested or provided for the connection that is being setup. 0: No available E-MBS 1: E-MBS in Serving ABS Only 2: E-MBS in a multi-ABS Zone supporting macro-diversity 3: E-MBS in a multi-ABS Zone not supporting macro-diversity | included both ABS/AMS initiated DSA-REQ |
| O |  |  | E-MBS_Zone_ID |  |  | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| O |  |  | E-MBS Service Flow Parameter |  |  | Variable | Mapping of E-MBS ID and FID |  |
| O |  |  | Physical Carrier Index |  |  | 6 | Target carrier which the AMS switches or is redirected by ABS to, only included in ABS initiated DSA-REQ | Only included in ABS initiated DSA-REQ |
| O | M |  | E-MBS Service |  |  | 3 | Indicates whether the MBS service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in Serving ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. | Present when both ABS/AMS initiates DSA-REQ |
|  | O | M | Num of E-MBS Zone ID |  |  | 3 | Number of E-MBS Zone ID to add | Present when ABS initiates DSA-REQ and at least one bitmap(Bit#0~#2) of E-MBS Service is set to 1 |
|  | O | M | fullNumE-MBSIdArr[1..8] | E-MBS_Zone_ID |  | 7 | Indicates an E-MBS Zone ID where the connection for associated service flow is valid |  |
|  |  | O |  | Physical Carrier Index |  | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the E-MBS Zone ID is served on a different carrier from the current carrier. |
|  |  | M |  | Number of E-MBS ID_FID Mappings (M) |  | 4 | Number of E-MBS ID_FID Mappings in the E-MBS_Zone_ID to add |  |
|  |  | M |  | fullEMBSIdFidMappingArr[0...16] | E-MBS ID and FID | Variable(16xN) | Mapping of E-MBS ID and FID. Based on the value of Number of E-MBS ID_FID Mappings |  |

[Remedy2: Modify the Table 741 in 16.2.3.45.2 of 802.16m/D5 as follows]

Table 741—AAI_DSA-RSP

| M/O |  |  | Attributes/Array of attributes |  |  | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M |  |  | Control Message Type |  |  | 8 |  | N.A. |
| M |  |  | FID Change Count |  |  | 1 | FID Change Count from corresponding the AAI_DSA-REQ message | N.A. |
| O |  |  | E-MBS Service |  |  | 2 | Indicates whether the MBS service is being requested or provided for the connection that is being setup. 0: No available E-MBS 1: E-MBS in Serving ABS Only 2: E-MBS in a multi-ABS Zone supporting macro-diversity 3: E-MBS in a multi-ABS Zone not supporting macro-diversity | included both ABS/AMS initiated DSA-REQ |
| O |  |  | E-MBS_Zone_ID |  |  | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | Only included in ABS initiated DSA-REQ |
| O |  |  | E-MBS Service Flow Parameter |  |  | Variable | Mapping of E-MBS ID and FID |  |
| O |  |  | Physical Carrier Index |  |  | 6 | Target carrier which the AMS switches or is redirected by ABS to |  |
| O | M |  | E-MBS Service |  |  | 3 | Indicates whether the MBS service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in Serving ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. | Present when both ABS/AMS initiates DSA-REQ |
|  | O | M | Num of E-MBS Zone ID |  |  | 3 | Number of E-MBS ID to add | Present when at least one bitmap(Bit#0~#2) of E-MBS Service is set to 1 |
|  | O | M | fullNumE-MBSIdArr[1..8] | E-MBS_Zone_ID |  | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
|  |  | O |  | Physical Carrier Index |  | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the E-MBS Zone ID is served on a different carrier from the current carrier. |
|  |  | M |  | Number of E-MBS ID_FID Mappings (M) |  | 4 | Number of E-MBS ID_FID Mappings in the E-MBS_Zone_ID to add |  |
|  |  | M |  | fullEMBSIdFidMappingArr[0...16] | E-MBS ID and FID | Variable(16xN) | Mapping of E-MBS ID and FID. Based on the value of Number of E-MBS ID_FID Mappings |  |

[Remedy3: Modify the Table 743 in 16.2.3.45.4 of 802.16m/D5 as follows]

Table 743—AAI_DSC-REQ

| M/O |  |  | Attributes/Array of attributes |  |  | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ..... |  |  | ……….. |  |  | .... | ……… | ……….. |
| O |  |  | MBS Service TLV |  |  | 8 | 0: No available MBS 1: MBS in Serving BS Only 2: MBS in a multi-BS Zone supporting macro-diversity 3: MBS in a multi-BS Zone not supporting macro-diversity 4-255: Reserved |  |
| O |  |  | MBS Zone Identifier Assignment parameter |  |  | 8 | MBS zone identifier (bit 6 through 0 are the MBS Zone Identifier, bit 7 is set to 0) |  |
| ..... |  |  | ………. |  |  | ..... | …………. | ……….. |
| O |  |  | E-MBS_Zone_ID |  |  | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| O |  |  | E-MBS Service Flow Parameter |  |  | Variable | Mapping of E-MBS ID and FID |  |
| O |  |  | Physical Carrier Index |  |  | 6 | Target carrier which the AMS switches or is redirected by ABS to, only included in ABS initiated DSA-REQ | Only included in ABS initiated DSC-REQ |
| O | M |  | Num of E-MBS Zone ID to update |  |  | 3 | Number of E-MBS ID to update | Present when ABS initiates DSC-REQ |
|  | O | M | fullNumE-MBSIdtoUpdateArr[1..8] | Service_flow_update_indicator |  | 1 | Indicator whether "bitmap + new service flow" or "current service flow + new service flow" is included. 0b0: "Service Flow Update Bitmap + New_E-MBS ID and FID" 0b1: "Current_ E-MBS ID and FID and New_ E-MBS ID and FID Mappings" | Present only if Num of E-MBS Zone ID to update > 0 |
|  |  | M |  | E-MBS_Zone_ID |  | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
|  |  | O |  | New E-MBS_Zone_ID |  | 7 | Indicates an E-MBS zone to update | Present only if the E-MBS Zone ID needs to update |
|  |  | O |  | Physical Carrier Index |  | 6 | The physical carrier index that is support for the new E-MBS Zone ID to update | Present only if the New E-MBS Zone ID to update is served on a different carrier from the previous one |
|  |  | O |  | Service Flow Update Bitmap |  | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. | Presented only if Service_flow_update_indicator is set to 0b0. |
|  |  | O |  | fullServiceFlowUpdateBitmapArr[0…16] | New_E-MBS ID and FID | Variable(16xN) | New E-MBS IDs and FIDs corresponding to the Service Flow Update Bitmap to update where N is the number ones in the Service Flow Update Bitmap. |  |
|  |  | O |  | Number of E-MBS ID_FID Mappings (M) |  | 4 | Number of E-MBS ID_FID Mappings in the E-MBS_Zone_ID to update | Presented only if Service_flow_update_indicator is set to 0b1. |
|  |  | O |  | fullEMBSIdFidMappingArr[0...16] | Current_E-MBS ID and FID and New_E-MBS ID and FID | Variable(32xM) | Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update. Based on the value of Number of E-MBS ID_FID Mappings |  |

[Remedy4: Modify the Table 746 in 16.2.3.45.7 of 802.16m/D5 as follows]

Table 743—AAI_DSD-REQ

| M/O |  | Attributes/Array of attributes |  | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| ..... |  | ……….. |  | .... | ……… | ……….. |
| O |  | E-MBS_Zone_ID |  | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| O |  | E-MBS Service Flow Parameter |  | Variable | Mapping of E-MBS ID and FID |  |
| O | M | E-MBS_Zone_ID |  | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | Present when ABS initiates DSD-REQ |
|  | M | Number of E-MBS ID_FID Mappings |  | 4 | Number of E-MBS ID_FID Mappings in the E-MBS_Zone_ID to delete |  |
|  | M | fullEMBSIdFidArr[0..16] | E-MBS ID and FID | Variable(16xN) | Mapping of E-MBS ID and FID. Based on the value of Number of E-MBS ID_FID Mappings |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” April 2010