| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the Unicast and E-MBS Region Configuration in IEEE P802.16m (16.9.3.1 & 16.2.3.30) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D5,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #31a: Announcement, IEEE 802.16-10/0032 |  |
| Abstract | E-MBS Configuration in AAI_SCD/AAI_E-MBS-CFG messages supporting E-MBS on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on the Unicast and E-MBS Region Configuration in IEEE P802.16m (16.9.3.1 & 16.2.3.30)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS).

According to the IEEE 802.16m Amendment Draft Standard [1], some E-MBS Configuration Parameters transmitted via AAI_SCD message have been proposed as follows.

- Zone_Allocation-Bitmap

- ZF

- MSI Length(NMSI)

- E-MBS AAI frame offset

Although those parameters are essential for the E-MBS operation, any E-MBS AMSs should receive the AAI_E-MBS-CFG message. Any unicast AMS may refer the Zone Allocation Bitmap and ZF. However, MSI length and E-MBS AAI frame offset are not essential for those AMSs.

Therefore, in this contribution, MSI length and E-MBS AAI frame offset are moved to AAI_E-MBS-CFG message from AAI_SCD message and ZF and Zone_Allocation-Bitmap are modified more efficiently.

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Delete the following rows in Table 717-AAI_SCD Message of 802.16m/D5]

Table 717 – AAI_SCD Message

| M/O |  | Attributes/Array of attributes |  | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| … |  | ... |  | … | … | … |
| O |  | E-MBS Configuration Parameters | Zone_Allocation-BitMap | DSACMAX-1 | See Section 16.9.3.1 |  |
|  | M |  | ZF | 12 | Zone Flag bit. Indicates the use of the last zone. 0b00: some last subbands are used for Unicast 0b01: all subbands are used for E-MBS 0b10: all subbands are used for unicast See section 16.9.3.1 | Present when E-MBS is configured |
|  | O |  | # of subbands for unicast (UNsubband) | variable | Number of subbands for unicast zone and the size of UNsubband is as below. 5bits for 20MHz 4bits for 10MHz 2bits for 5MHz | Present when ZF == 0b00 |
|  |  |  | MSI | 2 | The Length of an MSI in units of the number of superframes 0b00: 2 superframes, 40ms (NMSI =2) 0b01: 4 superframes, 80ms (NMSI=4) 0b10: 8 superframes, 160ms (NMSI=8) 0b11: 16 superframes, 320ms (NMSI=16) |  |
|  |  |  | E-MBS AAI frame offset | variable | The location of the AAI frame where the E-MBS data burst ends MSI length == 0b00: 3bits MSI length == 0b01: 4bits MSI length == 0b10: 5bits MSI length == 0b11: 6bits |  |
| … |  | ... | … | … | … | … |

[Remedy2: Modify the Table 974-AAI_E-MBS-CFG message format in 16.9.3.1 of 802.16m/D5]

Table 974—AAI_E-MBS-CFG message Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI-E-MBS-CFG_Message_Format() { |  |  |
| MAC Control Message Type | 8 | AAI_E-MBS-CFG |
| E-MBS_CFG_LIFETIME (m) | 4 | Indicates the duration of E-MBS_CFG_LIFETIME for which the E-MBS configuration information of the zone do not change. Duration of E-MBS_CFG_LIFETIME: 16(m+1) superframes |
| Applied SCD Change configuration count | 4 | Applied Change Configuration Count for the unicast/multicast zone |
| Zone Allocation Bit-Map | DSACMAX-1- UNsubband | Indicates the zone configuration. The size of DSACMAX is as below. 20bits for 20MHz 9bits for 10MHz 3bits for 5MHz |
| for (i = 0; i< Num_E-MBS_Zones; i++) { |  |  |
| E-MBS_Zone_ID | 7 | The EMBS_Zone_ID to which this EMBS MAP applies. |
| MSI Length (NMSI) | 2 | The Length of an MSI in units of the number of superframes 0b00: 2 superframes, 40ms (NMSI =2) 0b01: 4 superframes, 80ms (NMSI=4) 0b10: 8 superframes, 160ms (NMSI=8) 0b11: 16 superframes, 320ms (NMSI=16) |
| E-MBS MAP Resource Index | 11 | Resource index includes location and allocation size. |
| E-MBS MAP ISizeOffsetISizeOffset | 5 | Offset used to compute burst size index |
| E-MBS ID and FID Mappings List inclusion | 1 | Indicates whether E-MBS ID and FID Mapping list is included or not 0b0: No Zone boundary or Zone boundary but no inter-zone service continuity in neighbor ABS 0b1: Zone boundary and inter-zone service continuity |
| if (E-MBS ID and FID Mappings List inclusion ==1) { |  |  |
| E-MBS ID and FID Mappings List | Variable | E-MBS ID and FID Mappings List between serving and neighbor E-MBS Zones |
| } |  |  |
| } |  |  |
| } |  |  |

Applied SCD Change Configuration Count: Applied Change Configuration Count consists of the change count of AAI_SCD message for the unicast/multicast zone. If an AMS receives AAI_SCD with new configuration count (n+1) but the applied SCD Change Configuration Count(n), the AMS should preserve the previous configuration of AAI_SCD till receives the same applied SCD Change Configuration Count(n+1) in order to apply the configuration of AAI_SCD as well as AAI_E-MBS-CFG message.

Zone_Allocation Bit-MAP: Zone_Allocation Bit-MAP consists of sub-band indices reserved for all E-MBS zones the BS belongs to. The Zone Allocation Bit-MAP in the AAI-SCDAAI-E-MBS-CFG message identifies the use of the resource comprising a set of contiguous subbands in a DL AAI subframe. The Bit-MAP determines the size (S) of each E-MBS zone in number of contiguous SLRUs in frequency domain within the subframe, the index (L) of each E-MBS zone from where the allocated zone begins and the total number (Num_E-MBS_Zones) of allocated zones.

The Zone Allocation Bit-MAP is constructed using the following rules:

- If the use of the resource is changed between one sub-band and the next sub-band, the bit '1' is set in the location the same as the index of the sub-band in the Bit-MAP.

- Otherwise, the bit '0' is set in the Bit-MAP.

For a given system bandwidth with maximum number of subbands, DSACmax = 21, 10, 4 for 20, 10 and 5 MHz respectively, the size of the Zone Allocation Bit-MAP is (DSACmax -1-UNsubband). UNsubband is the number of subbnads for the unicast when ZF is ‘0b00’in AAI_SCD message. UNsubband is zero when ZF is ‘0b01’ in AAI_SCD message. Let the total number of the bit '1' in the Bit-MAP be denoted as K and the index in which the bit '1' in the Bit-MAP is located be denoted as Ji, where i is the E-MBS zone index which is from 1 to Num_E-MBS_Zones. J0 is fixed to 0.

Num_E-MBS_Zone = K+1

Li = 	Ji-1 + 1

Si = 	Ji-Ji-1, 				1 ≤ i ≤ K

DSACmax –UNsubband - JK, 	i=K+1

For example, consider the case where the Zone Allocation Bit-MAP field = 0000100100000100 and UNsubbnad is 2. The system bandwidth is 10MHz and DSACmax is 10. This is illustrated pictorially below in Figure 603. In this case, ZF=0b00 because the last zone is the unicast zone. Num_E-MBS_Zones = 2, Li = {1, 6}, Si = {5, 3} from K = 21, Ji = {0, 5, 8}. Hence, the number of zones is 2. Each index of two zones is 1, 6 respectively and each size of two zones is 5, 23 respectively.

Figure 603 – Zone Allocation Bit-MAP for 10MHz

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” April 2010