| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to disambiguate the initial vector of PRBS for Subcarrier Randomization for IEEE 802.16m (D5-16.3.11.3.1) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Yu-Chuan Fang, Zheng Yan-Xiu, Chang-Lan Tsai, Chung-Lien Ho ITRI | E-mail: zhengyanxiu@itri.org.tw |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “16.3.11.3.1” |  |
| Abstract | The contribution clarifies the bit mapping order of the initial vector for PRBS of subcarrier randomization in IEEE 802.16m/D5. |  |
| Purpose | To be discussed and adopted into P802.16m/D5 by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to disambiguate the initial vector of PRBS for Subcarrier Randomization for IEEE 802.16m (D5-16.3.11.3.1)

Yu-Chuan Fang, Zheng Yan-Xiu, Chang-Lan Tsai, Chung-Lien Ho

ITRI

# Introduction

In the PRBS for data randomization section (16.3.11.3.1) of P802.16m/D5 [1], the initial vector is composed of 10 bits of IDcell and 5 one-bits. The first 10 bits b9…b0 shall be assigned the IDcell from MSB to LSB. However, the bit mapping order between the first 10 bits b0…b9 and IDcell is not clarified. This contribution proposes change text to disambiguate the initial vector of PRBS for data randomization..

# References

[1]  IEEE P802.16m/D5, “Draft Amendment to IEEE standard for local and metropolitan area networks: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” Apr. 2010.

[2]  IEEE C802.16m-10/0276, “Proposed Text for Initial State of Data Randomization Scheme (Section 16.3.11 of the IEEE 802.16m/D4),” Mar. 2010

# Text proposal for the 802.16m amendment

The proposed changes are described as the following format.

<To suggest motions to fix up text>

======================= Start of Proposed Text #xxx ==============================

Proposed text

Instruction to editor:

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

======================= End of Proposed Text #xxx ==============================

# Proposed Change  (page 760)

<Modify the sentences at lines 33-43 of page 760 as follows:>

============================== Start of Proposed Text =================================

The initialization vector of the PRBS generator for both DL and UL shall be designated b14...b0 so that

b0b9…b9b0 = 10 bits of IDcell form MSB to LSB, where IDcell is corresponds to SA-Preamble (in the range 0 to 767)

b10 b14…b14 b10 = 1, the values of these bits are always set to 1 for avoiding initial state of all zeros.

The value of the subcarrier randomization sequence, on subcarrier k, are derived from bit wk+n, where n = mod(7*×m + f, 64), m = OFDMA symbol number within a frame (the first symbol in frame is indexed 0) and f = frame number within a superframe (in the range 0 to 3). Subcarrier randomization sequence shall be generated for every subcarrier up to the highest numbered usable subcarrier, in order of physical subcarriers, including the DC subcarrier and usable subcarriers that are not allocated. The first usable subcarrier is indexed 0. The output bits of PRBS shall be counted from zero.

============================== End of Proposed Text =================================