| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial Cleanup on Channel Coding and HARQ for IEEE 802.16m (D5-16.3.11) |  |
| Date Submitted | 2010-05-10 |  |
| Source(s) | Yu-Chuan Fang, Zheng Yan-Xiu, Chang-Lan Tsai, Chung-Lien Ho ITRI | E-mail: zhengyanxiu@itri.org.tw |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “16.3.11” |  |
| Abstract | This contribution proposes text modification to clean up the channel coding and HARQ section for IEEE 802.16m/D5. |  |
| Purpose | To be discussed and adopted into P802.16m/D5 by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Editorial Cleanup on Channel Coding and HARQ for IEEE 802.16m (D5-16.3.11)

Yu-Chuan Fang, Zheng Yan-Xiu, Chang-Lan Tsai, Chung-Lien Ho

ITRI

# Introduction

This contribution proposes some cleanup text for Channel Coding and HARQ section (16.3.11) of P802.16m/D5 [1]. We suggest modifying some words, sentences, notations and style to keep consistency throughout the section and removing ambiguous points to make readers understand the text easily.

# References

[1]  IEEE P802.16m/D5, “Draft Amendment to IEEE standard for local and metropolitan area networks: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” Apr. 2010.

# Text proposal for the 802.16m amendment

The proposed changes are described as the following format.

# Proposed Change #xxx  (page yyy)

- [Reason 1]

- …

<To suggest motions to fix up text>

======================= Start of Proposed Text #xxx ==============================

Proposed text

Instruction to editor:

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

======================= End of Proposed Text #xxx ==============================

Note that some proposed changes don’t show the corresponding proposed text but just describe the comments of suggested modification. Please do as the proposed motions, e.g., <Widen the field for index=37 at lines 34-41 in Table 935 of page 683.>.

# Proposed Change #1  (page 749)

- The style of parameters and notations, like NDB, NFB, idx, etc., should be consistent throughout the standard.

<Modify the notations at line 9 of page 749 as follows:>

<Modify the fonts of idx, NDB, KFB at line 17 in Table 956 of page 749.>

============================== Start of Proposed Text #1 ===============================

# 16.3.11.1.2 Burst partition

Only the burst sizes NDB listed in Table 956 are supported in the PHY layer. These sizes include the addition of CRC (per burst and per FEC block) when applicable. Other sizes require padding to the next burst size. When the burst size including 16 burst CRC bits for a data burst and/or FEC blocks exceeds the maximum FEC block size, NFB_MAXFB_MAX, the burst is partitioned into KFBFB FEC blocks.

Table 956—Burst Size

| idx idx | NDBDB (byte) | KFBFB | idx idx | NDBDB (byte) | KFBFB | idx idx | NDBDB (byte) | KFBFB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 6 | 1 | 23 | 90 | 1 | 45 | 1200 | 2 |

… (omitted).

============================== End of Proposed Text #1 ===============================

# Proposed Change #2  (page 750)

- To make the style consistent.

<Modify the sentences at lines 43 of page 750 as follows:>

<Modify the fonts of NFB and NDB at lines 59-65 of page 750;>

============================== Start of Proposed Text #2 ===============================

The modulation order Nmodmod (2 for QPSK, 4 for 16-QAM and 6 for 64-QAM) depends on the parameter ISizeOffset according to Table 933. Allocation size of 1 or 2 LRUs are special cases (separate columns in the table). For allocation size of at least 3 LRUs the modulation order depends only on ISizeOffset. The allocation size and the value of ISizeOffset are set by the ABS scheduler, which takes into account the resulting modulation order and effective code rate, and according to the link adaptation.

If a burst is partitioned into more than one FEC block, each partitioned FEC block has the same size. The size of the FEC encoder input is denoted by NFB NFB. The set of supported FEC encoder input sizes including FEC block CRC when applicable is the subset of the burst size table, i.e., NDB NDB of idx idx from 1 to 39 in Table 956.

The burst size NDB NDB including burst CRC and FEC block CRC is defined by Equation (303):

…… (omitted)

============================== End of Proposed Text #2 ===============================

# Proposed Change #3  (page 751)

- To make the style consistent.

- In the equation, the variables are too close together. The product operator may be inserted to make text readable.

<Modify the fonts of Nmod at line 3 in Table 958 of page 751.>

<Clarify Equation 304 as >

<Modify the fonts of IMFB, NFB-CRC, NDB-CRC and KFB at lines 34-39 of page 751.>

============================== Start of Proposed Text #3 ===============================

Table 958—Rules for modulation order

| ISizeOffset | Nmod Nmod (allocation size > 2) | Nmod Nmod (allocation size = 2) | Nmod Nmod (allocation size = 1) |
| --- | --- | --- | --- |
| 0 ~ 9 | 2 | 2 | 2 |

……

(304)

(304)

where:

IMFBMFB equals 0 when KFBFB = 1, 1 when KFBFB > 1

NFB-CRCFB-CRC equals 16, which is the size of the FEC block CRC

NDB-CRCDB-CRC equals 16, which is the size of the burst CRC.

The burst partition block generates KFBFB FEC blocks, with each FEC block processed by the randomization block as described in 16.3.11.1.3.

============================== End of Proposed Text #3 ===============================

# Proposed Change #4  (page 752)

- To make the style consistent.

- gFB-CRC(D) is not shown properly.

<Modify the fonts of KFB at line 27 of page 752.>

<Modify  at line 49 of page 752, since the parameter, , does not display properly.>

============================== Start of Proposed Text #4 ===============================

# 16.3.11.1.4 FEC block CRC encoding

The burst partition procedure generates KFBFB FEC blocks for each burst.

…… (omitted)

yields a remainder equal to 0 when divided by .

…… (omitted)

============================== End of Proposed Text #4 ===============================

# Proposed Change #5  (page 753)

- To make the style consistent.

< Modify NFB at lines 9-11 of page 753.>

< Modify  at line 55 of page 753.>

============================== Start of Proposed Text #5 ===============================

…… The encoder is fed by blocks of NFBFB bits or N couples (NFBFB = 2N bits).

…… (omitted, jump to line 55)

First, the encoder (after initialization by the circulation state Sc1, see 8.4.9.2.3.) is fed the sequence ……

============================== End of Proposed Text #5===============================

# Proposed Change #6  (page 754)

- To make the style of A, B, Y1, Y2, W1, W2 consistent with Figure 575 instead of italic A, B, Y1, Y2, W1, W2.

- P0, P1, P2 and P3 are incorrectly typed as P0, P1, P2, and P3.

- In table 959, the NFB is shown in bits but NDB is shown in byte in Table 956. We should note this in the text and table.

< Modify the notations, A, B, Y1, Y2, W1, W2, at line 3 of page 754 for consistency with Figure 575. >

< Modify P0, P1, P2 and P3 at line 12 of page 754, since the numbers should be subscript as follows.>

< Modify P0, P1, P2 and P3 and NFB at line 21 in Table 959 of page 754 for the consistent font as follows. >

============================== Start of Proposed Text #6 ==============================

……

AA,BB,YY1,YY2,WW1,WW2 =

A0,A1,A2…,AN-1,……

……

The CTC interleaver requires the parameters P0, P1, P2, and P3 P0, P1, P2 and P3 shown in Table 934 where the value of NFB is shown in bit.

The detailed interleaver structures except table for interleaver parameters correspond to 8.4.9.2.3.2.

Table 959—Interleaver Parameters

| Index Index | NFB NFB (bit) | P0 P0 | P1 P1 | P2 P2 | P3 P3 | Index Index | NFB NFB (bit) | P0 P0 | P1 P1 | P2 P2 | P3 P3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 48 | 5 | 0 | 0 | 0 | 21 | 568 | 19 | 102 | 140 | 226 |

… (omitted)

============================== End of Proposed Text #6 ==============================

# Proposed Change #7  (page 755)

- In Table 960 of page 755, when m=10, the field width is too narrow to display in one line. Please widen the field of m for index=37, 38 and 39.

<Widen the field for index=37, 38, 39 to put the value of m = “10” in one line at lines 34-41 in Table 960 of page 755.>

# Proposed Change #8  (pages 756)

- To make the style consistent.

< Modify  at line 8 of page 756.>

============================== Start of Proposed Text #8 ==============================

…… When the FEC block size NFB NFB is equal to multiple of the modulation order, ……

============================== End of Proposed Text #8 ==============================

# Proposed Change #9  (page 757)

- The variables before and after the operator ‘mod’ are too close to read the equation. Please insert a space before and after the operator ‘mod’. Besides, the operator ‘mod’ is not italic.

- At line 28 of page 757, NFB is the same for each FEC block of a burst so it doesn’t need subscript k.

< Modify NCTC,k at line 1 of page 757 for the consistent font.>

< Modify NSM at line 9 of page 757 for the consistent font.>

<Modify the sentences at lines 12-28 of page 757 as follows.>

============================== Start of Proposed Text #9 ==============================

Let NCTC,kCTC, k be the number of coded bits that shall be transmitted for the kth FEC block. The value of NCTC,kCTC, k is calculated by Equation (308):

(308)

where NSMSM is equal to the STC rate allocated for the burst.

The index in the HARQ buffer for the jth bit transmitted for the kth FEC block   shall be:

for k = 0, …, KFBFB – 1, and j = 0, 1, …, NCTC, k – 1, where i is the subpacket ID of the subpacket (SPID = i),  is the starting position for subpacket i of the kth FEC block as specified in 16.3.11.4.1, and  is the buffer size for the kth FEC block of a burst.

============================== End of Proposed Text #9 ==============================

# Proposed Change #10  (page 761)

- the word “antenNA” is a typo.

<Modify the word “antenNA” as antenna at line 33 of page 761.>

============================== Start of Proposed Text #10 ==============================

The constellation-mapped data zk after precoding shall be subsequently modulated onto the allocated data subcarriers for each antenNAna.

============================== End of Proposed Text #10 ==============================

# Proposed Change #11  (page 762)

- Don’t explain the parameter zk in Equation 311.

<Modify the sentence at line 16 of page 762.>

============================== Start of Proposed Text #11 ==============================

The pilot subcarriers,, of each stream shall be modulated after precoding according to Equation(311),

============================== End of Proposed Text #11 ==============================

# Proposed Change #12  (page 763-764)

- The variables before and after the operator ‘mod’ are too close to read the equation. Please insert a space before and after the operator ‘mod’. Besides, the operator ‘mod’ is not italic.

- At line 39 of page 691, for the tth transmission, the subpacket ID shall be set to SPID = t mod 4. t should be stated clearly starting from 0.

<Modify the sentences in section 16.3.11.4.1 of page 6763 as follows.>

============================== Start of Proposed Text #12 ==============================

# 16.3.11.4.1 IR HARQ

HARQ IR (Incremental Redundancy) is performed by changing the starting position, Pi,k, of the bit selection for HARQ retransmissions.

For downlink HARQ, the starting point for the bit selection algorithm as described in 16.3.11.1.6 is determined as a function of SPID using Table 964.

Table 964—Starting position determination for downlink HARQ

| SPID | Starting position Pi,k |
| --- | --- |
| 0 | 0 |
| 1 |  |
| 2 | (NFBBuffer,k/2−NCTC,k/2) mod NFBBuffer,k |
| 3 | (NFBBuffer,k−NCTC,k/2) mod NFBBuffer,k |

For uplink HARQ, the starting position for the bit selection algorithm as described in 16.3.11.1.6 is determined as a function of SPID for Equation (312).

(312)

(312)

For uplink HARQ, subpackets shall be transmitted in cyclic order of SPIDs starting from 0 (or, 0 ->→ 1 ->→ 2 ->→ 3 ->→ 0 ->→ ...). In other words, for the tth transmission, the subpacket ID shall be set to SPID = t mod 4, where t = 0, 1, 2, ….

============================== End of Proposed Text #12 ==============================