| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for a set of messages for location based service in connect state (16.8.2) |  |
| Date Submitted | 2010-05-13 |  |
| Source(s) | Lei Zhou, Xufeng Zheng, Fangmin Xu Beijing Samsung Telecom R&D Center Sangheon Kim, Ji-Yun Seol, Brian Shim, Naehyun Lim, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D5 |  |
| Abstract | The contribution proposes LBS indication message and SCN series messages to support Location based Service. The LBS indication message can support UL-TDOA and SCN series messages can support DL-TDOA. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for a set of messages for location based service             in connect state (16.8.2)

Lei Zhou, Xufeng Zheng, Fangmin Xu,

Beijing Samsung Telecom R&D Center

Sangheon Kim, Ji-Yun Seol, Brian Shim, Naehyun Lim,Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

In the current SDD, description on LBS report and connect state is as follows:

12.3 Reporting Methods for LBS

For E911 services, the AMS location can be reported to ABS through UL inband signaling. Other reporting methods are FFS.

12.3.1	Reporting Types

According to the measurement methods of LBS, some location information or some LBS measurement parameters such as CINR/RSSI/RD/RTD/Angle are transmitted to the ABS to measure the location.

12.3.2	Reporting Mode

An AMS supported LBS reports location information if any of following location information reporting condition is met.

-Timer based location information reporting

-Threshold based location information reporting

An LBS-capable AMS should support the following reporting modes: per-request, periodic, and event-triggered reporting modes. The event-triggered reporting mode is a variation of the periodic reporting mode with reporting criteria, such as a moving distance threshold and updated timer expiration. For example, the AMS will report the location when the distance between the current location and the last reported location beyond the “moving distance threshold”.

12.4.1 Connected State

The system should be able to locate the mobile when in connected state.

For connected state, LBS can be initiated by the ABS or the AMS. LBS message contains some LBS information, which may include identifier of the AMS, and indicator of LBS measurement method. Other associated parameters for LBS measurement are FFS. Indicator of LBS measurement is used to instruct the ABS and/or the AMS to perform LBS measurement and report location information.

Based on above-mentioned description of SDD, we can know that for connected state, LBS can be initiated by the ABS and AMS need report its measurement result to ABS. So in connect state, we need a set of control messages for LBS including Broadcast (AAI_LBS-ADV), SCN series messages(AAI_SCN-REQ/RSP/REP) and LBS indication message(AAI_LBS-IND).

So we propose LBS indication message(AAI_LBS-IND) for UL-TDOA and SCN series message for DL-TDOA.

Text to change in AWD

-------------------------------  Text Start  ---------------------------------------------------

[Adopt the following text modification started from line# 47 Pages 823 in P802.16m/D5]

16.8.2.4.2 LBS Measurement Message formats

The AAI defines MAC control AAI_SCN-REQ/RSP/REP messages to assign the unavailable interval for the AMS to measure its location by receiving the DL reference signals from the candidate ABSs.

The location measurement procedure may be initiated by either the AMS or the network, which is requested by the application associated with the AMS or the client attached at the core network.

An ABS may trigger a location measurement by sending a MAC control message AAI_SCN-RSP to the AMS. The AMS responds to this with a MAC control message AAI_SCN-REP with the parameters that enable location determination.

The AAI defines AAI_LBS-IND message for U-TDOA.

Table NNN—parameters for AAI_ LBS-IND

| Name | Value | Usage |
| --- | --- | --- |
| Dedicated CDMA ranging code of neighbor ABS | 5 | Indicates the dedicated ranging code |
| Ranging opportunity index of neighbor ABS | 3 | Indicates the index of the allocated ranging opportunity |
| Subframe Index of neighbor ABS | 3 | Indicates the subframe index of the allocated ranging opportunity |
| Dedicated CDMA ranging code for the serving ABS | 5 | Indicates the dedicated ranging code. |
| Ranging opportunity index for the serving ABS | 3 | Indicates the index of the allocated ranging opportunity. |
| Subframe Index for the serving ABS | 3 | Indicates the subframe index of the allocated ranging opportunity. |

-----------------------------------------  Text End ---------------------------------------------------------------