| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications to AMS Battery Level Report Signaling Header (16.2.2.1.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | Voice : + 886 3-5914629 E-mail: mhtao@itri.org.tw |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a on P802.16m/D5 |  |
| Abstract | The contribution proposes some modifications to AMS Battery Level Report Signaling Header (16.2.2.1.3.5). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications to AMS Battery Level Report Signaling Header (16.2.2.1.3)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

The ABS may need to poll the AMS about its battery status information if the ABS detects that an AMS didn’t report its battery status for a long time. We suggest adding additional information in the battery level report header to enable this capability.

- Proposed Text for IEEE P802.16m/D5

[Accept the following modifications marked in blue]

============================ Start of Proposed Text ==================================

[Insert the blue text into line 22, page 53 (16.2.2.1.3.5)]

#### 16.2.2.1.3.5 AMS Battery Level Report Header

Table 661—AMS battery Report header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AMS Battery Level Report header () { |  |  |
| FID | 4 | Flow Identifier. Set to 0001. |
| Type | 4 | MAC signaling header type = 0b0100 |
| Length | 3 | Indicates the length of Signaling header. Set to 0b010. |
| Polling Indication | 1 | 0b0: This is a report message from AMS to ABS 0b1: This is a polling message from ABS to AMS |
| If (Polling Indication = 0) { |  |  |
| AMS Battery Status | 1 | 0b0: The AMS is plugged into a power source 0b1: The AMS is not plugged into a power source |
| Battery Level Indication | 1 | 0b0: Detailed battery level report is not included. 0b1: Detailed battery level report is included. |
| If (Battery Level Indication = 1) { |  |  |
| AMS Battery Level | 3 | 0b000: Battery level is >75% and <= 100% 0b001: Battery level is >50% and <= 75% 0b010: Battery level is >25% and <= 50% 0b011: Battery level is >5% and <= 25% 0b100: Battery level is below 5% 0b101: Battery level report is not supported 0b1010b110 – 0b111: Reserved |
| Reserved | 35 | Shall be filled by 0 |
| else { |  |  |
| Reserved | 38 | Shall be filled by 0 |
| } |  |  |
| Else { |  |  |
| Reserved | 4 |  |
| } |  |  |
| } |  |  |

============================== End of Proposed Text ====================================

- References

- IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, April 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.