| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on AAI_SCD Update (16.2.3.30) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Xianming Chen, Feng Xie, Yanfeng Guan, Lei Zhang ZTE Corporation | guan.yanfeng@zte.com.cn |
| Re: | Comment on “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P802.16m/D5” 16.2.3.30 AAI_System Configuration Descriptor (SCD) Message |  |
| Abstract | This contribution proposes text on AAI_SCD Update. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on AAI_SCD Update

Xianming Chen, Feng Xie, Yanfeng Guan, Lei Zhang

ZTE Corporation

- Introduction

In the current 802.16m draft amendment [1], it is showed that an AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

- Problem

AAI_SCD message contains some important parameters associated with UL power control, ranging, sounding and handover etc. An AMS should ensure that AAI_SCD parameters being used are identical with those required by ABS; otherwise, AMS may not be in normal operation, and even may impact normal operation of other AMSs. Because AAI_SCD transmission process is very robust as that of broadcast message, an AMS in active mode usually can successfully receive and update AAI_SCD in time. But, as an AMS in sleep mode may not be able to receive the changed AAI_SCD during sleep interval, thus after the AMS wakes up, it shall not have up-to-date AAI_SCD parameters, and also shall not be in normal operation; in addition, if AAI_SCD transmission periodicity is very long, the above AMS may not return to normal operation condition within a very short time. Therefore, a scheme shall be needed to resolve the above problem.

- Proposed Method

In P802.16m/D5, the following description is provided about SFH update: “The ABS shall indicate any changes of S-SFH IE at the specific super-frame number(SFN) that satisfies SFN modulo S-SFH change cycle = 0. S-SFH change cycle = Rate of change of S-SFH Info * the periodicity of S-SFH SP1 IE / 20ms. Where the "Rate of change of S-SFH Info" and "the periodicity of S-SFH SP1 IE" are indicated in S-SFH SP3 IE.” Through introducing “SFH change cycle” and allowing an AMS to wake up in specific SFN in sleep interval, the problem on SFH update for an AMS in sleep mode shall be well resolved. Similarly, the method may also be used for AAI_SCD update for an AMS in sleep mode. But, considering that an AMS waking up separately to receive SFH or AAI_ACD is not helpful to power saving of the AMS, it is preferable to introduce a kind of association between “SFH change cycle” and “AAI_SCD change cycle” in order to ensure that SFH and AAI_ACD update are simultaneously finished for wake-up once. In addition, when the parameter of AAI_SCD is associated with that of S-SFH, ABS also may indicate whether the changed AAI_SCD and S-SFH are applied through “S-SFH applying offset” in P-SFH.

- References

[1] "DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Advanced Air Interface", P802.16m/D5 April. 2010.

- Proposed Text

----------------------------------------------------------- Start of the Proposed Text  --------------------------------------

Remedy: Line 45, Page 155, 16.2.3.30 AAI_System Configuration Descriptor (SCD) Message

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration, and changes at the specific super-frame number(SFN) that satisfies SFN modulo AAI_SCD change cycle = 0. For the synchronized update of SFH and AAI_SCD at ABS and AMS, AAI_SCD change cycle = SFH change cycle. Then based on "Rate of change of S-SFH Info" and "the periodicity of S-SFH SP1 IE" in S-SFH SP3 IE, the "SFH change cycle" and “AAI_SCD change cycle” shall be acquired by an AMS.

-----------------------------------------------------------End of the Proposed Text   -------------------------------------