| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frequency partition for CINR calculation in scan and handover triggers (16.2.3) |  |
| Date Submitted | 2010-05-09 |  |
| Source(s) | Daniel Cohn, Ariel Doubchak Alvarion Jia Lin Huawei Wongbook Lee LG Electronics | daniel.cohn@alvarion.com linjia@huawei.com wongbook.lee@lge.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D5) |  |
| Abstract | Define the frequency partition for which CINR should be calculated for scan and handover trigger purposes |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Frequency partition for CINR calculation in scan and handover triggers (16.2.3)

Daniel Cohn, Ariel Doubchak
Alvarion

Jia Lin
Huawei

Wongbook Lee
LG Electronics

Problem Statement

802.16m/D5 allows the ABS to define CINR-based scan and handover triggers. It does not however indicate how the CINR should be calculated by the AMS, specifically which subcarriers should be considered for noise and interference calculation. It is expected that ABS operating in different reuse schemes will specify scan and handover triggers that reflects signal quality for downlink data received at the S-ABS and/or T-ABS. 
802.16m explicitly defines the concept of different frequency partitions where different reuse factors are applied.

For example, in a single frequency channel, reuse-3 scheme scenario, AMSs see only one third of the ABSs as interferers and thus the received AMS CINR is different (higher) than the CINR experienced in a reuse-1 scenario (where all ABSs transmit over all the subcarriers in the channel). A strong MS in the equal border between two sectors would experience a reuse-1 CINR near 0 dB and a reuse-3 CINR in the 15-25 dB range. Thus it is clear that CINR calculation according to the relevant reuse scheme is critical for good handover performance.

Hence, ABSs need to be able to specify the reuse scheme for which S-ABS and T-ABS CINR should be calculated for scan and handover trigger purposes. This ability is supported in 802.16-2009 as detailed in section 8.4.12.3.

Suggested Solution

To support different FFR schemes, the reuse scheme to be used when calculating CINR will be implicitly indicated by specifying frequency partition information for which CINR values should be considered.

Frequency partition information (partitioning configuration and boosting/deboosting scheme) to be assumed for CINR calculations for a neighbor ABS will be explicitly advertised in the AAI_NBR-ADV message. When not explicitly included, the AMS will use the frequency partition information advertised in the SFH information for the neighbor ABS in the AAI_NBR-ADV message.

This allows the ABS to indicate different frequency partition information for each neighbor ABS, as a function of the FFR configuration. For example, in a pure reuse-3 scheme (“hard FFR”) with three frequency partitions, it is to be expected that the ABS will request CINR calculations for each neighbor ABS to be relevant to the frequency partition where this neighbor ABS transmits.

Frequency partition information to be assumed for CINR calculations for the serving ABS will be advertised in the AAI_SCD message. Considerations above also apply.

Specific formulas for CINR calculations in different reuse schemes should be included in the relevant PHY sections. See contribution C10/0306 for an example.

If the AMS is scanning a neighbor ABS not included in the AAI_NBR-ADV message, i.e. commanded by an AAI_SCN-RSP message, said AAI_SCN-RSP message should include FFR information for the neighbor ABS, to be used by the AMS to calculate neighbor ABS CINR for trigger purposes.

Suggested Standard Modifications

1.- Modify table 693 in section 16.2.3.12 (AAI_NBR-ADV), page 111 after line 64, as follows:

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... |
| M | BSID | 48 |  | Shall be included for each neighbor BS |
| O | FFR partitioning configuration for scan/handover trigger neighbor ABS CINR calculations | 4 | DFPC of the FFR partitioning configuration, as defined in 16.3.5.2.3 | If not included, AMS will use FFR partitioning configuration advertised in the SFH information for the neighbor ABS in this message |
| O | FFR power configuration for scan/handover trigger neighbor ABS CINR calculations | 3 | fpPowerConfig, as defined in table 814 | If not included, AMS will use FFR power configuration advertised in the SFH information for the neighbor ABS in this message |
| M | Frequency partition for scan/handover trigger neighbor ABS CINR calculations | 2 | The frequency partition number according to whose FFR parameters CINR is calculated | Shall be included for each neighbor BS |

2.- Modify table 696 (AAI_SCN-RSP) in section 16.2.3.14 in page 122 after line 31, as follows:

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | N_Recommended_Preambe_Index |  | 4 | Number of preambles to be scanned [0..15] | Present if AMS decides to request preamble scanning |
| O | preambleIndexArr[0..14] | Preamble Index | Variable ( 6 x N) | Preamble Index of Recommended BS ID | Present if N_Recommended_Preambe_Index > 0 |
| O |  | Physical carrier index |  | As defined in AAI_Global-Config message | Shall be included when target ABS is not included in AAI_NBR-ADV message |
| O |  | Channel bandwidth of recommended BS ID |  | Channel bandwidth of recommended BS ID in units of 125 KHz. Field length is 1 byte. | Shall be included when target ABS is not included in AAI_NBR-ADV message |
| O |  | CP length |  | CP length of the recommended target ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included when target ABS is not included in AAI_NBR-ADV message |
| O |  | FFR partitioning configuration for neighbor ABS CINR calculation | 4 | DFPC value (0..15), as defined in 16.3.5.2.3 | Shall be included when recommended BS ID is not included in AAI_NBR-ADV |
| O |  | FFR power configuration for neighbor ABS CINR calculation | 3 | fpPowerConfig (0..8), as specified in table 814 | Shall be included when recommended BS ID is not included in AAI_NBR-ADV |
| O |  | Frequency partition for scan/handover trigger neighbor ABS CINR calculations | 2 | FP number (0..3) | Shall be included when recommended BS ID is not included in AAI_NBR-ADV |

3.- Add the following text in table 717 (AAI_SCD) in section 16.2.3.29, page 115 after line 36:

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| ... | … | ... | ... | ... |
| M | Default Trigger Averaging Parameter |  | Provides default trigger averaging parameter of the BS 0 - 9 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 | Used in equation (1) |
| M | Frequency partition for scan/handover trigger serving ABS CINR calculations | 2 | FP number (0..3) |  |

4.- Add the following text in section 16.2.6.2 (Trigger condition definitions) in page 271 after line 58:

RSSI and CINR calculations are defined in 16.3.14. FFR partition for CINR calculation at the serving ABS is specified in the the AAI_SCD message. FFR configuration for CINR calculation at a neighbor ABS is specified in the AAI_NBR-ADV message if the neighbor ABS is included in the AAI_NBR-ADV message, and in the AAI_SCN-RSP message if it is not.