| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Changes in AAI_SCN-REQ message (16.2.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Daniel Cohn Alvarion | Daniel.cohn@alvarion.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D5) |  |
| Abstract | Changes in AAI_SCN-REQ message |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Changes in AAI_SCN-REQ message (16.2.3)

Daniel Cohn, Alvarion

Problem Description

1.- Scan purpose two values are the same, and in any event this field is not needed as this information can be derived by the ABS from the list of ABS the MS plans to scan and the contents of the AAI_NBR-ADV

2.- BS and MS should be replaced by ABS and AMS

3.- Several other minor errors and confusing sentences.

Suggested Standard Modifications

1.- Modify table 695 (AAI_SCN-REQ parameters) in section 16.2.3.13, from page 117 line 44 until page 119 line 34, as follows:

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| M | Scan duration |  |  | 8 | Duration (in units of AAI subframes) of the requested scanning period. |  |
| M | Scan purpose |  |  | 1 | 0b0: scan BS(s) which is in the list of the AAI_NBR-ADV message 0b1: scan BS(s) which is not in the list of the AAI_NBR-ADV message |  |
| M | Interleaving |  |  | 8 | The period of AMS's normal operation (in units of AAI subframes), which is interleaved between Scan Durations. |  |
| M | Scan Iteration |  |  | 6 | The requested number of iterating scanning interval by an AMS. |  |
| O | Recommended start super frame number |  |  | 6 | Represents the 6 least significant bits of the absolute super frame number for which the AMS recommends the first Scanning Interval to start. This field is set to 0 if an AMS has no preferred value. | Present if AMS recommends a start super frame |
| O | Recommended start frame number |  |  | 2 | Represents recommended start frame number within a super frame. The value indicates: 0b00: the 1st frame in a super frame 0b01: the 2nd frame in a super frame 0b10: the 3rd frame in a super frame 0b11: the 4th frame in a super frame | Present if Recommended start super frame number is present |
| O | N_Recommended_ABS_Index |  |  | 6 | This is the number of neighboring ABS [0 .. 63] the AMS plans to scan to be scanned, which are included in AAI_NBR-ADV message. When an AMS receives AAI_SCN-RSP message from ABS in response to AAI_SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAI_SCN-RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI_SCN-REQ message to the ABS. | Present if AMS decides to uses index instead of full 48 bits BSID to identify an ABS it plans to scan |
| O | Configuration Change Count for AAI_NBR-ADV |  |  | 8 | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID used for neighbor ABS index references | Present if N_Recommended_ABS_Index > 0 |
| O | nbrBsIndexArr [0..63] | Neighbor ABS index |  | Variable ( 8 x N) | Based on the value of N_Recommended_ BS_Index, ABS index corresponds to the position of ABS in AAI_NBR-ADV message is included. | Present if N_Recommended_ABS_Index > 0 |
| O | N_Recommended_ ABS_Full |  |  | 6 | Number of neighboring ABS [0..63] the AMS plans to scan to be scanned, which are using full 48-bit ABSID. | Present if AMS decides to use full 48 bits BSID to identify a ABS |
| O | fullBsIdArr [0..63] | Recommended ABS ID |  | Variable ( 48 x N) | Based on the value of N_Recommended_ BS_Full, identifiers ABS-ID of the ABSs the AMS plans to scan is included. | Present if N_Recommended_ ABS_Full > 0 |
| O | N_Recommended_Preamble_Index |  |  | 4 | Number of preambles to be scanned the AMS plans to scan [0..15] | Present if AMS decides to request to scan preambles |
| O | preambleIndexArr[0..145] | Preamble Index |  | Variable ( 6 x N) | Based on the value of N_Recommended_ ABS_Full, identifiers Preamble indices of the BSs that the AMS plans to scan is included. | Present if N_Recommended_Preamble_Index > 0 |
| O | nbrAbsArr[0..63] | N_Recommended_Carrier_Index |  | Variable ( 6 x N) | Number of carriers the AMS plans to scan to be scanned at each neighboring ABS [0..63]. | Present when a neighboring ABS is a multicarrier ABS and, N_Recommended_ ABS_Index > 0 or N_Recommended_ ABS_Full > 0 |
| O |  | carrierIndexArr[0..632] | Recommended_Carrier_Index | Variable ( 6 x N x M) | Recommended Requested physical carrier index for scan at each neighboring ABS. | Present if N_Recommended_Carrier_Index > 0 |
| O | N_Recommended_Carrier_Index_at_Serving_ABS |  |  | 6 | Number of carriers the AMS plans to scan to be scanned at each neighboring ABS. | Present if AMS decides to scan carrier index |
| O | carrierIndexArr[0..632] | Recommended Carrier index at serving ABS |  | Variable ( 6 x N) | Recommended physical carrier index of the serving ABS the AMS plans to scan to be scanned | Present if N_Recommended_Carrier_Index_at_Serving_ABS > 0 |
| O | csgIdArr[0..15] | CSG Identifier |  | Variable (8 x N) | Sent by AMS to aid ABS reselection | Present if CSG identifier is available |