| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Changes in AAI_SCN-REP message (16.2.3) |  |
| Date Submitted | 2010-05-11 |  |
| Source(s) | Daniel Cohn Alvarion | Daniel.cohn@alvarion.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D5) |  |
| Abstract | Changes in AAI_SCN-REP message |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Changes in AAI_SCN-REP message (16.2.3)

Daniel Cohn, Alvarion

Problem Description

1.- Metrics are reported for each of the reported carriers or preamble indices, not only once per message or once per preamble index.

2.- Report metric field is not necessary, ASN.1 automatically encodes optional fields

3.- BS and MS should be replaced by ABS and AMS

4.- Preamble should be replaced by SA Preamble

5.- “Recommended” should be replaced by “reported”

6.- The number of reported ABSs using AAI_NBR-ADV index encoding is not included

7.- ABS could include reports with different ABS references (full, index, preamble) in the same AAI_SCN-REP message

8.- Meaning of “Scanned FA index” not clear. Report format is complete without this field.

9.- Number of reported carriers is always greater than zero, thus no need to specify it as condition.

10.- AMS can report relative delay N-ABS, not RTD

11.- S-ABS metrics are missing

12.- Several other minor errors and confusing sentences.

Suggested Standard Modifications

1.- Modify table 697 (AAI_SCN-REP parameters) in section 16.2.3.15, from page 123 line 14 until page 125 line 38, as follows:

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| M | Report mode |  |  | 1 | Action code for an AMS's scan report of its measurement 0: Event-triggered report 1: Periodic report according to Scan report period of AAI_SCN-RSP |  |
| M | Report metric |  |  | 4 | Bitmap indicator of trigger metrics include in the report.that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. For each bit location, a value of 0 indicates the trigger metric is not included, while a value of '1' indicates the trigger metric is included in the message. The bitmap interpretation for the metrics shall be: Bit 0: BS CINR mean Bit 1: BS RSSI mean Bit 2: Relative delay Bit 3: BS RTD |  |
| O | S-ABS CINR mean |  |  | 8 | The S-ABS CINR Mean parameter indicates the CINR measured by the AMS from the particular serving ABS. The value shall be interpreted as a signed byte with units of 0.5 dB. The measurement shall be performed on the subcarriers of the frame preamble that are active in the serving ABS's segment and averaged over the measurement period. | Present if AMS is asked to report CINR |
| O | S-ABS RSSI mean |  |  | 8 | The S-ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from the particular serving ABS. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An AMS shall be able to report values in the range -103.75 dBm to -40 dBm range. The measurement shall be performed on the frame preamble and averaged over the measurement period. | Present if AMS is asked to report RSSI |
| O | Relative delay |  |  | 8 | This parameter indicates the delay of neighbor DL signals relative to the serving BS, as measured by the MS for the particular BS. The value shall be interpreted as a signed integer in units of samples. | Present if AMS is asked to report relative delay |
| O | S-ABS RTD |  |  | 8 | The S-ABS RTD parameter indicates the round trip delay (RTD) measured by the AMS from the particular serving ABS. RTD can be given by the latest time advance taken by MS. The value shall be interpreted as an unsigned byte with units of 1/Fs. | Present if AMS is asked to report RTD |
| O | N_Neighbor_ BS_Index N_Reported_ ABS_Index |  |  | 6 | Number of neighboring ABS reported in this message and which are included in AAI_NBR-ADV message [0..63]. | Present only when the scanning report is based includes ABSs referred using ABS index |
| O | Configuration Change Count for AAI_NBR-ADV |  |  | 8 | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID used for neighbor ABS index references | Present only when the scanning report is based BS index Present only when N_Reported_ ABS_Index > 0 |
| O | nbrBsIndexArr [0..63] | Neighbor ABS index |  | Variable ( 8 x N) | Based on the value of N_Recommended_ BS_Index, ABS index corresponds to position of ABS in AAI_NBR-ADV message is included. | Present only when the scanning report is based BS index Present only if N_Reported_ ABS_Index > 0 |
| O | N_Recommended_ BS_Full N_Reported_ BS_Full |  |  | 6 | Number of neighboring ABS [0..63] reported in this message to be scanned, which are using full 48-bit BSID. | Present only when the scanning report is based includes ABSs referred using ABS ID |
| O | fullBsIdArr [0..63] | Recommended Reported ABS ID |  | Variable ( 48 x N) | Based on the value of N_Recommended_ ABS_Full, identifiers ABS-IDs of the ABS s reported in this message the MS plans to scan is included. | Present if N_Recommended_BS_Full > 0 Present if N_Reported_ABS_Full > 0 |
| O | FA_Index |  |  | 4? | Scanned FA Index Note: needs definition of FA index |  |
|  | faIndexArr[0.14] | faIndex |  | ? | Note: needs definition of FA index |  |
| O | N_Recommended_Preambe_Index N_Reported_SA_Preamble_Index |  |  | 4 | Number of SA preambles to be scanned reported in this message [0..15] | Present if AMS is to report preamble scanning Present only when the scanning report includes SA preamble reports |
| O | preambleIndexArr[0..145] | SA Preamble Index |  | Variable ( 10 x N) | Based on the value of N_Recommended_ BS_Full, identifiers SA preamble indices reported in this message of the BSs the MS plans to scan is included. | Present if N_Recommended_Preambe_Index > 0 Present if N_Reported_SA_Preamble_Index > 0 |
| O |  | CINR mean |  | 8 | Indicates the CINR measured by the AMS from this SA preamble. The value shall be interpreted as a signed byte with units of 0.5 dB.. | Present if AMS is asked to report CINR |
| O |  | RSSI mean |  | 8 | Indicates the Received Signal Strength measured by the AMS from this SA preamble. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. | Present if AMS is asked to report RSSI |
| O |  | Relative delay |  | 8 | Indicates the delay of SA preamble DL signals relative to the serving ABS, as measured by the MS for this SA preamble. The value shall be interpreted as a signed integer in units of samples. | Present if AMS is asked to report relative delay |
|  |  | Report value |  | Variable (? X N) | Based on the value of Report metric Note: needs definition of report value |  |
| O | nbrAbsArr[0..63] | N_Recommended_Carrier_Index N_Reported_Carrier_Index |  | Variable ( 6 x N) | Number of carriers reported to be scanned at each neighboring ABS [0..63]. | Present when N_Recommended_ BS_Index > 0 or N_Recommended_ BS_Full > 0 Present when N_Reported_ABS_Index > 0 or N_Reported_ABS_Full > 0 |
| O |  | carrierIndexArr[0..632]] | Recommended_Carrier_Index Reported_Carrier_Index | Variable ( 6 x N x M) | Recommended Reported physical carrier index for scan at each neighboring ABS. | Present if N_Recommended_Carrier_Index > 0 |
| O |  |  | CINR mean | 8 | The ABS CINR Mean parameter indicates the CINR measured by the AMS from this carrier. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| O |  |  | RSSI mean | 8 | The ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from this carrier. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. | Present if AMS is asked to report RSSI |
| O |  |  | Relative delay | 8 | This parameter indicates the delay of neighbor DL signals relative to the serving ABS, as measured by the MS for this carrier. The value shall be interpreted as a signed integer in units of samples. | Present if AMS is asked to report relative delay |
| O | Neighbor_request_indication |  |  | 1 | Request indication for system information and the list of the neighbor Femto ABSs |  |