| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Increase DID field length to reduce paging collision probability (16.2.1) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Daniel Cohn, Oleg Marichenco, Gal Haklai, Vladimir Yanover, Mirit Elyada-Bar Alvarion | daniel.cohn@alvarion.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D5) |  |
| Abstract | Length of DID field in D5 (10 bits) results in paging collision probability significantly higher than 802.16e. To overcome this, this contribution proposes increasing the field size to 24 bits. |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Increase DID field length to reduce paging collision probability (16.2.1)

Daniel Cohn, Oleg Marichenco, Gal Haklai, Vladimir Yanover, Mirit Elyada-Bar
Alvarion

Problem Description

- In 802.16m/D5, the paging controller (PC) assigns each AMS entering idle mode with a deregistration identifier (DID), which is used by the PC to page the AMS. The AMS receives the assigned DID in the AAI_DREG-RSP message where the ABS commands or confirms the AMS should enter idle mode. The DID can also be modified at the location update.
If more than one AMS belonging to the same paging group is assigned with the same DID and also the same paging cycle and offset (also assigned by the network and received in the AAI_DREG-RSP message), when the PC pages one of these AMSs, all of them will exit idle mode, resulting in increased network overhead and lower battery savings for the AMSs that were not intended to be paged. This scenario is referred to as a paging collision (aka false positive result).
In the scenario shown in Figure 1 where a single PC serves all AMSs in a PG (i.e. the PC is associated with one ore more PGs), collisions can be avoided as long as there are less than P*210 idle AMSs in the PG, where P is the number of non-overlapping paging cycles and offsets supported.

Figure 1 – Centralized paging controller

It should be noted that the centralized scenario requires the PC to be topologically aware.

A different scenario should also be considered, described in Figure 2, where several PCs serve AMSs in the same PG.

Figure 2 – Distributed paging controllers

This scenario occurs for example when AMSs move across ABSs with different preferred anchor PCs, and AMSs currently in the same PG entered idle mode with different anchor PCs. . This occurs because the network allows “anchoring” of the PC function on a per-MS basis together with other MS-related network functions (such as authenticator and anchor GW). WMF NWG also developed and mandated a “topology-unaware” paging scheme where the anchor PC is not geographically associated with the particular PG. This results in many APCs (potentially as many as there are ASN GWs in the network) being able to serve idle MSs in the PG.

In such a scenario, it cannot be assumed that PCs will coordinate DID assignment to avoid collisions. No such a protocol exists, and it would not be scalable solution.

Assuming each PC assigns DIDs using a good randomization scheme, the probability of collision for a given pair of AMSs in the same PG served by different PCs becomes then2−10P. For large PGs with a high number of idle AMSs, the collision probability for any AMS in the PG becomes unacceptable. 
This problem has been acknowledged by the WiMAX Forum networking group, as described in [1], slide 9.

- Text in 802.16m/D5 requires that “The DID shall uniquely identify the AMS within the set of paging group ID, paging cycle and paging offset”. The assumption behind this statement appears to be that two AMSs sharing the same DID but with different assigned paging cycle/offset will not result in a paging collision. However this is only true if the two AMS paging cycle/offset don’t have any paging listening intervals in common, which cannot always be assumed to be true. 
Therefore the text of reference is not correct, even in the single-PC-per-PG scenario. In the multiple-PC-per-PG scenario, as explained above, DID uniqueness cannot be guaranteed.

Suggested Solution

Without presenting numerical results, it seems reasonable to expect that 802.16m paging collision probability should not be higher than that of 802.16e. In 802.16e, collision probability is 2-24. To achieve at least the same performance as in 802.16e without imposing requirements on the number of non-overlapping paging cycle and offsets that should be supported, the DID length should be increased to 24 bits.

DID uniqueness requirement should be removed, relying instead in good randomization at the PC for DID selection resulting in acceptable probability of paging collision.

Suggested Standard Modifications

1.- Modify section 16.2.1.2.3 - Deregistration Identifier (DID) in page 45, line 20 as follows:

The DID shall uniquely identify the AMS within the set of paging group ID, paging cycle and paging offset.

The network shall assign a 24 bit DID to the AMS it enters idle mode. The DID uniquely identifies the AMS in the paging controller context for the location update and idle mode re-entry processes.

2.- Modify table 680 (AAI_RNG-REQ message field descriptions) in page 78, lines 10-12, as follows:

| O | Paging Controller ID | 48 | The Paging Controller ID which the AMS currently maintains in idle mode. | It shall be included when the AMS is attempting to perform reentry from Idle Mode or location update |
| --- | --- | --- | --- | --- |
| O | Deregistration Identifier (DID) | 10 24 | The ID which the AMS is assigned for idle mode and currently maintains. |  |
| O | PGID | 16 | The identification of the paging group that the AMS is previously belonging to. |  |
| O | Paging Cycle | 4 | PAGING_CYCLE applied to the AMS |  |
| O | Paging Offset | 4 | PAGING_OFFSET applied to the AMS |  |

3.- Modify table 681 (AAI_RNG-RSP message field descriptions) in page 81, lines 48-50, as follows:

| O | Paging Controller ID | 48 | The new Paging Controller ID which the AMS shall maintain in idle mode. | It shall be included only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and Paging Controller ID has changed |
| --- | --- | --- | --- | --- |
| O | Deregistration Identifier (DID) | 10 24 | The new DID which the AMS shall maintain in idle mode. |  |

4.- Modify table 708 (AAI_DREG-RSP message) in page 138, lines 31-33 as follows:

| O | Deregistration Identifier (DID) | 10 24 | Used to indicate Deregistration ID used to identify the MS in idle mode 0..210224-1 | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| --- | --- | --- | --- | --- |

5.- Modify table 709 (AAI_PAG-ADV message) in page 141, lines 28-31 as follows:

| M | Paged MS info [1..32] | Deregistration Identifier | 1024 | Shall be present if the Network Configuration indicates ABS is attached to the advanced network and unless Num_AMSs is 0 and |
| --- | --- | --- | --- | --- |
| M |  | Paging Cycle | 4 | Shall be present unless Num_AMSs is 0 |
| M |  | Action Code | 1 | Shall be present unless Num_AMSs is 0 |

References

- http://members.wimaxforum.org/apps/org/workgroup/nwg/document.php?document_id=53212