| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Trigger function in Multi-carrier Operation in 802.16m (16.2.6.2) |  |
| Date Submitted | 2010-04-29 |  |
| Source(s) | Jia Lin, Lei Jin, Bin Chen, Junxian Mo, Ronald Mao, Kim Chang, Phillip Barber Huawei Technologies | E-mail: linjia@huawei.com jinlei60020191@huawei.com binchen@huawei.com |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “16.2.6.2” |  |
| Abstract | This contribution proposes amendment text describing trigger function in multi-carrier scenario in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m LB#31a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for Trigger function of Multicarrier Scenario

- in 802.16m

Jia Lin, Lei Jin, Bin Chen, Junxian Mo, Ronald Mao, Kim Chang, Phillip Barber

- Huawei Technologies

- Introduction

- This contribution proposes amendment text describing the Trigger function when a 16m ABS and AMSs support multiple carriers.

- In Multicarrier Scenario, AMS may use multiple carriers for data transmission. When AMS scans carriers of Serving ABS and Neighbor ABS's, it may trigger the actions such as respond on trigger with AAI_SCN-REP or AAI_HO-REQ. Since AMS need to communicate with ABS by multiple carriers, if AMS find there are other carriers of Neighbor ABS or Serving ABS satisfying trigger functions, it is possible that AMS is triggered to report these conditions or response with HO-REQ.

- In 16m/D4, it only considers trigger conditions definitions for single carrier scenario, but not for multicarrier scenario. In this case, Total bandwidth of carriers (The metric of these carriers is greater than absolute value) in neighbor ABS and Serving ABS should be measured and calculated.

- In multicarrier Scenario, if ABS defined current trigger functions for AMS to trigger actions (e.g. respond on trigger with AAI_SCN-REP or AAI_HO-REQ), it may happen that AMS will be triggered lots of  AAI_SCN-REP or AAI_HO-REQ that are not useful for serving ABS. Since AMS are communicate with serving ABS through multiple carriers, even though one neighbor ABS’s carrier is greater than absolute value, or greater than serving ABS’s carriers by relative value, but there still has other carriers available for AMS in serving ABS. Total bandwidth of available carriers in serving ABS is more than neighbor ABS. In this case, serving ABS can provide high data traffic for AMS, and AMS doesn’t need to trigger scan report or HO-REQ.

- Example 1: Serving ABS has 5 carriers, 3 carriers whose signal quality is greater than absolute value are configured as active carriers for the AMS. Meanwhile, AMS’s neighbor ABS has 3 carriers, and AMS measures that signal quality of neighbor ABS’s one carrier is greater than absolute value. If AMS follow current trigger function (e.g. 0x1: M Metric of neighbor ABS is greater than absolute value), it will send SCN-REP or HO-REQ to serving ABS. But in fact, Serving ABS has 3 carriers whose signal quality is greater than absolute value for AMS transmitting data. In this case, if ABS receives AAI-SCN-REP or AAI-HO-REQ from AMS, and it decides to make the AMS to HO, which will reduce AMS’s QoS and may possible cause ping-pong effects. So AMS is not need to be triggered sending AAI-SCN-REP or AAI-HO-REQ to serving ABS.

- Example 2: Serving ABS has 3 carriers, 2 carriers whose signal quality is greater than absolute value are configured as active carriers for the AMS. Meanwhile, AMS’s neighbor ABS has 5 carriers, and AMS measures that signal quality of neighbor ABS’s 4 carriers are greater than absolute value. In this case, neighbor ABS has 4 carriers and can provide higher data rate. Neighbor ABS’s 4 carriers are the same signal quality as Serving ABS’s 2 carriers. If AMS follow current trigger function (e.g. 0x3: Metric of neighbor ABS is greater than serving ABS metric by relative value), it will not send AAI-SCN-REP or AAI-HO-REQ to serving ABS. Then serving ABS will not make the AMS to HO to neighbor ABS for achieve higher data rate (since AMS has capability of MC can get several times data rate than single carrier).

- Conclusion

- Benefits for introducing trigger functions for MC scenario:

- Avoid useless trigger actions for MC scenario.

- Reduce signaling overhead.

- References

- IEEE P802.16-2009 “IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access”.

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE P802.16m/D4, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D5

========================== Start of Proposed Text ==========================

[Note to Editor: Add the following contents in Table 770 and 771 in “16.2.6.2 Trigger condition definitions”]

- 16.2.6.2 Trigger condition definitions

The S-ABS may define trigger conditions for the following actions:

1) Conditions that define when the AMS shall initiate scanning procedure

2) Conditions that define when the AMS shall report scanning measurement results to the serving ABS

3) Conditions that define when AMS shall initiate HO by sending AAI_HO-REQ.

4) Conditions for defining when a target ABS is unreachable

5) Conditions for defining when AMS is unable to maintain communication with the serving ABS

6) Conditions for HO cancellation

Table 770—Trigger Description

| Name | Length (Bits) | Value |
| --- | --- | --- |
| Number of conditions | 2 | The number of conditions that are included in this trigger (see loop definition below). When more than one condition is included, this trigger is referred to as a complex trigger and is the logical AND combination of all the included conditions. |
| for (i=0; i <= Number of conditions; i++) { |  |  |
| ABS Type | 8 | ABS type of target ABS for this Trigger definition: (Any, Macro Hot-zone ABS, Femto ABS, etc.). A value representing "any" means this trigger applies to all target ABSs |
| Type/Function/Action | 910 | See Table 771 for description |

Table 771—Trigger; Type/Function/Action Description

| Name | Length (Bits) | Value |
| --- | --- | --- |
| Type | 2 3(MSB) | Trigger metric type: 0x0: CINR metric 0x1: RSSI metric 0x2: RTD metric 0x3: Number of missed superframes metric 0x4: Number of carriers |
| Function | 34 | Computation defining trigger condition: 0x0: Reserved 0x1: Metric of neighbor ABS is greater than absolute value 0x2: Metric of neighbor ABS is less than absolute value 0x3: Metric of neighbor ABS is greater than serving ABS metric by relative value 0x4: Metric of neighbor ABS is less than serving ABS metric by relative value 0x5: Metric of serving ABS greater than absolute value 0x6: Metric of serving ABS less than absolute value 0x7: Reserved 0x8: Number of neighbor ABS’s carriers whose metric are greater than absolute value is more than specific value 0x9: Number of neighbor ABS’s carriers whose metric are greater than absolute value is more than number of serving ABS’s carriers whose metric are greater than specific value. 0x10-0x15: Reserved NOTE-0x1-0x4 and 0x8-0x9 not applicable for RTD trigger metric NOTE-When type 0x1 is used together with function 0x3 or 0x4, the threshold value shall range from -32 dB (0x80) to +31.75 dB (0x7F). When type 0x1 is used together with function 0x1, 0x2, 0x5 or 0x6, the threshold value shall be interpreted as an unsigned byte with units of 0.25 dB, such that 0x00 is interpreted as -103.75 dBm and 0xFF is interpreted as -40 dBm NOTE-Type 0x3 can only be used together with function 0x5 or function 0x6 NOTE- Functions 0x8-0x9 are only used for Multicarrier scenario. |
| Action | 3(LSB) | Action performed upon reaching trigger condition: 0x0: Reserved 0x1: Respond on trigger with AAI_SCN-REP 0x2: Respond on trigger with AAI_HO-REQ 0x3: Respond on trigger with AAI_SCN-REQ 0x4: Declare ABS unreachable: If this ABS is the serving ABS, AMS sends AAI_HO-IND with code 0x03 to the serving ABS and proceeds as specified in section 15.2.5.2.4. If this ABS is a target ABS, the AMS needs not take immediate action when this trigger condition is met for a single ABS. The AMS shall act only when this condition is met for all target ABSs included in AAI-HO-CMD during HO execution. The specific actions are described in section 15.2.5.2.4. 0x5: Cancel HO 0x6 and 0x7: Reserved NOTE-0x3 is not applicable when neighbor ABS metrics are defined (i.e., only Function values 0x5 or 0x6 are applicable). |

The ABS may define complex trigger conditions by including multiple conditions within the same trigger definition. In this case, all included conditions shall have the same Action code (as defined in Table 760). The AMS shall perform a logical AND of all the conditions in a complex trigger condition and invoke the action of the trigger only when all trigger conditions are met.

Whenever the condition of a simple trigger or all the conditions of a complex trigger are met, the AMS shall invoke the action of the trigger. If multiple trigger conditions are met simultaneously the AMS shall invoke the action of at least one of the triggers for which the trigger condition was met. Action 0x2: Respond on trigger with AAI_HO-REQ takes precedence over Action 0x1 and Action 0x3

The ABS may define neighbor-specific triggers by including neighbor-specific triggers/neighbor-MC-specific triggers in the AAI_NBR-ADV message. The AMS evaluates neighbor-specific triggers only for the specific neighbor BS metric, and evaluates neighbor-MC-specific triggers only for the specific neighbor BS’s MC metric. Neighbor-specific triggers use the format in Table 770, where only function types 0x1, 0x2, 0x3 and 0x4 and actions types 0x1 and 0x2 are allowed. When present, neighbor-specific handover triggers override any general triggers (define in the AAI_SCD message) of the same type, function and action. Neighbor-MC-specific trigger TLVs use the format in Table 770, where only function types 0x8 and 0x9 and actions types 0x1 and 0x2 are allowed.

[Note to Editor: Add the following contents in Table 693 in “16.2.3.12 AAI_NBR-ADV”]

Table 693—AAI_NBR-ADV parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Neighbor-specific trigger | Var. | Optional neighbor-specific triggers with encoding defined in Table 7670- Trigger description | May be included for each neighbor BS |
| O | Neighbor-MC-specific triggers | Var. | Optional neighbor-MC-specific triggers with encoding defined in Table 770- Trigger description | May be included for each neighbor BS |

========================== End of Proposed Text ===========================