| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Sounding Channel Power Control (16.3.9.4.5) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Jeongho Park, Hokyu Choi Samsung Electronics Co., Ltd. | jeongho.jh.park@samsung.com |
| Re: | IEEE 802.16-10/0032, “IEEE 802.16 Working Group Letter Ballot Recirc #31a: Announcement” |  |
| Abstract | The contribution proposes a clarification text for sounding channel power control |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 6) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Sounding Channel Power Control

Jeongho Park,,Hokyu Choi

Samsung Electronics

- Introduction

This contribution tries to clarify unclear part in D5 related to uplink sounding channel power control.

1. Sentence clarification

In section 16.3.9.4.5 Sounding Channel Power Control, a sentence is saying about power boosting field. However this sentence does not describe where this field comes from.

2. Default parameter for sounding channel power control

Table 943 – Default ULPC Parameters contains a lot of parameters which are going to be used by AMS who has not received AAI_SCD message yet. Mainly initial network entry procedure is when AMS should use these parameters.

However one parameter “Sounding Channel SINR Target Value” is not defined in sounding channel power control. And its usage is not defined anywhere. Most of all, sounding operation does not have to be done before AMS get AAI_SCD message because it is not that critical in terms of time just like periodic ranging. (SCD contains the necessary parmeters for periodic ranging.)

On top of that, in order to do sounding operation before SCD reception, decimationValueD and maxCyclicShiftIndexP and shiftValueUForSoundingSymbol are also necessary.

Now we have both options :

a. No support for sounding operation before AAI_SCD message is received at MS

b. Support for sounding operation before AAI_SCD message is received at MS : Need to add necessary parameters as default value

And this contribution suggest to go with option a.

3. Offset for sounding channel power control

Uplink power control includes the usage of offset parameter. Data channel power control and control channel power control have different offset usage with different parameters. And those parameters can be updated by message.

However D5 does not describe what offset value shall be used for sounding channel power control.

Since sounding operation is not like other UL control channels, it does not need the determined required SNR. And estimated SNR by sounding channel can be used for data scheduling. Considering these reasons, Offset for sounding channel is recommended to be same with offset of data – Offset_data.

- Proposed Text Changes

[Remedy#1: Change the line 65 in page 729 as]

------------------------------------------------Start proposed text----------------------------------------------------------------

If power boosting field in UL Sounding Command A-MAP IE is set 1, 3dB tone power boosting is applied to each TX antenna.

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy#2: Delete the line 41 in page 732 as]

------------------------------------------------Start proposed text----------------------------------------------------------------

|  | Sounding Channel SINR Target Value | 10dB |
| --- | --- | --- |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy#3: Change line 60~65 in page 729 as follows (add the blue color-coded sentence with underline at line 63 in page 729)]

------------------------------------------------Start proposed text----------------------------------------------------------------

When calculated sounding SINRTarget is higher than soundingSinrMax, the SINRTarget shall be set to soundingSinrMax.

Offset for sounding channel is set to OffsetData which is used for data channel transmission. -------------------------------------------------End proposed text----------------------------------------------------------------