| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Parameters of sleep mode in AAI_SLP-RSP |  |
| Date Submitted | 2010-4-30 |  |
| Source(s) | Guanghui Fan，Ronald Mao Huawei Technologies | E-mail: fanguanghui@huawei.com rmao@huawei.com |
| Re: | Call for LB #31a on “ P802.16m/D5”: |  |
| Abstract | This contribution proposes to reduce the overhead of the parameters of sleep mode in AAI_SLP-RSP |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 31a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Parameters of sleep mode in AAI_SLP-RSP

Guanghui Fan，Ronald Mao

Huawei Technologies

Introduction

In current AAI_SLP-RSP message, when the ABS approves the sleep mode entry request, all sleep cycle parameters are included. The total size of all the parameters is quite large. Actually, when the ABS decides to approve the request and doesn't want to change the parameters which are requested in AAL_SLP-REQ, the parameters don’t need to be re-indicated in the response message. Furthermore, if only parts of the parameters are needed to be change, the response message can only indicate the modified parameters for the purpose of reducing overhead.

This contribution proposes to use a bitmap to indicate the modified parameters in AAI_SLP-RSP. If all the required parameters needn’t to be modified by ABS, both the bitmap and the required parameters are not included in AAI_SLP-RSP, and in this case 48 bits total parameter size is decreased. If most of the parameters are need to be modified by ABS, all the parameters are included instead of using the bitmap.

Text proposal for inclusion in the P802.16m/D5

========================== Start of Proposed Text ==============================

[Editor’s Note: Modify the field descriptions in “16.2.3.25 AAI_SLP-RSP” in page 146]

16.2.3.25 AAI_SLP-RSP

The AAI_SLP-RSP message shall be sent from ABS to an AMS in response to an AAI_SLP-REQ message. The ABS may send the AAI_SLP-RSP message in unsolicited manner with Response_Code = 0b00 (i.e., Request by ABS in Unsolicited manner).

If the request sent by an AMS is rejected by an ABS, the AMS shall not retransmit the AAI_SLP-REQ message before the duration, indicated by the REQ_duration in AAI_SLP-RSP with Response_Code = 0b10 (i.e., Rejection of AAI_SLP-REQ), expires.

Table 712—Parameters for AAI_SLP-RSP

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Control Message Type |  | 8 | MAC control message type | N.A. |
| M | Response_Code |  | 2 3 | This indicates response type of AAI_SLP-RSP message. 0b00 : Request by ABS in Unsolicited manner 0b01 : Approval of AAI_SLP-REQ 0b10 : Rejection of AAI_SLP-REQ 0b11 : Reserved 0b000 : Request by ABS in Unsolicited manner 0b001 : Rejection of AAI_SLP-REQ 0b010 : Approval of AAI_SLP-REQ, no required parameter is included 0b011 : Approval of AAI_SLP-REQ, all required parameters are included 0b100: Approval of AAI_SLP-REQ, part of required parameters are included 0b101~0b111: Reserved | N.A. |
| O | Operation |  | 2 | This indicates operation type of AAI_SLP-RSP message. 0b00 : Exit Sleep Mode 0b01 : Enter Sleep Mode 0b10 : Change Sleep Mode 0b11 : Switch Sleep Cycle setting | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) When (Response_Code == 0b000 ¦¦ Response_Code == 0b010 ¦¦ Response_Code == 0b011 ¦¦ Response_Code == 0b100) |
| O | SCID |  | 4 | Sleep Cycle ID 0~15 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation != 0b00) When (Response_Code == 0b000 ¦¦ Response_Code == 0b010 ¦¦ Response_Code == 0b011 ¦¦ Response_Code == 0b100) && (Operation != 0b00) |
| O | CQI_Operation |  | 2 | 0: the CQICH assigned to the AMS is kept 1: the CQICH is de-allocated at the frame specified by Start_Frame_Number 2: the CQICH is automatically de-allocated at the beginning of the sleep window whenever the CQICH is newly assigned to the AMS during the Listening Window 0~3 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation != 0b00) When (Response_Code == 0b000 ¦¦ Response_Code == 0b010 ¦¦ Response_Code == 0b011 ¦¦ Response_Code == 0b100) && (Operation != 0b00) |
| O | Parameters Bitmap |  | 10 | This field appears when Response_Code = 0b100 and opreation code is 0b01 or 0b10, each bit in the Bitmap indicates whether the corresponding parameter is followed: Bit 0: Start Frame Number is included Bit 1: TIMF is included Bit 2: LWEF is included Bit 3: NSCF is included Bit 4: Initial Sleep Cycle is included Bit 5: Final Sleep Cycle is included Bit 6: Listening Window is included Bit 7: Listening subframe bitmap is included Bit 8: New Initial Sleep Cycle is included Bit 9: T_AMS is included | When (Response_Code == 0b100) && (Operation != 0b00) |
| O | Start Frame Number |  | 6 | Least Significant 6 bits of Frame Number. 0~63 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation != 0b00) When ((Response_Code == 0b000) ¦¦ (Response_Code == 0b011) ¦¦ (Response_Code == 0b100 && Bit 0 of Parameter Bitmap ==1)) && (Operation != 0b00) |
| O | MO | TIMF | 1 | Traffic Indication Message Flag 0 : AAI_TRF-IND message is not sent for the AMS 1 : AAI_TRF-IND message is sent to the AMS during every Listening Window | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) When ((Response_Code == 0b000) ¦¦ (Response_Code == 0b011) ¦¦ (Response_Code == 0b100 && Bit 1 of Parameter Bitmap ==1)) && (Operation != 0b00) |
|  | MO | LWEF | 1 | Listening window Extension Flag 0~1 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) When ((Response_Code == 0b000) ¦¦ (Response_Code == 0b011) ¦¦ (Response_Code == 0b100 && Bit 2 of Parameter Bitmap ==1)) && (Operation != 0b00) |
|  | MO | NSCF | 2 | Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved 0b00~0b11 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) When ((Response_Code == 0b000) ¦¦ (Response_Code == 0b011) ¦¦ (Response_Code == 0b100 && Bit 3 of Parameter Bitmap ==1)) && (Operation != 0b00) |
|  | MO | Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). 0~15 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) When ((Response_Code == 0b000) ¦¦ (Response_Code == 0b011) ¦¦ (Response_Code == 0b100 && Bit 4 of Parameter Bitmap ==1)) && (Operation != 0b00) |
|  | MO | Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). 0~1023 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) When ((Response_Code == 0b000) ¦¦ (Response_Code == 0b011) ¦¦ (Response_Code == 0b100 && Bit 5 of Parameter Bitmap ==1)) && (Operation != 0b00) |
|  | MO | Listening Window | 6 | Assigned duration of AMS's default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. 0~63 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) When ((Response_Code == 0b000) ¦¦ (Response_Code == 0b011) ¦¦ (Response_Code == 0b100 && Bit 6 of Parameter Bitmap ==1)) && (Operation != 0b00) |
|  | MO | Listening subframe bitmap | 1 8 | The bitmap indicates the sub-frames in each frame where the AMS needs to remain awake The size of the bitmap equals to the number of AAI subframes of a frame. Each bit in the bitmap indicates: 0: AMS does not wake up at the specific AAI subframe 1: AMS wakes up at the specific AAI subframe | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) When ((Response_Code == 0b000) ¦¦ (Response_Code == 0b011) ¦¦ (Response_Code == 0b100 && Bit 7 of Parameter Bitmap ==1)) && (Operation != 0b00) |
|  | O | New Initial sleep cycle | 5 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle). 0~31 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) When ((Response_Code == 0b000) ¦¦ (Response_Code == 0b011) ¦¦ (Response_Code == 0b100 && Bit 8 of Parameter Bitmap ==1)) && (Operation != 0b00) |
|  | O | T_AMS | 5 | This timer is for Listening Window Extension of AMS. 0~31 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) When ((Response_Code == 0b000) ¦¦ (Response_Code == 0b011) ¦¦ (Response_Code == 0b100 && Bit 9 of Parameter Bitmap ==1)) && (Operation != 0b00) |
| O | REQ_duration |  | 8 | Waiting value for the AAI_SLP-REQ message retransmission , which is the Least Significant 8 bits of Frame Number. 0~255 | When (Response_Code == 0b10) When (Response_Code == 0b001) |

Response_Code

This indicates response type of AAI_SLP-RSP message.

0b00 = AAI_SLP-RSP message is transmitted in unsolicited manner

0b01 = AAI_SLP-RSP message is transmitted to approve the request sent by AMS

0b10 = AAI_SLP-RSP message is transmitted to reject the request sent by AMS

Operation

This indicates operation type of AAI_SLP-RSP message.

0b00 = Approves/Requests the exit from sleep Mode

0b01 = Approves/Requests entrance to sleep Mode

0b10 = Approves/Requests the change of an existing Sleep Cycle Setting

0b10 = Approves/Requests the switch of a Sleep Cycle setting which has been negotiated since the AMS entered Sleep Mode

SCID

Assigned Sleep Cycle Identifier. The ID shall be unique within the AMS. This ID may used in further

AAI_SLP-REQ/RSP message for changing/switching the Sleep Cycle setting

LWEF

Listening window Extension Flag. If LWEF = 0, indicates that the Listening window is of fixed duration. Otherwise, it is extensible.

TIMF

1 = ABS will transmit an AAI_TRF-IND message during an AMS's Listening Window. When the ABS has DL pending data traffic for the AMS, the ABS shall inform the AMS of positive traffic indication via AAI_TRF-IND message.

0 = Traffic Indication via AAI_TRF-IND is disabled

NISCF

This indicates the inclusion of New Initial Sleep Cycle in AAI_SLP-RSP message.

Start_Frame_Number

Start frame number for first sleep window. This represents the 6 significant bits of frame number in which AMS enters the first sleep state in Sleep Mode.

Initial_Sleep_Cycle

This indicates an assigned duration for the initial sleep cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). The length of Initial Sleep Cycle shall be equal or longer than the default Listening Window.

Final Sleep Cycle

This indicates assigned duration for the final sleep cycle (measured in frames).

Listening_Window

Assigned Duration of AMS's default Listening Window (measured in frames). Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled.

Listening sub-frame bitmap

The bitmap indicates the sub-frames in each frame where the AMS needs to remain awake. Most significant bit is mapped to the 1st AAI subframe.

If this value is set to 0xF, the AMS shall remain awake during entire sub-frames in each frame during

Listening Window.

SLPID

This is an identifier assigned by the ABS when TIMF is set to 1. This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID while the AMS is still in sleep mode.

REQ-duration

Waiting value for the AAI_SLP-REQ message retransmission (measured in MAC frames) when AMS's request is rejected by ABS: the AMS may retransmit the AAI_SLP-REQ message after at least the frame designated by REQ-duration.

New Initial Sleep Cycle

When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle shall be reset to Initial Sleep Cycle.

T_AMS

This timer is required in AMS for Listening Window Extension. If LWEF =1, it shall be included in AAI_SLP-RSP.

========================== End of Proposed Text ==============================