| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SFH_info_in_NBR_ADV |  |
| Date Submitted | 2010-4-30 |  |
| Source(s) | Guanghui Fan, Lei Jin,Bin Chen Huawei Technologies | E-mail: fanguanghui@huawei.com jinlei60020191@huawei.com binchen@huawei.com |
| Re: | Call for LB #31a on “ P802.16m/D5” |  |
| Abstract | This contribution proposes about SFH information in AAI_NBR-ADV |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 31a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SFH_info_in_NBR_ADV

Bin Chen，Lei Jin, Guanghui Fan

Huawei Technologies

Introduction

The Control_bitmap information is not mandatory as the SFH_encoding_format may be 0b10 which indicates no SFH is included for non-macrocell an ABS. The Note of the SFH_encoding_format for Macrocell ABS is also wrong. The word “bitmap” shall be modified to “SFH_encoding_format ”.

Text proposal for inclusion in the P802.16m/D5

========================== Start of Proposed Text ==============================

[Editor’s Note 1 : modify the following field description in “16.2.3.12 AAI_NBR-ADV” in page 112]

Table 693—AAI_NBR-ADV parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | S-SFH change count | 4 | Indicates the value of S-SFH change count of this neighbor ABS | Shall be included for each carrier of each neighbor BS |
| M | SFH_encoding_format | 2 | 0b00: full Subpkt information 0b01: delta encoding with reference to the information of current carrier which transmits this AAI_NBR-ADV message. 0b10: no SFH included 0b11: delta encoding with reference to the information of the preceding carrier For macrocell ABS, the bitmap SFH_encoding_format shall be either 0b00 or 0b01 or 0b11. | Shall be included for each neighbor BS |
| M O | Control_bitmap | 4 | Each bit maps to one SFH subpacket Bit[0]: SFH SP1 Bit[1]: SFH SP2 Bit[2]: SFH SP3 Bit[3]: reserved | Shall be included for each neighbor BS when SFH_encoding_format is not 0b10 |

========================== End of Proposed Text ==============================