| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Idle mode entry in legacy ASN |  |
| Date Submitted | 2010-4-30 |  |
| Source(s) | Guanghui Fan,Yanhong Wang, Jia Lin,Li Li Huawei Technologies | E-mail: fanguanghui@huawei.com wangyanhong@huawei.com linjia@huawei.com lily08@huawei.com |
| Re: | Call for LB #31a on “ P802.16m/D5”: |  |
| Abstract | This contribution proposes about idle mode entry in legacy ASN |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 31a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Idle Mode Entry in Legacy ASN

Guanghui Fan,Yanhong Wang, Jia Lin,Li Li

Huawei Technologies

Introduction

In the deployment scenario that the ABS is connected to the legacy ASN elements, the idle mode entry procedure shall be different than in the total 16m advanced network. The parameters of idle mode in legacy network are in unit of frames while they are in unit of superframes in advanced network and their allowed value are also different.

This contribution proposes an option that how the AMS entries idle mode when the ABS is in a legacy ASN network.

The proposed option assumes that the ABS is a co-existence system which has both LZone and MZone.

Text proposal for inclusion in the P802.16m/D5

========================== Start of Proposed Text ==============================

[Editor’s Note 1 :change the field description in “16.2.3.21 AAI_DREG-RSP message” in page 137]

| M/O | Attributes/ Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Action code | 4 | Used to indicate the purpose of this message 0x00: AMS shall immediately terminate service with the ABS and should attempt network entry at another ABS 0x01: AMS shall listen to the current ABS but shall not transmit until a RES-CMD message or AAI_DREG-RSP message with action code 0x02 or 0x03 is received. 0x02: AMS shall listen to the current ABS but only transmit on the control connection. 0x03: AMS shall return to normal operation and may transmit on any of its active connections. 0x04: This option is valid in response to a AAI_DREG-REQ message with De-Registration Request Code=0x00. The AMS shall terminate current Normal Operation with the ABS. 0x05: AMS shall begin idle mode initiation: a) to signal AMS to begin idle mode in unsolicited manner or b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration 0x06: This option is valid only in response to a AAI_DREG-REQ message with De-Registration Code 0x01: a) to reject AMS-initiated idle mode request or b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration 0x07: This option is valid in response to a AAI_DREG-REQ message with De-registration-request-code= 0x01. 0x08: This option is valid only in response to an AAI_DREG-REQ message with De-Registration Request Code 0x04 to allow retention of the AMS's connection information 0x09: This option is valid only in response to an AAI_DREG-REQ message with De-Registration Request Code 0x04 to reject retention of the AMS's connection information. 0x10: This option is valid only when the ABS is in WirelessMAN-OFDMA network configuration and decides to signal the AMS to begin idle mode or to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration. In this case, the AMS shall enter idle mode in MZone and listen to the paging messages in LZone of the ABS. 0x10-0x15: reserved 0x11-0x15: reserved | N.A. |
| O | Paging cycle | If (Action code==0x05 or 0x07) 4 If (Action code==0x10) 16 | Used to indicate Paging cycle for the AMS If (Action code==0x05 or 0x07) 0x00: 8 superframes 0x01: 16 superframes 0x02: 32 superframes 0x03-0x15: reserved If (Action code==0x10) The value is in unit of frames. | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 or 0x 10 |
| O | Paging offset | If (Action code==0x05 or 0x07) 4 If (Action code==0x10) 16 | Used to indicate Paging offset for the AMS If (Action code==0x05 or 0x07) The value is in unit of superframes If (Action code==0x10) The value is in unit of frames | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 or 0x10 |
| O | Paging listening interval | 8 | The value is in unit of frames. | Shall be present when Action code in AAI_DREG-RSP message is set 0x 10 |
| O | Time offset between LZone and MZone | TBD | The value is in unit of subframes. | Shall be present when Action code in AAI_DREG-RSP message is set 0x 10 |
| O | Paging controller ID | 48 | Used to indicate Paging controller which manages and retains the MS's idle mode information 0..248–1 | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| O | Paging group ID | 16 | Used to indicate Paging group which the MS is located in 0..216–1 | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| O | Deregistration ID | 10 | Used to indicate Deregistration ID used to identify the MS in idle mode 0..210–1 | Shall be present when the Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 Shall not be present when the AMS is connected to the WirelessMAN-OFDMA network |
| O | Idle Mode Retain Information element | 4 | Provided as part of this message indicative only. Network reentry from idle mode process requirements may change at time of actual reentry. For each bit location, a value of 0 indicates the information for the associated reentry control messages shall not be retained and managed; a value of 1 indicates the information for the associated reentry control message shall be retained and managed. Bit 0: Retain MS service and operational information associated with AAI_SBC-REQ/RSP messages. Bit 1: Retain MS service and operational information associated with AAI_PKM-REQ/RSP messages. Bit 2: Retain MS service and operational information associated with AAI_REG-REQ/RSP messages. Bit 3: Retain MS service and operational information associated with network address. Bit 4: Retain MS state information. The information retained by setting bit 4 does not include the information associated with AAI_SBC-REQ/RSP messages, AAI_PKM-REQ/RSP messages, AAI_REG-REQ/RSP messages, network address unless otherwise specified by setting one or more bits 0-3. This information does not include blocks currently in the ARQ window or associated timers. The MS state information does include SFIDs and related description (QoS descriptors and CS classifier information) for all Service Flows that the AMS has currently established as well as any SAs with their related keying information. | Shall be Present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 or 0x08 |
| O | REQ-Duration | 8 | Used to indicate waiting value for the AAI_DREG-REQ message with De-registration_Request_Code=0x01 0..28-1 : measured in frames | May be present when Action code is set to 0x05 or 0x10 |

[Editor’s Note 2 :change the text as follows in page 380]

16.2.18.1.1 AMS initiated

In case of AMS initiated idle mode entry, an AMS may signal intent to begin idle mode by sending a AAI_DREG-REQ message with the De-registration_Request_Code parameter = 0x01 that indicates request for AMS deregistration from serving ABS and initiation of AMS idle mode. The AMS may request the paging controller to retain specific AMS service and operational information for idle mode management purposes through inclusion of the Idle Mode Retain Information element in the AAI_DREG-REQ control message. When the ABS decides to allow AMS-initiated idle mode request, the ABS shall send a AAI_DREG-RSP with action code 0x07 in response to the AAI_DREG-REQ message or with action code 0x10 to allow the AMS to enter idle mode in MZone and listen to the paging messages in LZone according to the legacy paging parameters in AAI_DREG-REQ if the ABS is a co-existence system and connected to the WirelessMAN-OFDMA network. When the ABS decides to reject AMS-initiated idle mode request, the ABS shall send a AAI_DREG-RSP with action code 0x06 in response to this AAI_DREG-REQ message. ABS may include REQ-Duration in this AAI_DREG-RSP message. In this case, the AMS may retransmit the AAI_DREG-REQ message after the expiration of REQ_Duration. If the AMS does not receive the AAI_DREG-RSP message within T45 timer expiry after it sends the AAI_DREG-REQ message to the ABS, the AMS shall retransmit the AAI_DREG-REQ message as long as DREG Request Retry Count has not been exhausted. Otherwise, the AMS shall reinitialize MAC, reselect its preferred ABS and perform network reentry with its preferred ABS. The ABS shall start Management_Resource_Holding_Timer to maintain connection information with the AMS as soon as it sends the AAI_DREG-RSP message with action code 0x07 or 0x10 to the AMS. If Management_Resource_Holding_Timer has been expired, the ABS shall release connection information with the AMS. The operation of idle mode entry during AMS initiated idle mode is shown in Figure 444 and Figure 445.

[Editor’s Note 3 :change the text as follows in page 382]

16.2.18.1.2 ABS initiated

Using ABS initiated idle mode entry, a serving ABS may signal for an AMS to begin idle mode by sending an AAI_DREG-RSP message with action code 0x05 or 0x10 in unsolicited manner. This unsolicited AAI_DREG-RSP may include REQ-Duration. When an AMS receives an unsolicited AAI_DREG-RSP without REQ_Duration, the AMS shall immediately start the idle mode initiation procedures. This method of ABS initiated idle mode entry is defined as Type 1 ABS-initiated idle mode entry. In case of Type 1 ABS-initiated idle mode entry, after sending the AAI_DREG-RSP message with action code 0x05 or 0x10, the serving ABS shall start T46 timer as well as Management_Resource_Holding_Timer at the same time. If the ABS does not receive the AAI_DREG-REQ message with the De-registration_Request_Code parameter = 0x02 or the AAI_DREG-REQ message with the De-registration_Request_Code parameter = 0x03 from the AMS in response to the unsolicited AAI_DREG-RSP message with action code 0x05 or 0x10 within T46 timer expiry, the ABS shall retransmit the AAI_DREG-RSP message with action code 0x05 or 0x10 in unsolicited manner as long as DREG command retry count has not been exhausted. When the AMS sends the AAI_DREG-REQ message with the De-registration_Request Code parameter = 0x02 in response to the unsolicited AAI_DREG-RSP message with action code 0x05 or 0x10, the AMS shall set the polling bit to 1 in MCEH. The AMS shall not enter idle mode immediately after sending a message indicating that it will enter idle mode, because that message could get lost in the air link. Rather, the AMS shall wait for AAI_MSG-ACK message from the ABS. If the AMS detects the AAI_MSG-ACK message, the AMS shall enter idle mode. Otherwise, the AMS shall resend the same AAI_DREG-REQ message with the request for the AAI_MSG-ACK message. If the AMS has a pending UL data to transmit, it shall send AAI_DREG-REQ message with De-registration_Request Code parameter = 0x03 in response to the unsolicited AAI_DREG-RSP message with action code 0x05 or 0x10 by the ABS. These procedures are illustrated in Type 1 in Figure 446, Figure 447 and Figure 448.

As another case of ABS initiated idle mode entry defined as Type 2 ABS-initiated idle mode entry, the serving ABS may also include a REQ-duration with an Action Code = 0x05 or 0x10 in the unsolicited AAI_DREG-RSP, signaling for an AMS to initiate an Idle Mode request through a AAI_DREG-REQ with De-registration_Request_Code = 0x01 at REQ-duration expiration. In this case, ABS shall not start T46 timer. AMS may include Idle Mode Retain Information with in AAI_DREG-REQ message with De-Registration_Request Code = 0x01 transmitted at the REQ-duration expiration. If the ABS receives the AAI_DREG-REQ with De-registration_Request_Code=0x01, the ABS shall transmit another AAI_DREG-RSP message with Action Code=0x07 including Idle Mode Retain Information. These procedures are illustrated in Type 2 in Figure 446, Figure 449 and Figure 450.

[Editor’s Note 3 :change the action code of AAI_DREG-RSP “0x05” to “0x05 or 0x07” in figure 446/ 447/448/449/450]

[Editor’s Note 4 :change the text as follows in page 389]

16.2.18.2 Operation during Idle mode

16.2.18.2.4 Operation during idle mode in LZone

If the AMS enters idle mode in MZone indicated by AAI_DREG-REQ with an Action Code=0b10, the AMS shall turn to work in LZone according to the time offset between MZone and LZone and listen to the paging messages  in LZone according to the legacy paging parameters indicated in AAI_DREG-REQ .

[Editor’s Note 5 : add the following text to page 835, modify the Action Code of legacy MOB_PAG-ADV]

16.10.2 Support for paging in idle mode

The encodings in Table 978 are specific to the PAG-ADV message sent in LZone of an ABS supporting WirelessMAN-OFDMA Reference System.

Table 978—MOB_PAG-ADV message

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| MOB_PAG-ADV_Message_format() { | - | - |
| Management Message Type = 61 | 8 | - |
| Num_Paging_Group_IDs | 8 | Number of Paging Group IDs in this message |
| for (i = 0; i < Num_Paging_Group_IDs; i++) { | - | - |
| Paging Group ID | 16 | - |
| } |  | - |
| Num_MACs |  | Number of MS MAC addresses |
| For (j = 0; j < Num_MACs; j++) { |  | - |
| MS MAC Address hash |  | The hash is obtained by computing a CRC24 on the MS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB |
| Action Code |  | Paging action instruction to MS 0b00 = No action required 0b01 = Perform ranging to establish location and acknowledge message 0b10 = Enter network (For AMS, Enter network in LZone) 0b11 = Enter network in MZone (for AMS only) |
| Reserved |  | - |
| } |  | - |
| Padding |  | Padding bits to ensure octet aligned |
| TLV Encoded Information |  | TLV-specific |
| } |  | - |

========================== End of Proposed Text ==============================