| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for MultiBS MIMO operation |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Shuangfeng Han, Sangwoo Lee, Taeyong Kim, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. Wookbong Lee LG Electronics Luciano Sarperi Fujitsu | shuangf.han@samsung.com wookbong.lee@lge.com luciano.sarperi@uk.fujitsu.com |
| Re: | Call for comments for Letter Ballot #31a |  |
| Abstract | The contribution proposes some text change to multiBS MIMO section of 802.16m D5. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

D5 Text Proposal for MultiBS MIMO operation

Shuangfeng Han, Sangwoo Lee, Taeyong Kim, Hokyu Choi, Heewon Kang

Samsung Electronics

Wookbong Lee

LG Electronics

Luciano Sarperi

Fujitsu

- Introduction

This contribution includes proposed text to Multi-BS MIMO in the 802.16m D5 document.

- Motivation of modifications

- The following proposed texts were accepted in March meeting, but not correctly implemented in D5. So we propose the text changes again.

- Proposed texts:

Remedy #1: To modify section 16.2.3.39

-------------------------------------------------- Text Start ---------------------------------------------------

16.2.3.39 AAI_MULTI_BS_MIMO-REQ

The AAI_MULTI_BS_MIMO-REQ message shall be transmitted by the AMS to report its preference on single BS precoding with Multi-BS MIMO coordination or multi-BS joint MIMO processing.

---ASN.1START

AAIMULTIBSMIMOREQ Message::SEQUENCE{

Multi-BS MIMO Request    Boolean,

NumBS	INTEGER (1..8)

-- NIPParam shall be transmitted from strongest to weakest

NIPList::=SEQUENCE (SIZE (1..NumBS)) OF NIPParam

}

NIPParam::=SEQUENCE{

Temp_BSID   INTEGER (0..15),

NIP Value   INTEGER (0..15)

}

--ASN1STOP

Table 725—AAI_MULTI_BS_MIMO-REQ message format

| Name | Value | Usage |
| --- | --- | --- |
| Multi-BS MIMO Request |  | 0: Single-BS precoding with Multi- BS Coordination 1: Multi-BS Joint MIMO Processing |
| NIP Value |  | Absolute NIP value encoded as difference to NIP_th_1 when Multi- BS MIMO Request = 0. The value shall be interpreted as unsigned 2bits with units of 0.5 dB, 0b00 is interpreted as NIP_th_1 and 0b11 is interpreted as NIP_th_1+1.5dB or higher. Absolute NIP value encoded as difference to NIP_th_2 when Multi- BS MIMO Request = 1. The value shall be interpreted as unsigned 2bits with units of 0.5 dB, 0b00 is interpreted as NIP_th_2 and 0b11 is interpreted as NIP_th_2+1.5dB or higher. |
| NumBS | 3 bits | Number of base station which exceeds threshold |
| Temp_BSID | 4 bits | Temp_BSID is broadcasted through AAI_DL_IM message |

Table 725: Parameters for AAI_MULTI_BS_MIMO-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | Type of AAI_MULTI_BS_MIMO-REQ | NA |
| M | multiBsMimoRequest | 1 | 0b0: Single-BS precoding with Multi-BS Coordination 0b1: Multi-BS Joint MIMO Processing | NA |
| O | nipValueForSingleBS | 2 | NIP value encoded as difference to NIP_th_1 when multiBsMimoRequest = 0b0. The value shall be interpreted as unsigned 2bits with units of 0.5dB, 0b00 is interpreted as NIP_th1 and 0b11 is interpreted as NIP_th1+1.5dB or higher | Present when multiBsMimoRequest = 0b0 |
| O | numbs | 3 | Number of base station which exceeds threshold | Present when multiBsMimoRequest = 0b0 |
| O | tempBsid | 4 | This represents Temp_BSID. Temp_BSID is broadcasted through AAI_DL_IM message | Present when multiBsMimoRequest = 0b0 |
| O | bitmapForRequestedAdjABSs | 8 | 0b0: ABS is not involved in multiBS MIMO 0b1: ABS is involved in multiBS MIMO Each bit in this bitmap represents one ABS and the i-th bit (i=1,…,8) represents the neighboring ABS (listed in AAI-DL-IM message) with the i-th strongest channel to the AMS. Please refer 16.5.1.4. | Present when multiBsMimoRequest = 0b1 |
| O | nipValueForJoint | 2 | Sum NIP value encoded as difference to NIP_th_2 when multiBsMimoRequest = 0b1. The value shall be interpreted as unsigned 2bits with units of 0.5dB, 0b00 is interpreted as NIP_th2 and 0b11 is interpreted as NIP_th2+1.5dB or higher | Present when multiBsMimoRequest = 0b1 |

------------------------------------------------- Text End---------------------------------------------------------

Remedy #2: To modify section 16.2.3.40 AAI_MULTI_BS_MIMO-RSP Message

-------------------------------------------------- Text Start ---------------------------------------------------

16.2.3.40 AAI_MULTI_BS_MIMO-RSP

The AAI_MULTI_BS_MIMO-RSP message shall be transmitted by the ABS to AMS indicating which of the adjacent ABSs listed in previously broadcasted AAI-DL-IMAAI_NBR-Adv message are involved in multiBS MIMO joint processing operation.

---ASN.1START

AAIMULTIBSMIMORSP Message::SEQUENCE{

BitmapforinvolvedadjacentABSsinmultiBSMIMO BIT STRING (SIZE (8))

}

[Replace Table 726 with the following table]

Table 726: AAI_MULTI_BS_MIMO-RSP

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | Type of AAI_MULTI_BS_MIMO-RSP | NA |
| M | adjAbsBitmapMultiBSMIMO | 8 | 0: ABS is not involved in multiBS MIMO 1: ABS is involved in multiBS MIMO Each bit in this bitmap represents one ABS and the i-th bit (i=1,…,8) represents the neighboring ABS (listed in AAI-DL-IM message) with the i-th strongest channel to the AMS. Please refer 16.5.1.4. | NA |

-------------------------------------------------- Text End ---------------------------------------------------

Remedy #3:

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

-------------------------------------------------- Text Start ---------------------------------------------------

[Add the following ASN.1 code to Annex P.2]

-- Multiple BS MIMO Request

AAI-MultiBS-MIMO-REQ ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

multiBsMimoRequest             ENUMERATED {

singleBsPrecoding   (0),

jointMimoProcessing (1)},

numBs                          INTEGER (1..8) OPTIONAL,

nIPList                        NIPList OPTIONAL,

bitmapForRequestedAdjABSs  BIT STRING (SIZE(8)) OPTIONAL,

nipValueForJoint             INTEGER (0..3) OPTIONAL

}

-- NIPParam shall be transmitted from strongest to weakest

-- Number of NIPParam report is same as numBs

NIPList ::= SEQUENCE {

nipParamForBS1                      NIPParam,

nipParamForBS2                      NIPParam OPTIONAL,

nipParamForBS3                      NIPParam OPTIONAL,

nipParamForBS4                      NIPParam OPTIONAL,

nipParamForBS5                      NIPParam OPTIONAL,

nipParamForBS6                      NIPParam OPTIONAL,

nipParamForBS7                      NIPParam OPTIONAL,

nipParamForBS8                      NIPParam OPTIONAL}

NIPParam :: SEQUENCE {

tempBsid                       INTEGER (0..15),

nipValueForSingleBS          INTEGER (0..3)}

-- Multiple BS MIMO Response

AAI-MultiBS-MIMO-RSP ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

adjAbsBitmapMultiBSMIMO      BIT STRING (SIZE(8))

}

-------------------------------------------------- Text End ---------------------------------------------------

Remedy #4:

To add a new MAC management message 16.2.3.x AAI-MULTI-BS-SOUNDING-CAL

16.2.3.x AAI-MULTI-BS-SOUNDING-CAL

The AAI-MULTI-BS-SOUNDING-CAL message shall be transmitted by the ABS to AMS indicating how AMS assists multiBS calibration in TDD systems for sounding based multiBS MIMO joint processing.

Table x- AAI-MULTI-BS-SOUNDING-CAL

| Name | Size (bit) | Usage |
| --- | --- | --- |
| Superframe number | 2 | The superframe number where multiBS sounding calibration is scheduled If this message is transmitted in i-th superframe, then the sounding signal shall be transmitted in i+Superframe number superframe 0b00: 2 0b01: 4 0b10: 6 0b11: 8 |
| Frame number | 2 | The frame number where multiBS sounding calibration is scheduled to start from |
| Sounding AAI subframe | 3 | The AAI subframe where multiBS sounding calibration is scheduled to start from |
| Sounding subband bitmap | Variable Max. 24 | FFT size dependent |
| Decimation offset d | 5 | Unique decimation offset |

Upon receiving AAI-MULTI-BS-SOUNDING-CAL, the AMS shall send calibration sounding sequence defined in section 16.5.1.3.1 at consecutive certain number (the number of ‘1’s in adjAbsBitmapMultiBSMIMO in AAI-MULTI-BS-MIMO-RSP plus 1) of sounding subframes. The first calibration sounding subframe is mapped to the serving ABS and the i-th calibration sounding subframe is mapped to the i-th involved adjacent ABS in adjAbsBitmapMultiBSMIMO.

---------------------------------Text End-------------------------------------------------