| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Clean up for AAI_DSx message in IEEE 802.16m |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Nan Li Kaiying Lv Bo Sun Yang Liu ZTE Corporation | E-mail: lv.kaiying@zte.com.cn |
| Re: | LB31a / Chapter 16.2.3.54(DSx MAC Control Message) |  |
| Abstract | This contribution proposes text to Clean up AAI_DSX-xxx. |  |
| Purpose | To be discussed and adopted. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Clean up for AAI_DSx message in IEEE 802.16m

Nan Li, Kaiying Lv, Bo Sun,Yang Liu

ZTE Corporation

# 1. Introduction

According to IEEE 802.16m/D5, there are MBS service TLV, MBS Zone Identifier as well as E-MBS service TLV and E-MBS zone ID in AAI_DSA-REQ/RSP message. Actually, MBS service TLV and E-MBS service TLV are nearly the same in function. MBS Zone Identifier and E-MBS zone ID have the same size. There is the same problem with AAI_DSC-REQ/RSP message. Besides, E-MBS service TLV is missing in this message.

We suggest combining these two groups of parameters in both AAI_DSA-REQ/RSP and AAI_DSC-REQ/RSP message. An MBS/E-MBS indicator is proposed to indicate whether the connections for associated service flows belong to a legacy zone or an AAI zone.

# 2. Text Proposal

============================== Start of Proposed Text ============================

[Change Table 740 in 16.2.3.45.1 AAI_DSA-REQ as follows:]

Table 740—AAI_DSA-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| O | MBS service TLV | 8 | 0: No available MBS 1: MBS in Serving BS Only 2: MBS in a multi-BS Zone supporting macro-diversity 3: MBS in a multi-BS Zone not sup-porting macro-diversity 4-255: Reserved |  |
| O | MBS Zone Identifier Assignment parameter | 8 | MBS zone identifier (bits 6 through 0 are the MBS Zone Identifier, bit 7 is set to 0) |  |
| O | E-MBS Service | 12 | Indicates whether the MBS service is being requested or provided for the connection that is being setup. 0: No available E-MBS 1: E-MBS in Serving ABS Only 2: E-MBS in a multi-ABS Zone supporting macro-diversity 3: E-MBS in a multi-ABS Zone not supporting macro-diversity | included in both ABS/AMS ini- tiated DSA- REQ |
| O | E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flowis valid. |  |
| O | MBS/E-MBS indicator | 1 | 0:MBS service 1:E-MBS service | This parameter indicates whether the connection for associated service flows belongs to a legacy zone or an AAI zone. |
| O | MBS/E-MBS service | 4 | 0: No available MBS/E-MBS 1: MBS/E-MBS in Serving BS Only 2: MBS/E-MBS in a multi-BS Zone supporting macro-diversity 3: MBS/E-MBS in a multi-BS Zone not sup-porting macro-diversity | If the value of MBS/E-MBS indicator is 0, it presents MBS; otherwise it presents E-MBS. |
| O | MBS/E-MBS Zone ID | 8 |  | If the value of MBS/E-MBS indicator is 0, it presents MBS; otherwise it presents E-MBS. |

[Change text in Page 190, Line 25 as follows:]

…

When an ABS updates E-MBS service flow, the following parameters may be included in the AAI_DSC-REQ message.

— E-MBS Service: Indicates whether the MBS service is being requested or provided for the connection that being setup.

— E-MBS Zone ID: Indicates E-MBS zone ID where the connection for associated service flow is valid

— E-MBS Service Flow Parameter: Mapping updates of E-MBS ID and FID.

- Physical Carrier Index: Target carrier which the AMS switches or is redirected by ABS to, only included in ABS initiated DSC-REQ.

…

[Change Table 743 in 16.2.3.45.4 AAI_DSC-REQ as follows:]

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| O | MBS service TLV | 8 | 0: No available MBS 1: MBS in Serving BS Only 2: MBS in a multi-BS Zone supporting macro-diversity 3: MBS in a multi-BS Zone not sup-porting macro-diversity 4-255: Reserved |  |
| O | MBS Zone Identifier Assignment parameter | 8 | MBS zone identifier (bits 6 through 0 are the MBS Zone Identifier, bit 7 is set to 0) |  |
| O | E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flowis valid. |  |
| O | MBS/E-MBS indicator | 1 | 0:MBS service 1:E-MBS service | This parameter indicates whether the connection for associated service flows belongs to a legacy zone or an AAI zone. |
| O | MBS/E-MBS service | 4 | 0: No available MBS/E-MBS 1: MBS/E-MBS in Serving BS Only 2: MBS/E-MBS in a multi-BS Zone supporting macro-diversity 3: MBS/E-MBS in a multi-BS Zone not sup-porting macro-diversity | If the value of MBS/E-MBS indicator is 0, it presents MBS; otherwise it presents E-MBS. |
| M | MBS/E-MBS Zone ID | 8 |  | If the value of MBS/E-MBS indicator is 0, it presents MBS; otherwise it presents E-MBS. |

============================== End of Proposed Text ========================