| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Multicarrier Capability Negotiation |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Nan Li Yuqin Chen Kaiying Lv Yang Liu ZTE Corporation | E-mail: li.nan25@zte.com.cn |
| Re: | LB31a / Chapter 16.2.8.2.3 (Network Entry) |  |
| Abstract | This contribution proposes text to clarify Multicarrier capability negotiation operation. |  |
| Purpose | To be discussed and adopted. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Multicarrier Capability Negotiation

Nan Li, Yuqin Chen,Kaiying Lv, Yang Liu

ZTE Corporation

# 1. Introduction

In the current IEEE 802.16m/D5, it is described that during the initial network entry, AMS will inform ABS of its support of multicarrier transmission by AAI_REG-REQ message and the ABS will indicate if it supports any of multicarrier modes for that AMS through AAI_REG-RSP message. However, multicarrier capability parameter is missing in AAI_REG-RSP message.

Another problem is that an ABS may simultaneously support multiple MC modes. Two Bits are not enough for an ABS to indicate its multicarrier capability to the AMS.

# 2. Text Proposal

============================== Start of Proposed Text ============================

[Change Table 688 and Table 689 as follows:]

Table 689—AAI_REG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| O | Multicarrier capabilities | 24 | 0b00: No MC modes 0b01: Basic MC mode 0b10: Multicarrier Aggregation 0b11: Multicarrier Switching If Bit#0 is set to 1, it indicates the AMS supports no MC modes; If Bit#1 is set to 1, it indicates the AMS supports Basic MC mode; If Bit#2 is set to 1, it indicates the AMS supports Multicarrier Aggregation; If Bit#3 is set to 1, it indicates the AMS supports Multicarrier Switching. |  |
| … |  |  |  |  |

Table 689—AAI_REG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| O | Multicarrier capabilities | 4 | If Bit#0 is set to 1, it indicates the ABS supports no MC modes; If Bit#1 is set to 1, it indicates the ABS supports Basic MC mode; If Bit#2 is set to 1, it indicates the ABS supports Multicarrier Aggregation; If Bit#3 is set to 1, it indicates the ABS supports Multicarrier Switching. |  |
| … |  |  |  |  |

[Change text in Page 298, Line 48 as follows:]

…

The basic multicarrier capability exchange uses a two four bit code in AAI_REG-REQ/RSP message with the following indications: as defined in section 16.2.3.7 and 16.2.3.8.

Table 773—Multicarrier capability in AAI_REG-REQ/RSP message

| b1, b2 | Multicarrier Capabilities |
| --- | --- |
| 00 | No MC modes |
| 01 | Basic MC mode |
| 10 | Multicarrier Aggregation |
| 11 | Multicarrier Switching |

============================== End of Proposed Text ========================