| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Parameter clean-up for DSx messages (16.2.3.45.1) |  |
| Date Submitted | 2010-05-04 |  |
| Source(s) | Hyunjeong Kang, Youngkyo Baek, Jaehyuk Jang, Youngbin Chang, Jungshin Park and Hyunkyu Yu Samsung Electronics Co., Ltd. | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16-10/0032: “IEEE 802.16 Working Group Letter Ballot #31a: Announcement” |  |
| Abstract | The contribution proposes a clean-up text for parameters in DSx messages. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Parameter clean-up for DSx messages

Hyunjeong Kang, Youngkyo Baek, Jaehyuk Jang, Youngbin Chang, Jungshin Park and Hyunkyu Yu

Samsung Electronics Co., Ltd.

- Introduction

In current 16m specification, parameters for service flow/convergence sublayer, AAI_DSA-REQ and AAI_DSC-REQ messages are defined.

But some parameters such as PSC (power saving class) assignment, MBS service and PDU SN Extended Subheaders for HARQ Reordering are useless in 16m because the operation using the corresponding parameters is applied only in 16e but it is not applicable in 16m.

Some fields of defined parameter will not be used in 16m system. For example, IEEE 802.3/Ethernet destination MAC address, IEEE 802.3/Ethernet source MAC address, Ethertype/IEEE 802.2 SAP, etc in Packet Classification Rule shall not be included in AAI_DSx messages.

The following service flow parameters may not be changed and shall not be present in AAI_DSC-REQ message:

- UL service flow scheduling type

- Type of data delivery services

- Request/Transmission policy

- Convergence sublayer specification

- ARQ parameters

- Target SAID

- HARQ channel mapping

In addition, the value or usage or description of the defined parameters is identified in this contribution.

This contribution proposes to clean up service flow/convergence sublayer parameters and the parameters in AAI_DSA-REQ and AAI_DSC-REQ messages.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:] Service Flow Encoding/Convergence Sublayer Parameters

[Change Table 784 on line 54 of page 332 in section 16.2.12.8 as follows:]

| Fields | Size (bits) | Description |
| --- | --- | --- |
| Flow ID | 4 | An identifier of a service flow |
| SFID | 32 |  |
| Uplink/Downlink Indicator | 1 | Whether parameters are for uplink or for downlink |
| Differentiated BR timer | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s) |
| MAC in-order delivery indicator | 1 | Indicate whether or not the order of delivery in non-ARQ connection is preserved by the MAC. 0 : not preserved 1 : preserved |
| GPI primary | 16 | Primary Grant and Polling Interval in unit of milliseconds. |
| Grant size primary | 16 | Primary grant size. If the primary grant size equals to x bytes (the newly defined bandwidth request header size), this indicates a primarily polling based service; otherwise, it is primarily granting based service. |
| GPI secondary | 16 | Secondary Grant and Polling Interval in unit of milliseconds. |
| Grant size secondary | 8 | Secondary grant size (optional). If the secondary grant size is defined and equals to x bytes (the newly defined bandwidth request header size), this indicates a secondarily polling based; otherwise, it is a secondarily granting based service. |
| Adaptation method | 2 | - 00: Implicit adaptation - 01: Explicit adaptation, sustained - 10: Explicit adaptation, one time only |
| Service Class Name | 16 ~ 1024 | Refer to 11.13.3 Null-terminated string of ASCII characters. The length of the string, including null-terminator may not exceed 128 bytes. Present when a predefined BS service configuration is used for this service flow. |
| QoS Parameter Set Type | 3 | Refer to 11.13.4 Bit 0: Provisioned Set Bit 1: Admitted Set Bit 2: Active Set |
| Traffic Priority | 3 | Refer to 11.13.5 0 to 7-Higher numbers indicate higher priority Default 0 |
| Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1 |
| Maximum Sustained Traffic Rate | 326 | Rate (in bits per second) Refer to 11.13.6 for the usage of this parameter |
| Maximum Traffic Burst | 326 | Burst size (bytes) Refer to 11.13.7 for the usage of this parameter |
| Minimum Reserved Traffic Rate | 326 | Rate (in bits per second) Refer to 11.13.8 for the usage of this parameter |
| Vendor-Specific QoS Parameter | variable | Refer to 11.13.9 for the usage of this parameter 3byte-Vendor ID shall be the first information of this parameter |
| Uplink Grant Scheduling Type | 3 | Refer to 11.13.11. In addition to the legacy scheduling services described in 11.13.10, aGP Service is supported. 0: Reserved 1: Undefined (BS implementation-dependent) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service |
| Request/Transmission Policy | 8 | Refer to 11.13.11 Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to'0' and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to '1', none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 6 is set to'0' and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 6 is set to '1', none of the SDUs shall be compressed. |
| Tolerated Jitter | 326 | Milliseconds Refer to 11.13.12 for the usage of this parameter |
| Maximum Latency | 326 | Milliseconds Refer to 11.13.13 for the usage of this parameter |
| Fixed-length versus Variable-length SDU Indicator | 1 | Refer to 11.13.14 |
| SDU Size | 8 | Refer to 11.13.15 |
| Target SAID | 816 | Refer to 11.13.16 SAID onto which service flow is mapped |
| ARQ Enable | 1 | Refer to 11.13.17.1 0 = ARQ Not Requested/Accepted 1 = ARQ Requested/Accepted |
| ARQ_WINDOW_SIZE | 16 | Refer to 11.13.17.2 >0 and <=(ARQ_BSN_MODULUS/2) |
| ARQ_BLOCK_LIFETIME | 16 | Refer to 11.13.17.4 0 = Infinite 1-6553500s (100 s granularity) |
| ARQ_SYNC_LOSS_TIMEOUT | 16 | Refer to 11.13.17.5 0 = Infinite 1-6553500s (100 s granularity) |
| ARQ_RX_PURGE_TIMEOUT | 16 | Refer to 11.13.17.7 0 = Infinite 1-6553500s (100 s granularity) |
| ARQ_SUB_BLOCK_SIZE | 38 | Bit 0–2: encoding for selected block size (P), where the selected block size is equal to 2^(P+3), 0<=P<=7. ARQ sub-block size is byte unit. |
| ARQ_ERROR_DETECTION_TIMEOUT | 16 | 0 = Infinite 1-6553500s (100 s granularity) |
| ARQ_FEEDBACK_POLL_RETRY_TIMEOUT | 16 | 0 = Infinite 1-6553500s (100 s granularity) |
| RECEIVER_ARQ_ACK_PROCESSING_TIME | 8 | 0-255 |
| CS Specification | 8variable | Refer to 11.13.18 0: reserved 1: Packet, IPv4 2: Packet, IPv6 3: reserved 4: Reserved 5: reserved 6: reserved 7: Reserved 8: Reserved 9: reserved 10: Reserved 11: Reserved 12: Reserved 13: Reserved 14: Packet, IPb 15: Multiprotocol flow 16-255 Reserved (b: SDUs for service flows of this CS type may carry either IPv4 or IPv6 in the header-compressed payload) |
| CS encodingRule | 4 | 99 reserved 100 Packet, IPv4 101 Packet, IPv6 102 reserved 103 reserved 104 reserved 105 reserved 106 Reserved 107 Reserved 108 Reserved 109 Reserved 110 Reserved 111 Reserved 112 reserved 113 Packet IP 114 Packet, Multiprotocol flow |
| Unsolicited Grant Interval | 166 | Milliseconds Refer to 11.13.19 for the usage of this parameter |
| Unsolicited Polling Interval | 166 | Milliseconds Refer to 11.13.20 for the usage of this parameter |
| MBS service | 2 | Refer to 11.13.22 |
| Global Service Class Name | variable | ISBRLPS1S2L1S3S4S5 Refer to 11.13.23 |
| I: Uplink/Downlink indicator | 1 | 0=UL 1=DL |
| S: Maximum Sustained Traffic Rate | 6 | Extensible look up Table 187 |
| B: Maximum Traffic Burst | 6 | Extensible look up Table 187 |
| R: Minimum Reserved Traffic Rate | 6 | Extensible look up Table 187 |
| L: Maximum Latency | 6 | Extensible look up Table 188 |
| P: Paging preference | 1 | 0 = No paging generation 1 = Paging generation |
| S1: Request/Transmission Policy | 8 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to'0' and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to '1', none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not compress payload headers |
| S2: Uplink Grant Scheduling Type | 3 | 0: Reserved 1: Undefined (BS implementation-dependent) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service |
| L1: Tolerated Jitter | 6 | Extensible look-up table 188 of 6.3.14.4.1. This is available only for Uplink grant scheduling type = ertPS or UGS. |
| S3: Traffic priority | 3 | This is used only for Uplink Grant Scheduling Type = rtPS, ertPS, nrtPS or BE. |
| S4: Unsolicited Grant Interval | 6 | Extensible look-up table 189 of 6.3.14.4.1. This is available only for Uplink grant scheduling type = ertPS or UGS. |
| S5: Unsolicited Polling Interval | 6 | Extensible look-up table 189 of 6.3.14.4.1. This is available only for Uplink grant scheduling type = rtPS. |
| Type of Data Delivery Services | 3 | Refer to 11.13.24 0: Unsolicited Grant Service 1: Real-Time Variable Rate Service 2: Non-Real-Time Variable Rate Service 3: Best Effort Service 4: Extended Real-Time Variable Rate Service |
| SDU Inter-arrival Interval | 16 | Refer to 11.13.25 SDU inter-arrival interval in the resolution of 0.5 ms |
| Time Base | 16 | Refer to 11.13.26 Time base in milliseconds |
| E-MBS_Zone_ID | 8 | E-MBS Zone identifier, which indicates an E-MBS zone where the connection for associated service flow is valid. To support E-MBS, it shall be included in AAI_DSA-REQ/RSP message. |
| Paging Preference | 1 | Refer to 11.13.28 0 = No paging generation 1 = Paging generation |
| SN Feedback Enabled | 1 | Refer to 11.13.29 |
| HARQ Service Flows | 1 | Refer to 11.13.30 |
| Authorization Token | variable | Authorization token that is used for authorizing the QoS for one or service flows generated by AMS-initiated higher level service flow creation or modification procedures. Refer to 11.13.32 |
| HARQ Channel Mapping | 16variable | Refer to 11.13.33 This is bitmap. If the k-th bit is set to 1, it indicates that the k-th HARQ channel is acceptable to the sender. |
| PDU SN Extended Subheader for HARQ Reordering | 2 | Refer to 11.13.34 |
| ROHC Parameter Payload | variable | Refer to Table xxx 11.13.36 |
| Packet Error Rate | 8 | Refer to 11.13.37 MSB (bit 7): 0 - PER measured by the application, post-ARQ and post-HARQ process 1 - PER measured on the airlink, before the application of ARQ and HARQ Bit 6: 0 - Interpret bits 0-5 as an integer %, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = N/100 (= N%) 1-Interpret bits 0-5 as 10 times a negative exponent of 10, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = 10-N/10 LSB 6 bits (bits 0-5): PER value If bit 6 =0, [0 to 63%] PER If bit 6=1, [~5x10-7 to 1x10-0] PER |
| PSC assignment | 3 | Refer to 11.13.38 |
| Emergency Indication | 1 | Indicates the associated flow is used for emergency purposes. |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow 0 = AGMH (Advanced Generic MAC Header) 1 = SPMH (Short-Packet MAC header) |
| Initial Backoff Window Size | 4 | Window size expressed as a power of 2 |
| Maximum Backoff Window Size | 4 | Window size expressed as a power of 2 |
| Backoff Scaling Factor | 4 | "0010" for Binary Exponential Backoff |
| E-MBS Service Flow parameter | 16 | Mapping of E-MBS STID+FID To support E-MBS, it shall be included in AAI_DSA-REQ/RSP message. |
| E-MBS service | 2 | 0: No available E-MBS 1: E-MBS in Serving ABS Only 2: E-MBS in a multi-ABS Zone supporting macro-diversity 3: E-MBS in a multi-ABS Zone not supporting macro-diversity |
| Quick Access Predefined BR Index Parameter | variable | Refer to 16.2.12.8.4 |
| SCID | 4 | Sleep cycle ID |

[Remedy 2:]

[Add the following new Table xsx after Table 784:]

Table xxx - ROHC Parameter Payload

| Name | Size (bit) | Value |
| --- | --- | --- |
| ROHC Max Context ID | 16 | Non-negative integer |
| Large Context IDs | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2-255: Reserved |
| ROHC MRRU | 16 | 0: no segmentation Otherwise: MRRU |

[Remedy 3:]

[Add the followings on line 37 of page 336 after subclause 16.2.12.8.4 Predefined BR Index Parameter:]

16.2.12.8.5 HARQ channel mapping

This parameter specifies the set of HARQ channels for carrying data on this service flow acceptable to the sender. A HARQ channel may be shared by more than one service flow. The absence of this parameter in AAI_DSA-REQ message relevant for the service flow means that the sender of the message accepts that all HARQ channels from ‘0’ up to the number of HARQ channels supported may be used by this service flow. HARQ channel mapping is enabled independently in the UL and DL directions.

[Remedy 4:] AAI_DSA-REQ message

[Change Table 740 on line 53 of page 180 in section 16.2.3.45.1 as follows:]

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note |  | Conditions |  |
| --- | --- | --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | AAI_DSA-REQ |  | N.A. |  |
| M | FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. |  | N.A. |  |
| O | SFID | 32 |  |  | Present when an ABS initiates AAI_DSA-REQ |  |
| O | FID | 4 |  |  | Present when an ABS initiates AAI_DSA-REQ |  |
| M | Uplink/Downlink Indicator | 1 | 0: uplink; 1:downlink |  | N.A. |  |
| O | Service Class Name uplink | 16 to 1024 2 to 128 | Null-terminated string of ASCII characters. The length of the string, including null-terminator may not exceed 128 bytes |  | Present when a predefined BS service configuration is used for this service flow. |  |
| O | Global Service Class Name field | variable (see 6.3.14.4.1) | Variable: Refer to Table 186 of 6.3.14.4.1 |  | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |  |
| O | A) I: Unlink/Downlink indicator | 1 | 0: UL 1: DL |  |  |  |
| O | B) S: Maximum sustained traffic rule | 6 | Extensible look-up table 187 of 6.3.14.4.1 |  |  |  |
| O | C) B: Maximum traffic burst | 6 | Extensible look-up table 187 of 6.3.14.4.1 |  |  |  |
| O | D) R: Maximum received traffic rate | 6 | Extensible look-up table 187 of 6.3.14.4.1 |  |  |  |
| O | E) L: Maximum latency | 6 | Extensible look-up table 188 of 6.3.14.4.1 |  |  |  |
| O | F) P: Paging preference | 1 | 0 = No paging generation 1 = Paging generation |  |  |  |
| O | G) S1: Request/Transmission policy | 8 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to'0' and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to '1', none of the |  |  |  |
| O | H) S2: Uplink grant scheduling type | 3 | 0: Reserved 1: Undefined (BS implementation-dependent) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service |  |  |  |
| O | I) L1: Tolerated Jitter | 6 | Extensible look-up table 188 of 6.3.14.4.1. This is available only for Uplink grant scheduling type = ertPS or UGS. |  |  |  |
| O | J) S3: Traffic priority | 3 | This is used only for Uplink Grant Scheduling Type = rtPS, ertPS, nrtPS or BE. |  |  |  |
| O | K) S4: Unsolicited grant interval | 6 | Extensible look-up table 189 of 6.3.14.4.1. This is available only for Uplink grant scheduling type = ertPS or UGS. |  |  |  |
| O | L) S5: Unsolicited polling interval | 6 | Extensible look-up table 189 of 6.3.14.4.1. This is available only for Uplink grant scheduling type = rtPS. |  |  |  |
| M | QoS parameter set type | 38 | Bit 0: Provisioned Set Bit 1: Admitted Set Bit 2: Active Set |  |  |  |
| O | QoS parameters | variable |  |  |  |  |
| O | A) Traffic Priority parameter | 3 | 0 to 7-Higher numbers indicate higher priority Default 0 |  |  |  |
| O | B) Maximum Sustained Traffic Rate parameter | 32 | Rate (in bits per second) |  |  |  |
| O | C) Maximum Traffic Burst parameter | 32 | Burst size (bytes) |  |  |  |
| O | D) Minimum Reserved Traffic Rate parameter | 32 | Rate (in bits per second) |  |  |  |
| O | E) Maximum Latency parameter | 32 | Milliseconds |  |  |  |
| O | F) Paging Preference parameter | 18 | 0: No paging generation 1: Paging generation |  |  |  |
| O | G) Request/Transmission Policy parameter | 8 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to'0' and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to '1', none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 6 is set to'0' and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 6 is set to '1', none of the SDUs shall be compressed. |  |  |  |
| O | H) UL Grant Scheduling Type parameter | 38 | 0: Reserved 1: Undefined (BS implementation-dependenta) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service |  | Present for UL |  |
| O | I) Tolerated Jitter parameter | 32 | Milliseconds |  | Present when needed |  |
| O | J) Unsolicited Grant Interval parameter | 16 | Milliseconds |  | Present when UL scheduling type is UGS or ertPS |  |
| O | K) Unsolicited Polling Interval parameter | 16 | Milliseconds |  | Present when UL scheduling type is rtPS |  |
| O | L) Primary Grant Polling Interval parameter | 16 | Milliseconds |  | Present when UL scheduling type is aGP Service |  |
| O | M) Primary Grant Size parameter | 16 | Number of bytes. |  | Present when UL scheduling type is aGP Service |  |
| O | N) Secondary Grant Polling Interval parameter | 16 | Milliseconds |  | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |  |
| O | O) Secondary Grant Size parameter | 16 | Number of bytes. |  | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |  |
| O | P) Adaptation Method | 2 | 00:Implicit adaptation 01:Explicit adaptation, sustained 10: Explicit adaptation, one time only |  | Present when UL scheduling type is aGP Service |  |
| O | Q) Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1. |  | Present for UL |  |
| O | R) Differentiated BR Timer | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s) |  | Present for UL |  |
| O | S) Quick Access Predefined BR Index Parameter | Variable | The mapping of predefined BR index used in quick access message to BR size and BR actions |  | Present for UL |  |
| O | T) MAC in-order delivery indicator | 1 | Indicate whether or not the order of delivery in non-ARQ connection is preserved by the MAC. 0 : not preserved 1 : preserved |  |  |  |
| O | U) Vendor Specific QoS Parameter | variable |  |  |  |  |
| O | Type of Data Delivery Services parameter | 8 | 0: Unsolicited Grant Service 1: Real-Time Variable Rate Service 2: Non-Real-Time Variable Rate Service 3: Best Effort Service 4: Extended Real-Time Variable Rate Service |  |  |  |
| O | SDU Inter-Arrival Interval parameter | 16 | SDU inter-arrival interval in the resolution of 0.5 ms |  |  |  |
| O | Time Base parameter | 16 | Time base in milliseconds |  |  |  |
| O | SDU Size parameter | 8 | Number of bytes. Default = 49 |  |  |  |
| O | Target SAID parameter | 816 | SAID onto which service flow is mapped |  |  |  |
| O | SCID | 4 | Sleep cycle ID |  |  |  |
| O | ARQ parameters | variable |  |  |  |  |
| O | A) ARQ Enable | 18 | 0 = ARQ Not Requested/Accepted 1 = ARQ Requested/Accepted |  |  |  |
| O | B) ARQ_WINDOW_SIZE | 16 | >0 and <=(ARQ_BSN_MODULUS/2) |  |  |  |
| O | C) ARQ_BLOCK_LIFETIME | 16 | 0 = Infinite 1-6553500 (100 granularity) |  |  |  |
| O | D) ARQ_SYNC_LOSS_TIMEOUT | 16 | 0 = Infinite 1-6553500 (100 granularity) |  |  |  |
| O | E) ARQ_RX_PURGE_TIMEOUT | 16 | 0 = Infinite 1-6553500 (100 granularity) |  |  |  |
| O | F) ARQ_SUB_BLOCK_SIZE | 38 | Bit 0–2: encoding for selected block size (P), where the selected block size is equal to 2^(P+3), 0<=P<=7. ARQ sub-block size is byte unit. |  |  |  |
| O | G) ARQ_ERROR_DETECTION_TIMEOUT | 16 |  |  |  |  |
| O | H) ARQ_FEEDBACK_POLL_RETRY_TIMEOUT | 16 |  |  |  |  |
| O | RECEIVER_ARQ_ACK_PROCESSING_TIME | 1 | 0-255 |  |  |  |
| O | CS Specification parameter | 8 | 0: GPCS (Generic Packet Convergence Sublayer) reserved 1: Packet, IPv4 2: Packet, IPv6 3: Packet, IEEE 802.3/Etherneta reserved 4: Reserved 5: Packet, IPv4 over IEEE 802.3/Etherneta reserved 6: Packet, IPv6 over IEEE 802.3/Etherneta reserved 7: Reserved 8: Reserved 9: ATM reserved 10: Reserved 11: Reserved 12: Reserved 13: Reserved 14: Packet, IPb 15: Multiprotocol flow 16-255 Reserved (b: SDUs for service flows of this CS type may carry either IPv4 or IPv6 in the header-compressed payload) |  |  |  |
| O | CS encodingRuler | 4variable | 99 ATM reserved 100 Packet, IPv4 101 Packet, IPv6 102 Packet, IEEE 802.3/Ethernet reserved 103 reserved 104 Packet IPV4 over IEEE 802.3/Ethernet reserved 105 Packet IPV6 over IEEE 802.3/Ethernet reserved 106 Reserved 107 Reserved 108 Reserved 109 Reserved 110 Reserved 111 Reserved 112 GPCS (Generic Packet Convergence Sublayer) reserved 113 Packet IP 114 Packet, Multiprotocol flow |  |  |  |
| O | Packet Classification Rule parameter | variable |  |  |  |  |
| O | A) Classification Rule Priority field | 8 | 0-255 |  |  |  |
| O | B) Protocol field | 8 | protocol |  |  |  |
| O | C) IP Masked Source Address parameter | 8 (IPv4) or 32 (IPv6) | src, smask |  |  |  |
| O | D) IP Masked Destination Address parameter | 8 (IPv4) or 32 (IPv6) | dst, dmask |  |  |  |
| O | E) Protocol Source Port Range field | 32 | sportlow, sporthigh |  |  |  |
| O | F) Protocol Destination Port Range field | 32 | dportlow, dporthigh |  |  |  |
| O | IEEE 802.3/Ethernet Destination MAC Address parameter | 96 | dst, msk |  |  |  |
| O | IEEE 802.3/Ethernet Source MAC Address parameter | 96 | src, msk |  |  |  |
| O | Ethertype/IEEE 802.2 SAP | 24 | type, eprot1, eprot2 |  |  |  |
| O | IEEE 802.1D User Priority field | 16 | pri-low, pri-high Valid range: 0-7 for pri-low and pri-high |  |  |  |
| O | IEEE 802.1Q VLAN ID field | 16 | vlan_id1, vlan_id2 |  |  |  |
| O | G) Associated PHSI field | 8 | Index value |  |  |  |
| O | H) Packet Classification Rule Index field | 16 | Packet Classification Rule Index |  |  |  |
| O | I) Vendor-specific classification parameters | variable | Compound |  |  |  |
| O | J) IPv6 Flow Label field | 24 | Flow Label |  |  |  |
| O | K) Classification Action Rule | 8 | Bit 0: 0 = none. 1 = Discard packet Bit 1-7: Reserved. |  |  |  |
| O | L) IP Type of Service | 8 | Bit 0-Bit 1: reserved. Shall be set to 0b00 Bit 2-Bit 7: DSCP value |  |  |  |
| O | M) PHS DSC Action field | 2 | 0: Add PHS Rule 1: Set PHS Rule 2: Delete PHS Rule 3: Delete all PHS Rules |  |  |  |
| O | N) PHS Rule field | variable |  |  |  |  |
| O | N.1) PHSI field | 8 |  | Index value |  |  |
| O | N.2) PHSF field | n |  | String of bytes suppressed |  |  |
| O | N.3) PHSM field | n |  | Bit 0: 0 = Don't suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don't suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don't suppress (x + 1) byte of the suppression field 1 = Suppress (x + 1) byte of the suppression field |  |  |
| O | N.4) PHSS field | 8 |  | Number of bytes in the suppression string |  |  |
| O | N.5) PHSV field | 1 |  | 0: Verify 1: Don't verify |  |  |
| O | N.6) Vendor-specific PHS parameters | variable |  | Compound |  |  |
| O | MBS service TLV | 8 | 0: No available MBS 1: MBS in Serving BS Only 2: MBS in a multi-BS Zone supporting macro-diversity 3: MBS in a multi-BS Zone not supporting macro-diversity 4-255: Reserved |  |  |  |
| O | MBS Zone Identifier Assignment parameter | 8 | MBS zone identifier (bits 6 through 0 are the MBS Zone Identifier, bit 7 is set to 0) |  |  |  |
| O | HARQ Channel Mapping | 16variable | This is bitmap. If the k-th bit is set to 1, it indicates that the k-th HARQ channel is acceptable to the sender. |  |  |  |
| O | ROHC Parameter | variable |  |  |  |  |
| O | A) ROHC Max Context ID | 16 | Non-negative integer |  |  |  |
| O | B) Large Context IDs | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2-255: Reserved |  |  |  |
| O | C) ROHC MRRU | 16 | 0: no segmentation Otherwise: MRRU |  |  |  |
| O | Packet Error Rate (PER) | 8 | MSB (bit 7): 0 - PER measured by the application, post-ARQ and post-HARQ process 1 - PER measured on the airlink, before the application of ARQ and HARQ Bit 6: 0 - Interpret bits 0-5 as an integer %, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = N/100 (= N%) 1-Interpret bits 0-5 as 10 times a negative exponent of 10, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = 10-N/10 LSB 6 bits (bits 0-5): PER value If bit 6 =0, [0 to 63%] PER If bit 6=1, [~5x10-7 to 1x10-0] PER |  |  |  |
| O | PSC assignment | 8 | Bit 0-5: The Power_Saving_Class_ID of the PSC to which the CID of this service flow is to be added. Bit 6-7: Reserved |  |  |  |
| O | Regional Emergency Indication parameter | 18 | Bit 0: Emergency indication Bits 1-7: Reserved Indicates the associated flow is used for emergency purposes. |  |  |  |
| O | MAC Header Type | 1 | Indicates whether AGMH or SPMHCMH is presented at the start of MPDUs of the service flow 0 = AGMH (Advanced Generic MAC Header) 1 = SPMHCMH (Short-Packet Compact MAC header) |  |  |  |
| O | E-MBS Service | 212 | Indicates whether the MBS service is being requested or provided for the connection that is being setup. 0: No available E-MBS 1: E-MBS in Serving ABS Only 2: E-MBS in a multi-ABS Zone supporting macro-diversity 3: E-MBS in a multi-ABS Zone not supporting macro-diversity |  | included in both ABS/AMS initiated AAI_DSA-REQ |  |
| O | E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid. |  |  |  |
| O | E-MBS Service Flow Parameter: | 16 variable | Mapping of E-MBS ID and FID |  |  |  |
| O | Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to, only included in ABS initiated AAI_DSA-REQ |  | only included in ABS initiated AAI_DSA-REQ |  |
| O | Initial Backoff Window Size | 4 | Window size expressed as a power of 2 |  |  |  |
| O | Maximum Backoff Window Size | 4 | Window size expressed as a power of 2 |  |  |  |
| O | Backoff Scaling Factor | 4 | "0010" for Binary Exponential Backoff |  |  |  |

[Remedy 5:] AAI_DSC-REQ message

[Change Table 743 on line 39 of page 190 in section 16.2.3.45.4 as follows:]

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | AAI_DSC-REQ | N.A. |
| M | FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | N.A. |
| O | SFID | 32 |  | Present when an ABS initiates AAI_DSA-REQ |
| O | FID | 4 |  | Present when an ABS initiates AAI_DSCA-REQ |
| M | Uplink/Downlink Indicator | 1 | 0: uplink; 1:downlink | N.A. |
| O | Service Class Nameuplink | 16~1024 2 to 128 | Null-terminated string of ASCII characters. The length of the string, including null-terminator may not exceed 128 bytes | Present when a predefined BS service configuration is used for this service flow. |
| O | Global Service Class Name field | variable (see 6.3.14.4.1) | Variable: Refer to Table 186 of 6.3.14.4.1 | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |
| M | QoS parameter set type | 38 | Bit 0: Provisioned Set Bit 1: Admitted Set Bit 2: Active Set |  |
| O | QoS parameters | variable |  |  |
| O | A) Traffic Priority parameter | 3 | 0 to 7-Higher numbers indicate higher priority Default 0 | Present when needed |
| O | B) Maximum Sustained Traffic Rate parameter | 32 | Rate (in bits per second) | Present when needed |
| O | C) Maximum Traffic Burst parameter | 32 | Burst size (bytes) | Present when needed |
| O | D) Minimum Reserved Traffic Rate parameter | 32 | Rate (in bits per second) | Present when needed |
| O | E) Maximum Latency parameter | 32 | Milliseconds | Present when needed |
| O | F) Paging Preference parameter | 18 | 0: No paging generation 1: Paging generation |  |
| O | Request/Transmission Policy parameter | 8 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to'0' and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to '1', none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 6 is set to'0' and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 6 is set to '1', none of the SDUs shall be compressed. |  |
| O | UL Grant Scheduling Type parameter | 8 | 0: Reserved 1: Undefined (BS implementation-dependenta) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service | Present for UL |
| O | G) Tolerated Jitter parameter | 32 | Milliseconds | Present when needed |
| O | H) Unsolicited Grant Interval parameter | 16 | Milliseconds | Present when UL scheduling type is UGS or ertPS |
| O | I) Unsolicited Polling Interval parameter | 16 | Milliseconds | Present when UL scheduling type is rtPS |
| O | J) Primary Grant Polling Interval parameter | 16 | Milliseconds | Present when UL scheduling type is aGP Service |
| O | K) Primary Grant Size parameter | 16 | Number of bytes. | Present when UL scheduling type is aGP Service |
| O | L) Secondary Grant Polling Interval parameter | 16 | Milliseconds | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O | M) Secondary Grant Size parameter | 16 | Number of bytes. | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O | N) Adaptation Method | 2 | 00:Implicit adaptation 01:Explicit adaptation, sustained 10: Explicit adaptation, one time only | Present when UL scheduling type is aGP Service |
| O | O) Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1. | Present for UL |
| O | P) Differentiated BR Timer | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s) | Present for UL |
| O | Q) Quick Access Predefined BR Index Parameter | Variable | The mapping of predefined BR index used in quick access message to BR size and BR actions | Present for UL |
| O | R) MAC in-order delivery indicator | 1 | Indicate whether or not the order of delivery in non-ARQ connection is preserved by the MAC. 0 : not preserved 1 : preserved |  |
| O | S) Vendor Specific QoS Parameter | variable |  |  |
| O | Type of Data Delivery Services parameter | 8 | 0: Unsolicited Grant Service 1: Real-Time Variable Rate Service 2: Non-Real-Time Variable Rate Service 3: Best Effort Service 4: Extended Real-Time Variable Rate Service |  |
| O | SDU Inter-Arrival Interval parameter | 16 | SDU inter-arrival interval in the resolution of 0.5 ms | Present when needed |
| O | Time Base parameter | 16 | Time base in milliseconds | Present when needed |
| O | Target SAID parameter | 16 | SAID onto which service flow is mapped |  |
| O | SCID | 4 | Sleep cycle ID |  |
|  | CS Specification parameter | 8 | 0: GPCS (Generic Packet Convergence Sublayer) 1: Packet, IPv4 2: Packet, IPv6 3: Packet, IEEE 802.3/Etherneta 4: Reserved 5: Packet, IPv4 over IEEE 802.3/Etherneta 6: Packet, IPv6 over IEEE 802.3/Etherneta 7: Reserved 8: Reserved 9: ATM 10: Reserved 11: Reserved 12: Reserved 13: Reserved 14: Packet, IPb 15: Multiprotocol flow 16-255 Reserved |  |
|  | CS encodingRuler | 4variable | 99 ATM reserved 100 Packet, IPv4 101 Packet, IPv6 102 Packet, IEEE 802.3/Ethernet reserved 103 reserved 104 Packet IPV4 over IEEE 802.3/Ethernet reserved 105 Packet IPV6 over IEEE 802.3/Ethernet reserved 106 Reserved 107 Reserved 108 Reserved 109 Reserved 110 Reserved 111 Reserved 112 GPCS (Generic Packet Convergence Sublayer) reserved 113 Packet IP 114 Packet, Multiprotocol flow |  |
| O | Classifier DSC Action | 8 | 0: DSC Add classifier 1: DSC Release classifier 2: DSC Delete classifier | Present for AAI_DSC-REQ |
| O | Packet Classification Rule parameter | variable |  |  |
| O | A) Classification Rule Priority field | 8 | 0-255 |  |
| O | B) Protocol field | 8 | protocol |  |
| O | C) IP Masked Source Address parameter | 8 (IPv4) or 32 (IPv6) | src, smask |  |
| O | D) IP Masked Destination Address parameter | 8 (IPv4) or 32 (IPv6) | dst, dmask |  |
| O | E) Protocol Source Port Range field | 32 | sportlow, sporthigh |  |
| O | F) Protocol Destination Port Range field | 32 | dportlow, dporthigh |  |
| O | IEEE 802.3/Ethernet Destination MAC Address parameter | 96 | dst, msk |  |
| O | IEEE 802.3/Ethernet Source MAC Address parameter | 96 | src, msk |  |
| O | Ethertype/IEEE 802.2 SAP | 24 | type, eprot1, eprot2 |  |
| O | IEEE 802.1D User Priority field | 16 | pri-low, pri-high Valid range: 0-7 for pri-low and pri-high |  |
| O | IEEE 802.1Q VLAN ID field | 16 | vlan_id1, vlan_id2 |  |
| O | G) Associated PHSI field | 8 | Index value |  |
| O | H) Packet Classification Rule Index field | 16 | Packet Classification Rule Index |  |
| O | I) Vendor-specific classification parameters | variable | Compound |  |
| O | J) IPv6 Flow Label field | 24 | Flow Label |  |
| O | K) Classification Action Rule | 8 | Bit 0: 0 = none. 1 = Discard packet Bit 1-7: Reserved. |  |
| O | L) IP Type of Service | 8 | Bit 0-Bit 1: reserved. Shall be set to 0b00 Bit 2-Bit 7: DSCP value |  |
| O | M) PHS DSC Action field | 2 | 0: Add PHS Rule 1: Set PHS Rule 2: Delete PHS Rule 3: Delete all PHS Rules |  |
| O | N) PHS Rule field | variable |  |  |
| O | N.1) PHSI field | 8 | Index value |  |
| O | N.2) PHSF field | n | String of bytes suppressed |  |
| O | N.3) PHSM field | n | Bit 0: 0 = Don't suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don't suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don't suppress (x + 1) byte of the suppression field 1 = Suppress (x + 1) byte of the suppression field |  |
| O | N.4) PHSS field | 8 | Number of bytes in the suppression string |  |
| O | N.5) PHSV field | 1 | 0: Verify 1: Don't verify |  |
| O | N.6) Vendor-specific PHS parameters | variable | Compound |  |
| O | MBS service TLV | 8 | 0: No available MBS 1: MBS in Serving BS Only 2: MBS in a multi-BS Zone supporting macro-diversity 3: MBS in a multi-BS Zone not supporting macro-diversity 4-255: Reserved |  |
| O | MBS Zone Identifier Assignment parameter | 8 | MBS zone identifier (bits 6 through 0 are the MBS Zone Identifier, bit 7 is set to 0) |  |
| O | HARQ Channel Mapping | variable |  |  |
| O | ROHC Parameter | variable |  |  |
| O | A) ROHC Max Context ID | 16 | Non-negative integer |  |
| O | B) Large Context IDs | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2-255: Reserved |  |
| O | C) ROHC MRRU | 16 | 0: no segmentation Otherwise: MRRU |  |
| O | Packet Error Rate (PER) | 8 | MSB (bit 7): 0 - PER measured by the application, post-ARQ and post-HARQ process 1 - PER measured on the airlink, before the application of ARQ and HARQ Bit 6: 0 - Interpret bits 0-5 as an integer %, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = N/100 (= N%) 1-Interpret bits 0-5 as 10 times a negative exponent of 10, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = 10-N/10 LSB 6 bits (bits 0-5): PER value If bit 6 =0, [0 to 63%] PER If bit 6=1, [~5x10-7 to 1x10-0] PER |  |
| O | PSC assignment | 8 | Bit 0-5: The Power_Saving_Class_ID of the PSC to which the CID of this service flow is to be added. Bit 6-7: Reserved |  |
| O | Regional Emergency Indication parameter | 18 | Bit 0: Emergency indication Bits 1-7: Reserved Indicates the associated flow is used for emergency purposes. |  |
| O | MAC Header Type | 1 | Indicates whether AGMH or CMH is presented at the start of MPDUs of the service flow 0 = AGMH (Advanced Generic MAC Header) 1 = CMH (Compact MAC header) |  |
| O | E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid. | only included in ABS initiated AAI_DSC-REQ |
| O | E-MBS Service Flow Parameter | 16 variable | Mapping of E-MBS ID and FID |  |
| O | Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to, only included in ABS initiated AAI_DSA-REQ |  |

[Remedy 6:] AAI_DSA-RSP message

[Change Table 741 on line 59 of page 188 in section 16.2.3.45.2 as follows:]

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | N.A. |
| M | FID Change Count | 4 | FID Change Count from corresponding the AAI_DSA-REQ message | N.A. |
| M | Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure |  |
| O | Flow ID | 4 | An identifier of a service flow | Present when an AMS initiates AAI_DSA-REQ and Confirmation code is set to 0 |
| O | E-MBS Service | 2 | Indicates whether the MBS service is being requested or provided for the connection that is being setup. 0: No available E-MBS 1: E-MBS in Serving ABS Only 2: E-MBS in a multi-ABS Zone supporting macro-diversity 3: E-MBS in a multi-ABS Zone not supporting macro-diversity | included in both ABS/AMS initiated DSA-REQ |
| O | E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid. | only included in ABS initiated DSA-REQ |
| O | E-MBS Service Flow Parameter: | variable | Mapping of E-MBS ID and FID |  |
| O | Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to |  |

-------------------------------  Text End  ---------------------------------------------------