| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for coupled group create/change parameter (16.2.26) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Hyunjeong Kang, Jungshin Park Samsung Electronics Co., Ltd. | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16-10/0032: “IEEE 802.16 Working Group Letter Ballot #31a: Announcement” |  |
| Abstract | The contribution proposes a clean-up for coupled group parameter create/change in section 16.2.26 |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for coupled group create/change parameter

Hyunjeong Kang, Jungshin Park

Samsung Electronics Co., Ltd.

- Introduction

Subclause 16.2.26 Coupled Group parameter Create/Change TLV should be a part of Table 784 Service Flow/Convergence Sublayer Parameters and the parameter can be transmitted in an AAI_DSA-REQ or AAI_DSC-REQ to create/change a DL service flow and an UL service flow in one DSA/DSC transaction. In addition, Group Parameter Create/Change parameter is missing in the Service Flow/Convergence Sublayer Parameters, which provides a method to allow an AMS or an ABS to create or change a number of service flows in a single DSA or DSC procedure. The Group Parameter Create/Change parameter can be a part of AAI_DSA-REQ or AAI_DSC-REQ message.

This contribution proposes to clean up coupled group create/change parameter and group parameter create/change parameter in AAI_DSA-REQ and AAI_DSC-REQ messages.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

Remove the whole section 16.2.26

[Remedy 2:]

[Add the following parameters into table 784 on line 54 of page 332 in section 16.2.12.8 Service Flow/Convergence Sublayer Parameters:]

-------------------------------------------------------------------------------------------------------------------

Group Parameter Create/Change | variable   | Only one instance of the Group Parameter

|          | Create/Change may be included in AAI_DSA or

|          | AAI_DSC message.

-------------------------------------------------------------------------------------------------------------------

Common parameters          | variable   | Common service flow encodings that are

|          | common to all service flows specified in Group

|          | Parameter Create/Change. All the rules and

|          | settings that apply to the service flow encodings

|          | when used in an AAI_DSA or AAI_DSC

|          | message apply to the contents encapsulated in

|          | this parameter.

|          | Shall be included only once in a Group

|          | Parameter Create/Change as the first attribute.

--------------------------------------------------------------------------------------------------------------------

Qty SFID request             |    M    | Qty SFID request is the quantity of service

|          | flows of the same common parameter set

|          | configuration. Shall only be sent by the AMS as

|          | the last attribute in AAI_DSA-REQ or

|          | AAI_DSC-REQ

--------------------------------------------------------------------------------------------------------------------

FID List                    |   N  4   | List of N FIDs

--------------------------------------------------------------------------------------------------------------------

FID Parameter List           |  variable  | FID parameter list that encapsulates an FID

|           | of an service flow and associated non-common

|           | service flow encodings for that service flow,

|           | specified in this Group Parameter

|           | Create/Change. All the rules and settings that

|           | apply to the service flow encodings when used

|           | in an AAI_DSA or AAI_DSC message apply to

|           | the contents encapsulated in this parameter.

|           | If included, shall be the last attribute of Group

|           | Parameter Create/Change.

--------------------------------------------------------------------------------------------------------------------

FID                       |     4     | FID of an service flow in the SFID Parameter

|           | List

--------------------------------------------------------------------------------------------------------------------

Non-Common parameters      | variable   | Encapsulates the non-common service flow

|          | encodings that are specific to individual service

|          | flows specified in the FID Parameter List.

|          | All the rules and settings that apply to the

|          | service flow encodings when used in

|          | an AAI_DSA or AAI_DSC message apply to the

|          | contents encapsulated in this parameter.

--------------------------------------------------------------------------------------------------------------------

Coupled Group Create/Change   | variable   | this parameter encapsulates service flow

|          | encodings for Coupled UL and DL unicast

|          | service flows in an AAI_DSA or AAI_DSC

|          | message.

--------------------------------------------------------------------------------------------------------------------

Common parameters           | variable   | the common service flow encodings that are

|          | common to the Coupled UL and DL unicast service

|          | flows of an AMS in this Coupled Parameter

|          | Create/Change parameter.

|          | If included, this parameter shall be the first

|          | attribute.

--------------------------------------------------------------------------------------------------------------------

Qty Coupled SFID request      |    M    | Qty Coupled SFID request is the quantity of

|          | Coupled UL and DL unicast service flows of an

|          | AMS, of the same common parameter set

|          | configuration. Used only as the last attribute of

|          | a Coupled Parameter Create/Change

|          | in an AAI_DSA-REQ or an AAI_DSC-REQ

--------------------------------------------------------------------------------------------------------------------

Coupled FID List            |   N 8    | N  (4MSB for DL FID and 4LSB for UL FID)

--------------------------------------------------------------------------------------------------------------------

FID parameter list           |  variable   | FID list that encapsulates an FID of an service

|           | flow and associated non-common service flow

|           | encodings for that service flow, specified in

|           | this Coupled Parameter Create/Change.

|           | All the rules and settings that apply to the

|           | service flow encodings when used in an

|           | AAI_DSA or AAI_DSC message apply to

|           | the contents encapsulated in this parameter.

|           | If included, shall be the last attribute of Coupled

|           | Parameter Create/Change.

--------------------------------------------------------------------------------------------------------------------

FID                       |     4     | FID of an service flow in the FID Parameter

|           | List

--------------------------------------------------------------------------------------------------------------------

Non-Common parameters      | variable   | Encapsulates the non-common service flow

|          | encodings that are specific to individual service

|          | flows specified in the FID Parameter List.

|          | All the rules and settings that apply to the

|          | service flow encodings when used in

|          | an AAI_DSA or AAI_DSC message apply to the

|          | contents encapsulated in this parameter.

--------------------------------------------------------------------------------------------------------------------

[Remedy 3:]

[Add the followings on line 37 of page 336 after subclause 16.2.12.8.4 Predefined BR Index Parameter:]

16.2.12.8.5 Coupled Group Create/Change

This parameter provides a method to allow an AMS or an ABS to create or change a DL service flow and a UL service flow together in a single DSA or DSC procedure. If any requested SFID fails to create or change, then an AAI_DSA-RSP or AAI_DSC-RSP shall be sent with Confirmation code set to Non-zero.

Coupled Group Create/Change parameter shall not be included together with Group Parameter Create/Change in an AAI_DSA or AAI_DSC message.

16.2.12.8.5.1 Qty Coupled SFID request

This parameter is used to inform the number of service flows to create or change with Coupled Group Create/Change parameter in the AAI_DSA or AAI_DSC procedure.

16.2.12.8.6 Group Parameter Create/Change

This parameter provides a method to allow an AMS or an ABS to create or change a number of service flows in a single DSA or DSC procedure. If at least one service flow is successfully created or changed, then an AAI_DSA-RSP or AAI_DSC-RSP shall be sent with Confirmation code set to 0. The FID of a service flow that fails to create or change shall not be transmitted in the AAI_DSA-RSP or AAI_DSC-RSP message.

16.2.12.8.6.1 Qty SFID request

This parameter is used to inform the number of service flows to create or change with Group Parameter Create/Change parameter in the AAI_DSA or AAI_DSC procedure.

[Remedy 4:]

[Add the following parameters into Table 740 on line 53 of page 180 in subclause 16.2.3.45.1 AAI_DSA-REQ:]

| O | Group Parameter Create/Change | variable |  |  |
| --- | --- | --- | --- | --- |
| O | A) Common parameters | variable | Common service flow encodings that are common to all service flows specified in Group Parameter Create/Change. |  |
| O | B) Qty SFID request | M | the number of service flows to create | Shall only be sent by an AMS |
| M | C) FID List | 4  N | FIDs of service flows created with this Group Parameter Create/Change |  |
| O | D) FID Parameter List | variable |  |  |
| O | D.1) FID | 4 |  |  |
| O | D.2) Non-common parameters | variable | Non-common service flow encodings that are specific to individual service flows specified in FID Parameter List |  |
| O | Coupled Group Create/Change | Variable |  |  |
| O | A) Common parameters | Variable | Common service flow encodings that are common to all service flows specified in Coupled Group Create/Change parameter. |  |
| O | B) Qty Coupled SFID request | M | the number of DL service flow and UL service flow to create |  |
| M | C) Coupled FID List | N  8 | FIDs of DL and UL service flow (4MSB for DL FID and 4LSB for UL FID) |  |
| O | D) FID parameter list | Variable |  |  |
| O | D.1) FID | 4 |  |  |
| O | D.2) Non-common parameters | Variable | Non-common service flow encodings that are specific to individual service flows specified in FID Parameter List |  |

[Remedy 5:]

[Add the following parameters into Table 743 on line 40 of page 190 in subclause 16.2.3.45.4 AAI_DSC-REQ:]

| O | Group Parameter Create/Change | variable |  |  |
| --- | --- | --- | --- | --- |
| O | A) Common parameters | variable | Common service flow encodings that are common to all service flows specified in Group Parameter Create/Change. |  |
| O | B) Qty SFID request | M | the number of service flows to change | Shall only be sent by an AMS |
| M | C) FID List | 4  N | FIDs of service flows changed with this Group Parameter Create/Change |  |
| O | D) FID Parameter List | variable |  |  |
| O | D.1) FID | 4 |  |  |
| O | D.2) Non-common parameters | variable | Non-common service flow encodings that are specific to individual service flows specified in FID Parameter List |  |
| O | Coupled Group Create/Change | Variable |  |  |
| O | A) Common parameters | Variable | Common service flow encodings that are common to all service flows specified in Coupled Group Create/Change parameter. |  |
| O | B) Qty Coupled SFID request | M | the number of DL service flow and UL service flow to change |  |
| M | C) Coupled FID List | N  8 | FIDs of DL and UL service flow 4MSB for DL FID and 4LSB for UL FID |  |
| O | D) FID parameter list | Variable |  |  |
| O | D.1) FID | 4 |  |  |
| O | D.2) Non-common parameters | Variable | Non-common service flow encodings that are specific to individual service flows specified in FID Parameter List |  |

-------------------------------  Text End  ---------------------------------------------------