| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed change on AES-CCM input parameters for extended header protection(16.2.5) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Youngkyo Baek, Youngbin Jang, Anil Agiwal Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “16.2.5” |  |
| Abstract | This contribution provides a solution to protect extended header in AES-CCM |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed change on AES-CCM input parameters for extended header protection(16.2.5)

Youngkyo Baek, Youngbin Jang, Anil Agiwal

Samsung Electronics

# Introduction

This contribution suggests changing AES-CCM input parameters for extended header protection.

According to current CCM blocks construction, extended headers are not protected even for integrity. However CCM mode supports protection for the additional associated data(e.g. extended headers), we suggest updated CCM block construction to support extended header protection.

Additionally the size of ICV is negotiated during key agreement procedure in Network entry. And initial CCM block B0 is defined depending on the size of ICV. But current text does not reflect changes.

Depending on the decision of whether 16m supports extended header protection, we should take either proposed text #1 or #2.

# Proposed Text #1

[For consistency with ICV size variation and to protect Extended headers]

Modify the sentences from line 5, page 241 as follows.

----------------------------------------------------- Start of Proposed Text #1--------------------------------------------------

The flag of initial CCM block B0 is decided based on some information such as size of an extended header group length(i.e., a) and the size of ICV(i.e., t) and size of length field Q(i.e, q).

'Tlen' shall equal 64 and t shall equal 8, meaning, the number of bytes in the message authentication field shall be set to 8. Consistent with the CCM specification, the 3-bit binary encoding [(t–2)/2)] of bits 5, 4, and 3 of the Flags byte field in B0 shall be either 001(if t=4) or 011(if t=8).

The size q of the Length field Q shall be set to 2. Consistent with the CCM specification, the 3-bit binary encoding [q-1] of the q field in bits 2, 1, and 0 of the Flags byte field in B0 shall be 001.

Consistent with the CCM specification, the bit 6 of the Flag field in B0 shall be either 0(if there exists no EH in the MPDU; i.e., a=0) or 1(if there exist some EHs in the MPDU).

Bit 7 of the Flag field in B0 is reserved and set by 0.

For example, if t=8 and there is no EH then the Flag field is set to 0x19( see Table xxx)

The length a of the associated data string A shall be set to 0.

| a t | 4 | 8 |
| --- | --- | --- |
| a=0 | 00001001 (0x09) | 00011001 (0x19) |
| a>0 | 01001001 (0x49) | 01011001 (0x59) |

Table xxx— Flag field in Initial CCM Block B0

………………………

………………………

| Byte Number | 0 | 1 13 | 14 15 |
| --- | --- | --- | --- |
| Byte significance |  |  | MSB LSB |
| Number of bytes | 1 | 13 | 2 |
| Field | Flag | NONCE | LQ |
| Contents | 0x19 As specified in table xxx | As specified in figure 402 | Length of plaintext payload |

Figure 403—Construction of Initial CCM Block B0

Note the ordering of the LQ value is MSB first, consistent with the NIST CCM specification.

Consistent with the CCM specification, the CCM Blocks consist of B0 and EH blocks and payload blocks. If there is no extended header in the MPDU, then EH block shall be omitted. The last EH block and the last payload block are padded with zeros, if necessary, for 16 byte alignment. Those EH and payload block are sequentially named as CCM Blocks B0,B1,… Bn by 16-byte unit( see Figure yyy).

| Number of bytes | Multiple of 16 byte(EH blocks) |  |  |  | Multiple of 16 byte(payload blocks) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 1 | extended header group length |  | Length of plaintext payload |  |
| Contents | 0x00 | extended header group length | Extended Headers | Padding the block with zero | Plaintext payload | Padding the block with zero |

Figure yyy— Construction of sequential CCM Blocks B1|B2|….|Bn

----------------------------------------------------- End of Proposed Text #1---------------------------------------------------

# Proposed Text #2

[evenif protection of extended headers are not supported, we have to take the propsed text #2 for consistency with ICV size variation]

Modify the sentences from line 5, page 241 as follows.

----------------------------------------------------- Start of Proposed Text #2--------------------------------------------------

The flag of initial CCM block B0 is decided based on some information such as the size of ICV(i.e., t) and size of length field Q(i.e, q).

'Tlen' shall equal 64 and t shall equal 8, meaning, the number of bytes in the message authentication field shall be set to 8. Consistent with the CCM specification, the 3-bit binary encoding [(t–2)/2)] of bits 5, 4, and 3 of the Flags byte field in B0 shall be either 001(if t=4) or 011(if t=8).

The size q of the Length field Q shall be set to 2. Consistent with the CCM specification, the 3-bit binary encoding [q-1] of the q field in bits 2, 1, and 0 of the Flags byte field in B0 shall be 001.

The length a of the associated data string A shall be set to 0 and so bit 6 of the Flag field in B0 shall be 0.

Bit 7 of the Flag field in B0 is reserved and set by 0.

Consistent with the CCM specification, the Flag field in B0 shall be either 0x09(00001001; for t=4) or 0x19(00011001; for t=8). See the figure 403.

………………………

………………………

| Byte Number | 0 | 1 13 | 14 15 |
| --- | --- | --- | --- |
| Byte significance |  |  | MSB LSB |
| Number of bytes | 1 | 13 | 2 |
| Field | Flag | NONCE | LQ |
| Contents | 0x09 or 0x19 | As specified in figure 402 | Length of plaintext payload |

Figure 403—Construction of Initial CCM Block B0

Note the ordering of the LQ value is MSB first, consistent with the NIST CCM specification.

----------------------------------------------------- End of Proposed Text #2---------------------------------------------------