| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on CRID update during re-authentication (16.2.3.41.5) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Youngkyo Baek Jicheol Lee Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “16.2.15.6” |  |
| Abstract | This contribution proposes CRID update scheme during re-authentication to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on CRID update during re-authentication (16.2.3.41.5)

Youngkyo Baek, Jicheol Lee

Samsung Electronics

# 1. Introduction

According to 16m/D5[1], CRID is used for MS identification when the AMS performs Network reentry from the coverage loss/DCR mode. And this CRID contains information which network entity(assumed to be the anchor authenticator) manages and AMS’s identifier. During re-authentication, AMS’s anchor authenticator may be changed according to network’s decision. In that case, in order to guarantee uniqueness of the CRID, the CRID has to be refreshed.

We propose a mechnism to refresh the CRID during keyagreement 3way handshake procedure after EAP re-authentication.

# 2. Solution

EAP (re) authentication procedure is followed by a key agreement 3way handshake procedure. So when anchor authenticator is changed during the reauthentication procedure and CRID refreshness is requried, the new CRID can be transferred to the AMS during the a key agreement 3way handshake procedure(i.e. it can be transmitted through the PKMv3 keyagreement MSG#3 message)

However, the PKMv3 keyagreement MSG#3 message is protected by integrity sense, but not in confidentiality sense. That is, the AAI_PKM-RSP(PKMv3 keyagreement MSG#3) is protected by CMAC, not by encryption.

Hence, if the PKMv3 keyagreement MSG#3 message carries a CRID as a plaintext, then the CRID is exposed. So we need another solution to transmit the CRID confidentially.

We suggest using a disguise CRID which is defined by current CRID  new CRID.

When ABS should transfer a new CRID after re-authentication, ABS transmits a disguise CRID through the PKMv3 keyagreement MSG#3 message.

Then, the AMS obtains the new CRID by apply XOR the current CRID to the disguise CRID.

Since the current CRID is securely managed, the new CRID can be transmitted confidentially.

# 3. Text Proposal

Modify the sentences at page 175, line 39 as follows

======================== Start of Proposed Text =====================

16.2.3.41.5 PKMv3 Key_Agreement-MSG#3 message

The ABS transmits the PKMv3 Key_Agreement-MSG#3 message as a final step in the 3-way handshake key agreement after successful CMAC value verification of PKMv3 Key_Agreement-MSG#2 message received from the AMS.

On receiving PKMv3 Key_Agreement-MSG#2 message from the AMS, the ABS derives PMK and AK from MSK depending on NONCE_BS and NONCE_MS, which are shared by PKMv3 Key_Agreement-MSG#2 messages, and validate the CMAC value contained in PKMv3 Key_Agreement-MSG#2 message.

During reauthentication when the CRID is changed, ABS shall notice a new CRID to the AMS by using a disguise CRID in PKMv3 Key_Agreement-MSG#3 message.

Code: 5

Attributes are shown in Table 733.

| Attribute | contents |
| --- | --- |
| …. | ….. |
| Disguise CRID | 72bit identifier used for transferring a new CRID securely, where disguise CRID = current CRID  new CRID. AMS retrieves the new CRID by disguise CRID  current CRID. It shall be included when anchor authenticator is changed with re-authentication. |
| …. | … |

============================== End of Proposed Text ===============

# 4. References

- IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Mar. 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.