| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up for AAI_REG-REQ/RSP messages (16.2.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Youngkyo Baek Hyeonjeong Kang Jicheol Lee Jungshin Park Lei Zhou Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “16.2.3.4” |  |
| Abstract | This contribution provides Clean up AAI_REG-REQ/RSP messages |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up AAI_REG-REQ/RSP messages (16.2.3)

Youngkyo Baek, Hyeonjeong Kang, Jicheol Lee, Jungshin Park, Lei Zhou

Samsung Electronics

# Introduction

AAI_REG-REQ/RSP messages are converted to ASN.1 format. Table 688 and Table 698 capture parameter s to be negotiated by AAI_REG-REQ and AAI_REG-RSP, respectively.

But since current message formats are not good to read and some parameters are missing, we suggest clean-up those messages in the natural way as the proposed texts.

The folowings are list of modification.

- Message format is changed

- M/O types are modified according to its conditions.

- Some attributes/ values are added or modified(marked by yellow color background)

- Some are removed (dynamic ranging for HO and 3-step BR in AAI_REG-REQ/RSP, MS scanning capability in AAI_REG-REQ and maximum ARQ buffer size in AAI_REG-RSP).

- Other some minor trivial fix

# Proposed Text

Modify the sentences and table, line 46, page 92 as follows.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

16.2.3.7 AAI_REG-REQ

An AAI_REG-REQ message is transmitted by AMS to negotiate general AMS capabilities and do registration during network entry.

The AAI_REG-REQ message shall be encrypted and not contain CMAC Tuple if authentication has been completed.

The following parameters may be included in AMS capability negotiation parameters of AAI_REG-REQ.

•AMS initiated aGP Service Adaptation Capability:

0b0: no support

0b1: support

[note to editor : replace table 688 with the following table]

[note to editor : yellow color backgrounds indicate modified parts. Please neglect them during implementation]

Table 688 - AAI_REG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | AMS MAC address | 48 | This is used to derive security keys | N.A. |
| M | ARQ parameters | variable | ARQ related parameters | N.A |
| M | - ARQ_SN_MODULUS | 10 | the number of unique SN values | N.A |
| M | - ARQ_WINDOW_SIZE | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks | N.A |
| M | - ARQ_SUB_BLOCK_SIZE | TBD | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | N.A |
| M | - ARQ_BLOCK_LIFETIME | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | N.A |
| M | - ARQ_RX_PURGE_TIMEOUT | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START | N.A |
| M | - MAXIMUM_ ARQ_BUFFER_SIZE | TBD | The maximum ARQ buffer size Depend on AMS’s memory | N.A |
| M | Multicarrier capabilities | 2 | 0b00: No MC modes 0b01: Basic MC mode 0b10: Multicarrier Aggregation 0b11: Multicarrier Switching. | N.A. |
| M | Capabilities for Zone switching mode | 1 |  | N.A. |
| M | - Zone Switch Mode Support | 1 | Indicates whether the AMS has a capability to maintain its data communication with the ABS in LZone while performing network reentry in MZone 0b0: Not supported 0b1: Supported | N.A. |
| M | LBS Capabilities | 1 |  | N.A. |
| M | - Capability for supporting GPS method | 1 | 0b0: no support 0b1: support | N.A. |
| M | Capabilities for interference mitigation support | 5 |  | N.A. |
| M | - DL PMI coordination capability | 1 | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable | N.A. |
| M | - DL collaborative multi-BS MIMO capability | 1 | 0: AMS is not DL collaborative multi-BS MIMO capable 1:AMS is DL collaborative multi-BS MIMO capable | N.A. |
| M | - DL closed-loop multi-BS macro diversity capability | 1 | 0: AMS is not DL closed-loop multi-BS macro diversity capable 1:AMS is DL closed-loop multi-BS macro diversity capable | N.A. |
| M | - Multi-BS sounding calibration capability | 1 | 0: not support 1: support | N.A |
| M | - UL PMI combination capability | 1 | 0: AMS is not UL PMI combination capable 1: AMS is UL PMI combination capable | N.A. |
| M | E-MBS capabilities | 3 | If Bit#0 is set to 1, it indicates E-MBS in Serving ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. | N.A. |
| M | channel BW and Cyclic prefix | 15 | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) | N.A. |
| M | frame configuration to support legacy(5MHz) | 1 | If Bit#0 =1,it supports(TDD only) | N.A. |
| M | frame configuration to support legacy(10MHz) | 1 | If Bit#0 =1,it supports(TDD only) | N.A. |
| M | Wireless MAN-OFDMA system(FDM-based UL PUSC zone) support | 1 | If Bit#0 =1,it supports | N.A. |
| M | MAX Tx Power | 24 | Bit 0–7: Maximum transmitted power for QPSK. Bit 8–15: Maximum transmitted power for 16-QAM Bit 15–23: Maximum transmitted power for 64-QAM. | N.A. |
| M | Persistent Allocation support | 1 | If Bit#0 =1,it supports | N.A. |
| M | Group Resource Allocation support | 1 | If Bit#0 =1,it supports | N.A. |
| M | Co-located coexistence capability support | 5 | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class | N.A. |
| M | HO trigger metric | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports | N.A. |
| M | Capabilities for Handover EBB mode | variable |  | N.A. |
| M | - EBB support | 1 | Indicates whether the AMS has a capability to support EBB operation during handover 0b0: Not supported 0b1: Supported | N.A. |
| O | - Minimal HO_Reentry_Interleaving_Interval | 8 | The minimal HO_Reentry_Interleaving_Interval measured in frames. For MC HO capable AMS, this value shall be 0. | It shall be included only if EBB support is set to 1 |
| M | AMS initiated aGP Service Adaptation Capability: | 1 | 0b0: no support 0b1: support | N.A. |
| M | DCR mode support | 1 | 0b0: no support 0b1: support | N.A. |
| M | Convergence sublayer capabilities | 22 |  | N.A. |
| M | - CS type support | 16 | It indicates which CS type(s) the AMS supports( refer to table xxx CS type to support) | N.A. |
| M | - Maximum number of classification rules | 2 | Maximum number of simultaneous admitted classification rules that a service flow supports. | N.A. |
| M | - ROHC support | 1 | 0: not support 1: support | N.A. |
| M | - PHS support | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS | N.A. |
| M | - Supported-IP-Versions IE | 2 | Indicates whether the AMS supports IPv4, IPv6 or both. 0b00: reserved 0b01: IPv4 support 0b10: IPv6 support 0b11: both IPv4 and IPv6 support | N.A. |
| M | Host configuration capabilities and parameters | variable |  | N.A. |
| O | - Host-Configuration-Capability-Indicator IE | 1 | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI_REG-RSP message. This shall be omitted if Requested-Host-Configurations IE is included in the message. 0b0: not support 0b1: support | To be included when Requested-Host-Configurations IE is not included |
| O | - Requested-Host-Configurations IE | variable | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI_REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. | To be included when additional host configurations are required. |
| M | The Number of Uplink transport FID Supported | 4 | The number of uplink transport FIDs the AMS supports | N.A. |
| M | The Number of Downlink transport FID Supported | 4 | The number of donwlink transport FIDs the AMS supports | N.A. |
| M | Global carrier configuration change count | 3 | Indicates the AMS's last received value of Global carrier configuration change count of the network. If set to 0, it implies that AMS never received a Global carrier configuration information. | N.A. |

Table xxx – CS type to support

| Bit | CS | CS subpart | Traffic constraint |
| --- | --- | --- | --- |
| 0 | Reserved |  |  |
| 1 | Packet (5.2) | IP (5.2.5) | IPv4 Traffic Only |
| 2 | Packet (5.2) | IP (5.2.5) | IPv6 Traffic Only |
| 3 | Reserved |  |  |
| 4 | Reserved |  |  |
| 5 | Reserved |  |  |
| 6 | Reserved |  |  |
| 7 | Reserved |  |  |
| 8 | Reserved |  |  |
| 9 | Reserved |  |  |
| 10 | Reserved |  |  |
| 11 | Reserved |  |  |
| 12 | Reserved |  |  |
| 13 | Reserved |  |  |
| 14 | Packet (5.2) | IP (5.2.5) | None |
| 15 | Packet (5.2) | Multiprotocol (5.2.6) | IPv4 or IPv6 Traffic |

16.2.3.8 AAI_REG-RSP

An AAI_REG-RSP message is transmitted by ABS in response to AAI_ REG-REQ message during initialization.

The AAI_REG-RSP message shall be encrypted and not contain CMAC Tuple if authentication has been completed.

[note to editor : replace table 689 with the following table]

Table 689 - AAI_REG-RSPmessage Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | STID | 48 | AMS identifier which the ABS assigns to the AMS in place of the temporary STID which has been transferred by AAI-RNG-RSP message. | N.A. |
| O | CRID | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode | To be included when the DCR mode or coverage loss recovery is supported |
| M | ARQ parameters | variable | ARQ related parameters | N.A |
| M | - ARQ_SN_MODULUS | 10 | the number of unique SN values | N.A |
| M | - ARQ_WINDOW_SIZE | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks | N.A |
| M | - ARQ_SUB_BLOCK_SIZE | TBD | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | N.A |
| M | - ARQ_BLOCK_LIFETIME | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | N.A |
| M | - ARQ_RX_PURGE_TIMEOUT | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START | N.A |
| O | Femto ABS LDM parameters | 21 |  | to be included when the femto ABS support LDM |
| O | - Start_frame_number | 9 | Start superframe Offset |  |
| O | - Available_interval_length | 4 | Length of available intervals in units of four frames |  |
| O | - Unavailable_interval_length | 8 | Length of unavailable intervals in units of four frames |  |
| M | Multicarrier capabilities | 2 | 0b00: No MC modes 0b01: Basic MC mode 0b10: Multicarrier Aggregation 0b11: Multicarrier Switching | N.A. |
| M | LBS Capabilities | 1 |  | N.A. |
| M | - Capability for supporting GPS method | 1 | 0b0: no support 0b1: support | N.A. |
| M | Capabilities for interference mitigation support | 5 |  | N.A. |
| M | - DL PMI coordination capability | 1 | 0: ABS is not DL PMI coordination capable 1: ABS is DL PMI coordination capable | N.A. |
| M | - DL collaborative multi-BS MIMO capability | 1 | 0: ABS is not DL collaborative multi-BS MIMO capable 1:ABS is DL collabortive multi-BS MIMO capable | N.A. |
| M | - DL closed-loop multi-BS macro diversity capability | 1 | 0: ABS is not DL closed-loop multi-BS macro diversity capable 1:ABS is DL closed-loop multi-BS macro diversity capable | N.A. |
| M | - Multi-BS sounding calibration capability | 1 | 0: not support 1: support | N.A |
| M | - UL PMI combination capability | 1 | 0: ABS is not UL PMI combination capable 1: ABS is UL PMI combination capable | N.A. |
| M | E-MBS capabilities | 3 | If Bit#0 is set to 1, it indicates E-MBS in Serving ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. | N.A. |
| M | Persistent Allocation support | 1 | If Bit#0 =1,it supports | N.A. |
| M | Group Resource Allocation support | 1 | If Bit#0 =1,it supports |  |
| M | Co-located coexistence capability support | 5 | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class | N.A. |
| M | HO trigger metric | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports | N.A. |
| M | AMS initiated aGP Service Adaptation Capability: | 1 | 0b0: no support 0b1: support | N.A. |
| M | DCR mode support | 1 | 0b0: no support 0b1: support | N.A. |
| M | Convergence sublayer capabilities | 38 |  | N.A. |
| M | - CS type support | 16 | It indicates which CS type(s) the AMS supports( refer to table xxx CS type to support) | N.A. |
| M | - Maximum number of classification rules | 2 | Maximum number of simultaneous admitted classification rules that a service flow supports. | N.A. |
| M | - ROHC support | 1 | 0: not support 1: support | N.A. |
| M | - PHS support | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS | N.A. |
| M | - Resource_Retain_Time | 16 | 0-65535: In units of 100 milliseconds Duration that the serving ABS shall retain the AMS context | N.A. |
| M | - IP-Service-Type IE | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved | N.A. |
| O | Host configuration capabilities and its parameters | variable |  |  |
| O | - IPv4-Host-Address IE | 32 | The allocated IPv4 Host Address for the AMS. | May be included when AMS indicates its capability of configuring host parameters |
| O | - IPv6-Home-Network-Prefix IE | 64 | The allocated IPv6 Home Network Prefix for the AMS. | May be included when AMS indicates its capability of configuring host parameters |
| O | - Additional-Host-Configurations IE | variable | Includes additional host configuration options in DHCP Options format | May be included when AMS sends Requested-Host-Configurations IE in AAI_REG-REQ or network decided to configure Additional-Host-Configuration parameters |
| O | Redirection Info[0..256] | variable |  | May be included when the AMS does not support CSG whitelist capability or does not have any CSGID(s) provisioned in its CSG whitelist |
| O | - ABSID for neighbor ABS | 48 | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry. |  |
| O | - preamble index for neighbor ABS | 10 |  |  |
| O | - center frequency for neighbor ABS | 32 |  |  |
| M | The Number of Uplink transport FID Supported | 4 | The number of uplink transport FIDs the ABS supports | N.A. |
| M | The Number of Downlink transport FID Supported | 4 | The number of donwlink transport FIDs the ABS supports | N.A. |
| M | Total number of provisioned service flow | 4 | Total number of provisioned service flows to be setup for an AMS | N.A. |
| M | Global carrier configuration change count | 3 | Indicates the current value of Global carrier configuration change count of the network | N.A |

----------------------------------------------------- End of Proposed Text ---------------------------------------------------

# References

- IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Feb 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.