| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Multicarrier Operation for Femto ABS for the IEEE802.16m/D5 (16.4) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Motoki Morita, Yasuhiko Matsunaga, Nader Zein, Linghang Fan, Hassan Al-Kanani, Tetsu Ikeda NEC | m-morita@bx.jp.nec.com |
| Re: | LB comment to 802.16m Amendment Working Document D5 |  |
| Abstract | This contribution provides a scheme allowing multi-carrier operation for Femtocell ABS. |  |
| Purpose | For discussion and approval by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text for the IEEE 802.16m AWD (section 16.4):

# Multicarrier Operation for Femtocell ABS (section 16.4)

Motoki Morita, Yasuhiko Matsunaga, Nader Zein, Linghang Fan, Hassan Al-Kanani, Tetsu Ikeda

NEC

# Introduction

This contribution proposes a scheme that allows multicarrier operation for Femtocell ABS without increasing interference to macro/micro cells.

# Multicarrier Operation

## Multicarrier Operation for Femtocell ABS

Multicarrier operation may be supported by Femtocell ABS as depicted in Fig.1. All operational principles for multicarrier operation outlined in section 16.2.8 of D5 are applied to the multicarrier operation for Femtocell. In addition, the following principles shall also be applied to multicarrier operation for Femtocell in order to mitigate interference.

Fig.1 Illustration of the concept of multicarrier operation for Femtocell ABS

## Interference in Multicarrier Operation for Femtocells

If Femto ABS selects a secondary carrier which is the same carrier as the one used by neighbor ABS, the interference from Femtocell to neighbor cells may subsequently increase. This may occasionally happen when multicarrier operation scheme is enabled for Femtocell since secondary carrier management does not consider the interference impact to the neighbor cells.

A scenario in which interference may increase in multicarrier operation for Femtocell ABS is described in Fig.2. RF2 is assigned to a MS connected to the macro ABS (macro MS) as a primary carrier. RF1 is assigned to a MS connected to the Femto ABS as a primary carrier. RF2 is to be assigned as a secondary carrier for the MS connected to the Femto ABS. If RF2 is assigned by the Femtocell ABS, the interference to the macro MS may increase and performance of the macro cell will subsequently be degraded.

Fig.2 Illustration of the scenario that interference increases in multicarrier operation for Femtocell ABS

## Proposed Scheme for Interference Mitigation in Multicarrier Operation

In case of multicarrier operation for Femtocell ABS, the Femtocell ABS should consider interference avoidance or mitigation when assigning the secondary carrier.

In assigning the secondary carrier, the Femtocell ABS should decide activating or deactivating of secondary carrier(s) based on factors such as load condition of each carrier of the neighbor ABS.

Femto ABSs monitor the load condition at the initial setting and then monitor it periodically. If the DL or UL available radio resource of the neighbor ABS for the carrier is small, then the Femtocell ABSs should avoid using this carrier as the secondary carrier. The carrier load condition can be communicated through to the Femto ABS by the SON server.

.

Load condition

The

Femtocell

ABS receives the

ABS

’

s

load condition and avoids using the

RF2 as the secondary carrier

RF1

RF2

Primary

carrier

Not to be used as

secondary carrier

Carrier assignment for a MS in

Femtocell

.

Load condition

(Through SON server)

The

Femtocell

ABS receives the

ABS

’

s

load condition and avoids using the

RF2 as the secondary carrier

RF1

RF2

Primary

carrier

Not to be used as

secondary carrier

Carrier assignment for a MS in

Femtocell

Fig.3 Proposed Scheme for interference mitigation in multi-carrier operation

# Proposed Text

-------------------------------------------------------Start of Text -------------------------------------------------------

[Insert text in subclause 16.4 as follows]

16.4 Support for Femto ABS

16.4.x Multicarrier Operation

Multicarrier operation may be supported by Femto ABS. All operational principles for multicarrier operation set out in section 16.2.8 are applied to the multicarrier operation for Femto ABS. In addition, the following principles shall also be adopted for multicarrier operation for Femtocell in order to mitigate interference.

In assigning the secondary carrier, the Femto ABS should decide activating or deactivating the secondary carrier(s) based on factors such as load condition of each carrier of the neighbor ABS.

Femto ABSs may monitor the load condition at the initial setting and then monitor it periodically. If the DL or UL available radio resource for a carrier from a neighbor ABS is small, the Femto ABSs should avoid using this carrier as a secondary carrier. The carrier load condition may be communicated through with the femto ABS by the SON server.

--------------------------------------------------------End of the Text-------------------------------------------------------