| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to “Modulation Accuracy and Error Vector Magnitude (16.3.13)” |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Rongzhen Yan, Yang-seok Choi, Tom Harel, Hujun Yin, Amir Rubin, Jin Fu and Rui Huang Intel Corp | Rongzhen.Yang@intel.com |
| Re: | P802.16m/D5 LB31a |  |
| Abstract | Modifications are proposed for “Modulation Accuracy and Error Vector Magnitude (16.3.13)” |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 5) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to “Modulation Accuracy and Error Vector Magnitude (16.3.13)”

Rongzhen Yan, Yang-seok Choi, Tom Harel, Hujun Yin, Amir Rubin, Jin Fu and Rui Huang

Intel Corp

- Introduction

EVM definition in 802.16m is directly referenced to 802.16-2009 section 8.1.8.2.3. This definition is not appropriated due to

- Section 8.1.8.2.3 is EVM definition and limitation for WirelessMAN-SC, not for OFDM/OFDMA;

- The definition don’t consider the impaction of EVM noise on modulated tone and un-modulated tones;

In 802.16-2009 section 8.4.13.3 “Transmitter Constellation Error and Test Method” has defined the relative constellation RMS error average over subcarriers, for modulated and un-modulated tones. However, the definition and limitation is not appreciated due to:

- The limitation in Table 544 was derived from very simply rule, the impaction of EVM noise for capacity loss is not studied;

- The EVM noise on modulated and un-modulated tones, are evaluated and limited separately. It is not appreciated because EVM noise on un-moudulated will be changed according to the ratio of tested bandwidth of modulated tones;

- For highest modulation in uplink, 64 QAM, is low possibility to used in full Tx power, so, the test condition of power reduction comparing to the maximum Tx power, which is not addressed;

In this contribution, we propose the 16m EVM definition base on the study of EVM noise.

- EVM Definition Analysis

(To be added).

- References

[1] Draft Amendment, P802.16m/D5, Apr. 2010.

- Text proposal for inclusion in the 802.16m amendment

Instruction to editor;

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

[Remedy 1:]

[Change the text from line 24 on the page 119, section 16.2.3.32 as follows:]

============================== Start of Proposed Text =================================

16.3.13 Modulation accuracy and error vector magnitude (EVM) Error Vector Magnitude and Requirements

16.3.13.1 Definition

The method for calculation of the EVM in AAI shall be the same as that described in 8.1.8.2.3.

The EVM is calculated using Equation (xxx):

EVM=10log101Nfi=1Nfj=1LSk∈S⊕SuI(i,j,k)−I0(i,j,k)2+Q(i,j,k)−Q0(i,j,k)2j=1LPk∈SI0(i,j,k)2+Q0(i,j,k)2

Where

Nf is the number of frames for the measurement;

LS is the number of symbols used for the measurement;

S is the group of modulated data subcarriers;

Su is the group of un-modulated data subcarriers. It includes all subcarriers in the range 0 … Nused – 1, except the DC and the modulated subcarriers (in S);

I0(i,j,k), Q0(i,j,k) denotes the ideal symbol point of the i-th frame, j-th OFDMA symbol, k-th subcarrier in the complex plane. It is set as (0, 0) in the un-modulated data subcarriers.

I(i,j,k), Q(i,j,k)denotes the observed point of the i-th frame, j-th OFDMA symbol, k-th subcarrier in the complex plane;

By using the equation (xxx), the EVM noise in both modulated and un-modulated tones are summed.

16.3.13.2 Basic Requirements

To ensure that the system performance does not degrade obviously due to EVM noise, the relative basic EVM measurement, averaged over subcarriers, symbols and frames, shall not exceed the value in Table xxx.

Table xxx EVM Requirement

| Modulation | Unit | Required EVM Level | Power Reduction (Only Applied for Uplink) |
| --- | --- | --- | --- |
| QPSK | dB | -16 | 0 |
| 16QAM | dB | -19 | 0 |
| 64QAM | dB | -24 | 3 |

Power reduction is performed for the EVM test in uplink comparing to the maximum transmission power.

============================== End of Proposed Text =================================