| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Correction of Power Boosting/De-Boosting Values for Reuse-2 (Section 16.3.6.5.1.2) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Bill Hillery, Fred Vook, Mark Cudak Motorola Inc. | bill.hillery@motorola.com |
| Re: | Category: P802.16m/D5 comments for LB31a Area: Sections 16.3.6.5.1.2 (S-SFH IE) |  |
| Abstract | The table mapping the fpPower Config field value to the frequency partition power boosting and de-boosting values is clearly incorrect for reuse-2 (FPCT=2 or 3 and FPS3=0). This contribution proposes table entries which correctly implement this feature for reuse-2. |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Correction of Power Boosting/De-Boosting Values for Reuse-2 (Section 16.3.6.5.1.2)

Bill Hillery, Fred Vook, Mark Cudak

Motorola, Inc.

# Introduction

Table 840 in the IEEE 802.16m draft amendment [1] maps the value of the fpPowerConfig field in the S-SFH SP3 IE to the power boosting and de-boosting values for the FFR frequency partitions.  The values currently given in this table for reuse-2 (FPCT = 2 or 3 and FPS3 = 0 in the last two columns of the table) are clearly incorrect since both FFR partitions always have the same power.  Either both should be boosted by 0 dB or one partition (FP1) should be boosted (power boosting > 0 dB) and the other partition (FP2) should be de-boosted (power boosting < 0 dB).  This contribution proposes de-boosting values for reuse-2 which fix this problem.  The proposed de-boosting values were calculated by accepting the boosting values given for FP1 for reuse-2 and calculating the required de-boosting which maintains constant total power.  This methodology is consistent with the approach used for the reuse-3 boosting and de-boosting choices.

In addition, modified column headings were proposed and accepted in comment A0642 during session 66 in Orlando, FL [2], but were not implemented.  These column heading changes are also proposed in this contribution and make the table headings consistent with Figures 458 and 459 and related text in [1].

# References

[1] 	IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[2]	80216-10/0018r5, “Letter Ballot #31 Comment Database,” 2010-04-10.

# Proposed Text Changes in IEEE P802.16m/D5

[Modify Table 840 on page 535 as follows:]

Table 840—Power boosting/de-boosting values

| fpPower Config | FPCT=3 or 4 and FPSi>0 for 1≤i≤3 |  |  | FPCT=2 or 3 and FPS3 = 0 |  |
| --- | --- | --- | --- | --- | --- |
|  | FP1 power FP1_Power (dB) | FP2 power FP2_Power (dB) | FP3 power FP3_Power (dB) | FP1 power FP1_Power (dB) | FP2 power FP2_Power (dB) |
| 0b000 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 0b001 | 1.0 | -0.5 | -0.5 | 0.5 | 0.5-0.5 |
| 0b010 | 2.0 | -1.5 | -1.5 | 1.0 | 1.0-1.5 |
| 0b011 | 2.5 | -2.0 | -2.0 | 1.5 | 1.5-2.5 |
| 0b100 | 3.0 | -3.0 | -3.0 | 2.0 | 2.0-4.0 |
| 0b101 | 3.5 | -4.5 | -4.5 | 2.5 | 2.5-6.5 |
| 0b110 | 4.0 | -6.0 | -6.0 | 3.0 | 3.0-Inf |
| 0b111 | 4.75 | -Inf | -Inf | reserved | reserved |