| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup for the AAI_LBS-ADV MAC Control Message in IEEE P802.16m (16.2.4.1) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D5,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #31a: Announcement, IEEE 802.16-10/0032 |  |
| Abstract | ASN.1 format for the AAI_LBS-ADV message on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Cleanup for the AAI_LBS-ADV MAC Control Message in IEEE P802.16m (16.2.4.1)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

This contribution reformats the current AAI_LBS-ADV message in [1] as the ASN.1 format.

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Replace Table 973 with the following table in 16.8.2.4.1 of 802.16m/D5]

Table 973—AAI_LBS-ADV message format

| M/O |  | Attributes/Array of attributes |  |  |  |  | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M |  | Control Message Type |  |  |  |  | 8 | AAI_LBS-ADV | N.A. |
| M |  | Position Indication |  |  |  |  | 1 | Represent absolute position whether using long format or short format. The value indicates: 0b0: long format is used 0b1: short format is used | N.A |
| O |  | Type indication of short format |  |  |  |  | 1 | 0b0: short format including altitude is used. 0b1: short format excluding altitude is used | Present when Position Indication is set to 0b1 |
| O | M | Absolute Position (Long Format) |  | Longitude |  |  | 40 | Bit 0-5: longitude resolution 1-34 - # of valid bits in fixed-point value of longitude value 35 – LBS not supported Other – Reserved Bit 6-14: longitude integer Bit 15-39: longitude fraction | Present when Position Indication is set to 0b0. The transmitting ABS's coordinates. This Absolute Position (Long Format) shall only be used for the transmitting ABS and includes Longitude, Latitude, and Altitude. |
|  | M |  |  | Latitude |  |  | 40 | Bit 0-5: latitude resolution 1-34 - # of valid bits in fixed-point value of latitude value 35 – LBS not supported Other – Reserved Bit 6-14: latitude integer Bit 15-39: latitude fraction |  |
|  | M |  |  | Altitude |  |  | 40 | Bit 0-3: altitude type 1 – meter 2 – floor Other – Reserved Bit 4-9: altitude resolution 1-30 - # of valid bits in fixed-point value of altitude value 31 – LBS not supported Other – Reserved Bit 10-31: altitude integer Bit 32-39: altitude fraction |  |
| O | M | Absolute Position (Short Format) |  | Longitude |  |  | 24 | Longitude expressed in 2-15 parts of a degree | The transmitting ABS's coordinates. This Absolute Position (Short Format) shall only be used for the transmitting ABS. It includes Longitude, Latitude, and Altitude if type indication of short format is 0b0. It includes Longitude and Latitude only if Position Indication is 0b1. |
|  | M |  |  | Latitude |  |  | 24 | Latitude expressed in 2-16 parts of a degree |  |
|  | O |  |  | Altitude |  |  | 16 | Bit 0-15: altitude in meters above sea level |  |
| M | M | GPS Time |  | GPS time in units of frame duration |  |  | 22 | Refer to 11.21.4 | Information about GPS time and time accuracy. |
|  | M |  |  | GPS frame transmission time offset |  |  | 10 |  |  |
|  | M |  |  | GPS time accuracy |  |  | 5 |  |  |
| M |  | Frequency Accuracy |  |  |  |  | 8 | Information about the frequency accuracy. It is included once in the message. Refer to 11.21.5 |  |
| M |  | Number of BS |  |  |  |  | 8 | Number of neighbor BSs included in this message that are identified using the BSID. |  |
| O | M | fullNumberOfBS[1…256] | BSID |  |  |  | 16 | 16-bit LSB of unique 48-bit identifier of the Neighbor BS for number of BS. | Present when Number of BS > 0 It excludes Altitude when Position Indication is set to 0b1 and Type indication of short format is 0b1 |
|  | M |  | Relative Position |  |  | Longitude | 16 | Distance east of reference point in meters |  |
|  | M |  |  |  |  | Latitude | 16 | Distance north of reference point in meters |  |
|  | M |  |  |  |  | Altitude | 16 | Distance above of reference point in meters |  |
| M |  | Number of BS Index |  |  |  |  | 8 | Number of neighbor BSs included in this message that are identified using an index to their position in the AAI_NBR-ADV message |  |
| O | M | Configuration change count for AAI_NBR-ADV |  |  |  |  | 8 |  | Present when Number of BS Indices > 0 It excludes Altitude when Position Indication is set to 0b1 and Type indication of short format is 0b1 |
|  | M | fullNumberOfBSIndex[1…256] | BSID |  |  |  | 16 | 16-bit LSB of unique 48-bit identifier of the Neighbor BS for number of BS. |  |
|  | M |  | Relative Position |  | Longitude |  | 16 | Distance east of reference point in meters |  |
|  | M |  |  |  | Latitude |  | 16 | Distance north of reference point in meters |  |
|  | O |  |  |  | Altitude |  | 16 | Distance above of reference point in meters |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” April 2010