| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up AAI_SON-ADV/AAI_NBR-REQ/AAI_SII-ADV |  |
| Date Submitted | 2010-05-12 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes changes toclean-up AAI_SON-ADV, AAI_NBR-REQ, and AAI_SII-ADV messages |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Clean-up AAI_SON-ADV/AAI_NBR-REQ/AAI_SII-ADV

- Joey Chou, Intel

- Introduction

Some MAC messages use the tables that are not consistent with the table template or do not use table at all. This contribution provides tables to AAI_SON-ADV, AAI_NBR-REQ, and AAI_SII-ADV messages.

- Proposed text

II.1 Proposed text 1

16.2.3.6 AAI_SON-ADV

------------------------------------------------- Start of proposed text I --------------------------------------------------

[Replace Table 687 with the following table]

Table 688: AAI_SON-ADV

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Action type | 2 | Duration (in units of AAI subframes) of the requested scanning period. | Used to indicate the purpose of this message 0b00: ABS Reconfiguration 0b01: ABS Restart 0b10: ABS Scanning 0b11: ABS Reliability |
| O | Unavailable start time (UST) |  | Start of unavailable time | Present if Action type = 0b00, 0b01, 0b10) |
| O | Unavailable Time Interval (UTI) |  | Interval of unavailable time | Present if Action type = 0b00, 0b01, 0b10) |
| O | Reason | 2 | 0b00: Power down 0b01: Power reduction 0b10: FA change 0b11: Backhaul link down | Present if Action type=0b11 |
| O | Tx power reduction |  | dB value of Tx power reduction | Present if Reason = 0b01 |
| O | FA index |  | FA index | Present if Reason = 0b10 |
| O | Time of power down |  | Expected power down time or resource adjustment time or current FA downtime | Present if Reason = 0b11 |
| O | Expected uptime or new FA uptime |  |  | Present if Reason = 0b11 |
| O | Recommended BSID list |  | Recommended BSID list | Present to help AMS to HO. |

------------------------------------------------- End of proposed text I--------------------------------------------------

II.2 Proposed text 2

16.2.3.36 AAI_NBR-REQ

------------------------------------------------- Start of proposed text II --------------------------------------------------

[Add a new Table xxx]

Table xxx: AAI_NBR-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | BSID | 16 | 16-bit LSB of unique 48-bit identifier of neighbor BS | NA |
| M | Request BS type | 2 | 00: CSG Femto ABS 01: OSG Femto ABS 10-11: reserved | NA |

------------------------------------------------- End of proposed text II --------------------------------------------------

II.3 Proposed text

16.2.3.15 AAI_SII-ADV

------------------------------------------------- Start of proposed text III --------------------------------------------------

[Replace Table 751 with the following table]

Table 670: AAI_SII-ADV

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | nspArray [1..16] | Variable | List of one or more Network Service Provider 24 bit identifier |  |
| M | A) nspIdentifier | 24 | NSP identifiers to report. The serving ABS shall indicate only the trigger |  |
| M | verboseNspNameArray [1..16] | Variable | List of the verbose names of the NSPs. The value of Verbose NSP Name List is a compound list of verbose NSP name lengths and verbose NSP names. The order of the Verbose NSP Name Lengths and Verbose NSP Names presented shall be in the same order as the NSP IDs presented in the NSP List. |  |
| M | A) verboseNspNames | Variable | Verbose NSP name string |  |

- ------------------------------------------------- End of proposed text III ----------------------------