| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Enhanced LBS Support (16.8.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Alexey Khoryaev, Kamran Etemad, Alexander Maltsev, Mikhail Shilov, Yang-seok Choi, Hujun Yin, Amir Rubin, Tom Harel Intel Corporation Roman Maslennikov University of Nizhny Novgorod | E-mails: alexey.khoryaev@intel.com kamran.etemad@intel.com alexander.maltsev@intel.com roman.maslennikov@wcc.unn.ru |
| Re: | Comments on IEEE P802.16m/D5 for IEEE 802.16 Working Group Letter Ballot # 31a Topic: LBS (Section 16.8.3) |  |
| Abstract | This contribution proposes amendment text describing design of downlink LBS zone for enhanced LBS support |  |
| Purpose | To be discussed and adopted by TGm for theP802.16m/D6. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Enhanced LBS Support in Downlink
for the IEEE 802.16m AWD

Alexey Khoryaev, Kamran Etemad, Alexander Maltsev, 
Mikhail Shilov, Yang-seok Choi

Intel Corporation

- Introduction

This contribution proposes amendment text describing the design of Downlink-LBS (D-LBS) zone for Enhanced LBS support in IEEE 802.16m systems and to be included in the IEEE 802.16m amendment draft [5]. The proposed text is developed so, that it is compliant to the IEEE 802.16m SRD [2] and the IEEE 802.16m SDD [3], and it follows the style and format guidelines in [4]. Current IEEE 802.16m SDD [3] is used as a basis for the text development.

The D-LBS zone is introduced to improve the performance characteristics of non-GPS-based location methods which rely on signal location parameters measured using reference signals transmitted by serving and neighboring ABSs/ARSs of the network [3]. The proposed D-LBS zone facilitates more accurate measurements of signal location parameters that are used for user positioning. The efficiency of proposed method was verified in severe multipath channel and strong interference environment using LBS evaluation assumptions defined in [6]. It was shown by link and system level simulations [7], that implementation of D-LBS zone substantially improves accuracy of user positioning and meet LBS requirements [2].

- References

- IEEE P802.16Rev2/D9a, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,” March 2009.

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/0034r2, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- IEEE P802.16m/D3, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- IEEE C802.16m-09/2202 “System Level Evaluation Assumptions for LBS Performance Analysis” (Alexey Khoryaev, Alexander Maltsev, Lei Zhou, Sangheon Kim, Fangmin Xu, Jinsoo Choi, Kanghee Kim, Kunmin Yeo; 2009-09-22)

- IEEE C802.16m-09/2201r1 “Evaluation of D-TDOA Positioning” (Alexey Khoryaev, Alexander Maltsev, Kamran Etemad et al; 2009-09-22)

- IEEE C802.16m-09/2314r2 “Proposed Text for Enhanced LBS Support (15.8.3)” (Lei Zhou, Fangmin Xu, Xufeng Zheng, Zheng Zhao; 2009-11-17)

=========================Start of Proposed Text  ==============================

[Note to Editor: Add the following underlined blue text into 16.8.3 Enhanced LBS Support section on page 746]

16.8.3.1 Enhanced LBS support in downlink

The enhanced LBS support in downlink involves coordinated transmission of special location beacons across multiple ABS’s in a designated Downlink LBS zone to facilitate AMS’s measurements of parameters related to location determination.

16.8.3.1.1 Basic description of downlink LBS zone

To enable enhanced location determination, a dedicated Downlink LBS zone (D-LBS zone) may be defined to transmit location beacons to be received by AMS’s and facilitate their measurement of location related parameters (RD, RTD, RSSI, etc.) with the higher accuracy.

Such dedicated D-LBS zone, when supported, shall be spread over eight consecutive superframes. The location beacon shall be transmitted on even superframes of D-LBS zone and shall be placed in the last frame of superframe at the position of MIMO-Midamble.

The ABSs/ARSs configured to support D-LBS zone shall coordinate and transmit location beacons in accordance with the predefined D-LBS transmission plan.

16.8.3.1.2 Allocation of D-LBS zone in frame structure

When the D-LBS zone is allocated, the position of location beacon transmission shall be fixed inside superframes of D-LBS zone.

The one D-LBS zone shall span eight consecutive superframes. The only even superframes of D-LBS zone shall carry location beacons.

When the D-LBS zone is enabled the MIMO-Midamble transmitted on the last frame of even superframe shall be replaced by location beacon. Figure 1 shows an example of the D-LBS zone allocation. The parameter DLZP in Figure 1 is the D-LBS zone transmission period (DLZP) measured in superframes units.

Figure 1. D-LBS zone allocation

16.8.3.4 D-LBS zone location beacon signals

The SA-Preamble shall be used as a reference location beacon signal for transmission inside of D-LBS zone. The physical structure of the SA-Preamble signal transmitted by each ABS/ARS in the D-LBS zone shall be the same as defined in section 16.3.6.1.2 for given superframe.

Each ABS/ARS shall transmit the corresponding SA-Preamble signal in the D-LBS zone accordance with the predefined transmission plan that depends on the IDcell/segment parameter assigned to ABSs/ARSs. The location beacon transmission plan provides the time multiplexed transmission of these reference signals across neighboring ABS/ARS’s to simplify AMS’s detection and measurements of the relevant signal location parameters from several ABSs/ARSs.

The D-LBS zone transmission plan spreads location beacon transmissions from different ABS/ARSs over D-LBS zone OFDMA symbols. To facilitate the wideband channel measurement the SA-Preamble reference signals allocated in the D-LBS zone shall be transmitted from one transmit antenna.

16.8.3.4 Predefined D-LBS zone transmission plan

The predefined D-LBS zone transmission plan specifies on which orthogonal resource (symbol and carrier set) of D-LBS zone the location beacon shall be transmitted. To define transmission plan the existing set of SA-Preambles shall be partitioned into Q preamble location/LBS groups (PLGs). To determine PLG the following equation shall be used:

PLGi=mod(mod(IDcell, 256), Q)				(1)

The number of preamble location/LBS groups Q shall be set to 12, that is equal to the number of orthogonal resources available in one D-LBS zone. The Table 1 determines the predefined D-LBS zone transmission plan that shall be used for transmission of location beacons.

Table 1. Predefined D-LBS zone transmission plan.

| Allocated carrier set | 1st symbol of D-LBS zone | 2nd symbol of D-LBS zone | 3rd symbol of D-LBS zone | 4th symbol of D-LBS zone |
| --- | --- | --- | --- | --- |
| Carrier Set n = 0 | PLG0 | PLG1 | PLG2 | PLG3 |
| Carrier Set n = 1 | PLG4 | PLG5 | PLG6 | PLG7 |
| Carrier Set n = 2 | PLG8 | PLG9 | PLG10 | PLG11 |

In accordance with the predefined D-LBS zone transmission plan, each ABS/ARS shall determine PLG number using equation (1). The ABS/ARS shall transmit the location beacon signal on corresponding D-LBS zone symbol and carrier set as it is defined in Table 1. When one station has three segments, all segments shall use the same SA-Preamble sequence for location beacon transmission. In this case, the IDcell that belongs to corresponding PLG and carrier set shall be used for transmission of location beacon represented by the SA-Preamble signal.

========================== End of Proposed Text ==============================