| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Restrictions on Burst Allocation per AMS Considering DL/UL long TTI bursts (16.3.3.1) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Ji-Yun Seol Samsung Electronics Co., LTD. | jiyun.seol@samsung.com |
| Re: | Call for LB #31a on “ P802.16m/D5” Proposed text changes to P802.16m/D5 16.3.3.1 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D6 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Restrictions on Burst Allocation per AMS

Considering DL/UL long TTI bursts (16.3.3.1)

Ji-Yun Seol

Samsung Electronics Co., LTD.

- 1. Introduction

With reference to the previously accepted contribution C80216m-09_2309r6 (adopted in Jan. meeting), it is desired to set a reasonable allocation limitation to avoid implementation complexity without substantial gain for the AMS. However, the proposed text was not added in a proper section that generally describes the allocation scheme. Furthermore, it was missed to add a similar text regarding DL long TTI burst.

It is proposed to further clarify the restrictions on the burst allocation per AMS considering DL/UL long TTI bursts with more reasonable terms for both ABS and AMS.

- 2. References

[1] Draft Amendment, P802.16m/D5

[2] Contribution, C80216m-09_2309r6

- 3. Proposed Text Changes

[Remedy #1: Insert the following texts in line 18, page 419, section 16.3.3.1,]

In the downlink, the total amount of Allocation Sizes (the number of LRUs multiplied by the STC rate per AMS as defined in section 16.3.11.1.2) of the DL bursts allocated to a specific AMS, where the DL bursts end at the the same subframe boundary irrespective of the burst type (default TTI burst and long TTI burst), shall not be larger than the number of LRUs in a single subframe duration multiplied by the maximum allowable DL STC Rate of the AMS as capability negotiated during network entry. That is, ABS shall not allocate to a specific AMS the total amount of Allocation Sizes of the DL bursts, where the sum of the number of Allocation Sizes that default TTI bursts occupy within a specific subframe and the number of Allocation Sizes that long TTI bursts ending in the specific subframe occupy over the long TTI durations is considered, larger than the maximum allowable Allocation Size within a subframe considering the maximum DL STC Rate supported by the AMS. In case of multi-carrier aggregation support, the number of LRUs in a subframe duration considers the total number of LRUs within a subframe duration over the used carriers by the AMS.

In the uplink, the total amount of Allocation Sizes (the number of LRUs multiplied by the STC rate per AMS as defined in section 16.3.11.1.2) of the UL bursts allocated to a specific AMS, where the UL bursts start from the the same subframe boundary irrespective of the burst type (default TTI burst and long TTI burst), shall not be larger than the number of LRUs in a single subframe duration multiplied by the maximum allowable UL STC Rate of the AMS as capability negotiated during network entry. That is, ABS shall not allocate to a specific AMS the total amount of Allocation Sizes of the UL bursts, where the sum of the number of Allocation Sizes that default TTI bursts occupy within a specific subframe and the number of Allocation Sizes that long TTI bursts starting from the specific subframe occupy over the long TTI durations is considered, larger than the maximum allowable Allocation Size within a subframe considering the maximum UL STC Rate supported by the AMS. In case of multi-carrier aggregation support, the number of LRUs in a subframe duration considers the total number of LRUs within a subframe duration over the used carriers by the AMS.

[Remedy #2: Delete the texts in line 45 ~ line 49, page 548, section 16.3.6.5.2.4.2 as follows,]

In the UL, the amount of LRUs that the ABS allocates to a specific AMS, starting in a specific AAI subrame, shall not be larger than the amount of LRUs in that AAI subframe over all used carriers. This limitation considers both bursts that are long TTI and bursts that are not.