| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HARQ timing of Assignment A-MAP IE (16.2.14.2.2) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Yaniv Weizman Alvarion | yaniv.weizman@alvarion.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D5) |  |
| Abstract | Emphasize that HARQ timing is relevant to all Assignment A-MAP IEs resource allocations for HARQ transmission. |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HARQ timing of Assignment A-MAP IE (16.2.14.2.2)

Yaniv Weizman
Alvarion

Problem Statement

Within all subsections of section 16.2.14.2.2, the HARQ timing refers only to HARQ allocations made by Basic Assignment A-MAP IE and Subband Assignmnt A-MAP IE. However, the timing relevant to all Assignment A-MAP IEs used for HARQ resource allocations.

Suggested Solution

Change the text to refer to every Assignment A-MAP IE used for HARQ resource allocation.

Suggested Standard Modifications

Modify text in page 349, line 47 in section 16.2.14.2.2.1.1, as follows:

DL HARQ subpacket transmission corresponding to a DL Assignment A-MAP IE or DL Subband Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th DL subframe of the i-th frame. A HARQ feedback for the DL HARQ subpacket shall be transmitted in the n-th UL subframe of the j-th frame. The subframe index m, n and frame index j shall be determined by using l and i, as shown in Table 787.

Modify text in page 350, line 64 in section 16.2.14.2.2.1.1, as follows:

Figure 433 shows an example of the timing relationship between a DL Basic Assignment A-MAP IE or DL Subband Assignment A-MAP IE, a DL HARQ subpacket with the default TTI, corresponding HARQ feedback, and retransmission in FDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example, TDL_Rx_Processing is 3 AAI subframes.

Modify text in page 351, line 8  in section 16.2.14.2.2.1.2, as follows:

UL HARQ subpacket transmission corresponding to an UL Assignment A-MAP IE or UL Subband Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th UL subframe of the j-th frame. A HARQ feedback for the UL HARQ subpacket shall be transmitted in the l-th DL subframe of the k-th frame. When the DL HARQ feedback indicates a negative acknowledgement, retransmission of the UL HARQ subpacket shall begin in the m-th UL AAI subframe of the p-th frame. The AAI subframe index m, nand frame index j, k, p shall be calculated as shown in Table 788.

Modify text in page 352, line 36 in section 16.2.14.2.2.1.2, as follows:

UL HARQ subpacket transmission corresponding to an UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE in l-th DL AAI subframe of the i-th frame shall begin in the m-th UL AAI subframe of the j-th frame. A HARQ feedback for the UL HARQ subpacket shall be transmitted in the l-th DL AAI subframe of the k-th frame. When the UL HARQ feedback indicates a negative-acknowledgement, retransmission of the UL HARQ subpacket shall begin in the m-th UL AAI subframe of the p-th frame. The AAI subframe index m, n and frame index j, k, p shall be determined by using l and i, as shown in Table 788. Note that the AAI subframe index l shall range from 0 to.

UL HARQ transmission offset v shall be set to 1 only if a time gap from completion of the UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE transmission to the HARQ subpacket transmission time derived with v = 0 is shorter than TUL_Tx_Processing. Otherwise, v shall be set to 0:

Modify text in page 353, line 21  in section 16.2.14.2.2.1.2, as follows:

Figure 434 shows an example of the timing relationship between an UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE, a UL HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission in FDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example TUL_Tx_Processing, and TUL_Rx_Processing are 3 AAI subframes.

Modify text in page 353, line 32  in section 16.2.14.2.2.2.1, as follows:

DL HARQ subpacket transmission corresponding to a DL Assignment A-MAP IE or DL Subband Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th DL subframe of the i-th frame. A HARQ feedback for the DL HARQ subpacket shall be transmitted in the n-th UL subframe of the j-th frame. The subframe index m, n and frame index j shall be determined by using l and i, as shown in Table 789.

Modify text in page 355, line 27  in section 16.2.14.2.2.2.1, as follows:

Figure 435 shows an example of the timing relationship between a DL Basic Assignment A-MAP IE or DL Subband Assignment A-MAP IE, a DL HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission in TDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example, TDL_Rx_Processing is 3 AAI subframes.

Modify text in page 282, line 48 in section 16.2.14.2.2.2.2, as follows:

UL HARQ subpacket transmission corresponding to an UL Assignment A-MAP IE or UL Subband Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th UL subframe of the j-th frame. A HARQ feedback for the UL HARQ subpacket shall be transmitted in the l-th DL subframe of the k-th frame. When the DL HARQ feedback indicates a negative acknowledgement, retransmission of the UL HARQ subpacket shall begin in the m-th UL AAI subframe of the p-th frame. The AAI subframe index m, nand frame index j, k, p shall be calculated as shown in Table 790.

Modify text in page 357, line 46  in section 16.2.14.2.2.2.2, as follows:

Figure 436 shows an example of the timing relationship between an UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE, a UL HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission in TDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example, TUL_Tx_Processing and TUL_Rx_Processing are 3 AAI subframes.

Modify text in page 284, line 22 in section 16.2.14.2.2.2.1, as follows:

Figure 438 and Figure 439 show examples of the DL and UL timing relationships between a Assignment A-MAP IE, a HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission, for 5, 10 and 20 MHz channel bandwidths. The ratio of whole DL AAI subframes to whole UL AAI subframes, D' : U' is 5 : 3. In this example, FRAME_OFFSET is 1, UL AAI subframes of the WirelessMAN-OFDMA and the Advanced Air Interface are frequency-division multiplexed, the ratio of DL to UL AAI subframes for the Advanced Air Interface, D : U is 4 : 3, and TDL_Rx_Processing, TUL_Tx_Processing  and TUL_Rx_Processing are 3 AAI subframes.