| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications for efficient signaling of the modulation information in DL MU-MIMO in the basic A-MAP (16.3.6.5.2.4.1) |  |
| Date Submitted | 2010-04-28 |  |
| Source(s) | Mohammad A. (Amir) Khojastepour, Narayan Prasad, Sampath Rangarajan NEC Laboratories America | amir@nec-labs.com |
| Re: | P802.16m/D5 LB31 |  |
| Abstract | This contribution proposes efficient signaling of the modulation information in DL MU-MIMO, which results in the reduction of the Si field size from 5 bits to 4 bits. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modifications for efficient signaling of the modulation information in DL MU-MIMO in the basic A-MAP (16.3.6.5.2.4.1)

Mohammad A. (Amir) Khojastepour, Narayan Prasad, Sampath Rangarajan

NEC Laboratories America

- Introduction

This contribution proposes a more efficient signaling of the modulation information of the co-scheduled user in the DL basic A-MAP without sacrificing any of the features currently available. Currently, the field Si is used to signal 18 possible options to an MS and thus requires 5 bits. This proposal suggests reducing the size of Si field to 4 bits while still retaining the ability to convey the modulation information of the paired user in the case of TNS = 2.

Three modulations A = QPSK, B = 16-QAM, and C = 64-QAM are possible for DL transmission. Let the pair (., .) denote the modulation information associated with pilot stream0 and pilot stream 1, respectively. We enforce the following restriction for TNS=2. In particular, if one user is assigned the modulation A and the other user modulation B, the user with modulation A will get the pilot stream0 and the user with modulation B will get the pilot stream1. This restriction will not result in a performance loss since both the pilot streams enable similar channel estimation performance. Therefore, as indicated in the following table, there are total of 6 possibilities for the joint stream and modulation assignment in the case of TNS=2.

| ASMI | Modulation type of stream0, and stream1 |
| --- | --- |
| 0 | A, A |
| 1 | B, B |
| 2 | C, C |
| 3 | A, B |
| 4 | B, C |
| 5 | C, A |

ASMI (All Stream Modulation Index) for TNS = 2

Considering that each user knows its own modulation information from its allocated MCS, it can use this information to find out the modulation information of its co-scheduled user. For example, consider Table 848 and the value Si =0b00000 which specifies that the allocated pilot stream is stream0 and that the ASMI lies in the set {0, 1, 2}. In this case from Table 848(a) the user can infer that the modulation type of the first stream is the same as the modulation type of the second stream. Therefore the user with allocated stream0 will know that the modulation type of the co-scheduled user is the same as its own modulation type. Now consider the case of Si=0b00001. In this case the user knows that its allocated stream is stream0 and ASMI is one of three possibilities 3, 4, and 5. Now depending on the modulation type of this user there is only one possibility which matches the type A or B or C. Therefore the modulation type corresponding to stream1 can be found unambiguously.

References

[1] DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D5, March. 2010.

Text Proposal

<Modify the section 16.3.6.5.2.4.1 on line 1, page 543, Table 848 as shown >

# 16.3.6.5.2.4.1 DL basic assignment A-MAP IE

_______________________________Begin proposed text change #1 _________________________________

| Table 848 - DL basic assignment A-MAP IE |  |  |
| --- | --- | --- |
| Syntax | Size in bits | Description/Notes |
| DL_Basic_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Multi-layer encoding 0b11: CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission Nt : Number of transmit antennas at the ABS 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 2 | Reserved bit |
| } else if(MEF == 0b10) { |  | Parameters for Multi-layer encoding |
| Si | 4 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of paired user in the case of 2 stream transmission. ASMI stands for All Stream Modulation Index and its choices are defined in Table 848(a). 0b00000: TNS = 2 with PSI=stream0 and ASMI = 0 or 1 or 2 0b00001: TNS = 2 with PSI=stream0 and ASMI = 3 or 4 or 5 0b00010: TNS = 2 with PSI=stream1 and ASMI = 0 or 1 or 2 0b00011: TNS = 2 with PSI=stream1 and ASMI = 3 or 4 or 5 0b00100: TNS = 2 with PSI=stream0 0b00101: TNS = 2 with PSI=stream1 0b00110: TNS = 3 with PSI=stream0 0b00111: TNS = 3 with PSI=stream1 0b01000: TNS = 3 with PSI=stream2 0b01001: TNS = 4 with PSI=stream0 0b01010: TNS = 4 with PSI=stream1 0b01011: TNS = 4 with PSI=stream2 0b01100: TNS = 4 with PSI=stream3 0b01101: TNS = 3 with PSI=stream0 and stream1 0b01110: TNS = 4 with PSI=stream0 and stream1 0b01111: TNS = 4 with PSI=stream2 and stream3 |
| Reserved | 1 | Reserved bit |
| } else { |  |  |
| Reserved | 5 | Reserved bit |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID | 2 | HARQ subpacket identifier for HARQ IR 0b00: 0 0b01: 1 0b10: 2 0b11: 3 |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Reserved | 1 | Reserved bits |
| } |  |  |

| ASMI | Modulation type of stream0, and stream1 |
| --- | --- |
| 0 | A, A |
| 1 | B, B |
| 2 | C, C |
| 3 | A, B |
| 4 | B, C |
| 5 | C, A |

Table 848(a) – ASMI (All Stream Modulation Index) for TNS = 2

________________________________End proposed text change _________________________________