| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the AAI_HO-CMD Messages (16.2.3.11) |  |
| Date Submitted | 2010-05-12 |  |
| Source(s) | Kelvin Chou MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D5) |  |
| Abstract | Clarification on the AAI_HO-CMD Message |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the AAI_HO-CMD Messages (16.2.3.11)

Kelvin Chou

MediaTek Inc.

Text Proposal 1

In P802.16m/D5, Section 16.2.3.11, Page 105, modify Table 692 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 692 — AAI_HO-CMD parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Mode | 2 | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. | NA |
| O | HO Reentry Mode | 1 | 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS (Mode=0b00); or the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier (Mode=0b01). 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS (Mode=0b00); or the AMS disconnects from the MZone before performing network reentry with the LZone (Mode=0b01). | Should Shall be included when Mode = 0b00 or 0b01. (Note: This should be changed to SHALL) Shall be included when Mode = 0b01. |
| O | HO Reentry Interleaving Interval | 8 | If HO Reentry Interleaving Interval > 0, the AMS performs network reentry to the target ABS within the HO Reentry Interleaving Interval and continues data transmission with the Serving ABS in the remaining time. If HO Reentry Interleaving Interval =0, the AMS performs multi-carrier EBB (Established Before Break) HO procedure per 16.2.8.2.9.2.2 | Shall be included when HO Reentry Mode is set to 1 |
| O | HO Reentry Iteration | 3 | The requested number of iterating HO Reentry Intervals by an AMS. | Shall be included when HO Reentry Interleaving Interval > 0 |
| O | Disconnect time offset | 8 | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset). | Should Shall be included when Mode = 0b00. |
| O | Resource_Retain_Time | 8 | The time when the serving BS discards MS’s context. | Should be included when Mode = 0b00. Shall be included when Mode = 0b00 or 0b01. |
| O | targetBSID | 48 | - | Shall be included for each target ABS when Mode=0b00 |
| O | Preamble Index | 10 |  | Shall be included for each target ABS when Mode=0b00 |
| O | FA index | 8 |  | Shall be included for each target ABS when Mode=0b00 |
| O | Action Time | 8 | (Mode=0b00) Action Time included in this message is the absolute frame number at the serving ABS (Mode=0b01) This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| O | CDMA_RNG_FLAG | 1 | If CDMA_RNG_FLAG=0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. This parameter shall be included for each target ABS when Mode=0b00.; It shall be included when Mode = 0b01. If CDMA_RNG_FLAG is set to 0, pre-assigned STID at the target BS shall be assigned. | Shall be included for each target ABS when Mode=0b00 |
| O | Dedicated Ranging Code Flag | 1 | If set to 0, it indicates that no dedicated ranging code is provided. If set to 1, it indicates that no dedicated ranging code is provided. | May be included for each target ABS when CDMA_RNG_FLAG=1 |
| O | Dedicated Ranging Opportunity Flag | 1 | If set to 0, it indicates that no dedicated ranging opportunity is provided. If set to 1, it indicates that dedicated ranging opportunity is provided. | May be included for each target ABS when CDMA_RNG_FLAG=1 |
| O | Dedicated CDMA ranging code | 5 | Indicates the dedicated ranging code. | Shall be included when Dedicated Ranging Code Flag =1 |
| O | Ranging opportunity index | 3 | Indicates the index of the allocated ranging opportunity. Ranging opportunity shall be located in the first UL subframe. | Shall be included when Dedicated Opportunity Code Flag=1 |
| O | HO process optimization | 4 | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines | Shall be included for each target ABS when Mode=0b00 When Bit #2 is set to 0 and IP address refresh is needed, the AAI_REG-REQ/RSP includes Host configuration capabilities and its parameters to provide IP address information. |
| O | Seamless HO | 1 | Indicates whether seamless HO is supported | Shall be included for each target ABS when Mode=0b00. If CDMA_RNG_FLAG is set to 1, and both Dedicated Ranging Code Flag and Dedicated Ranging Opportunity Flag is set to 0, Data packets shall be exchanged after receiving the AAI-RNG-REQ from the AMS. |
| O | Ranging initiation deadline | 8 | An AMS shall send the AAI_RNG-REQ message during HO until Ranging initiation deadline | Shall be included for each target ABS when Mode=0b00 |
| O | Pre-assigned STID | 12 | STID assigned to the AMS by the target ABS | Should be included for each target ABS when Mode = 0b00 |
| O | Service level prediction | 2 | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0 = No service possible for this AMS 1 = Some service is available for one or several service flows authorized for the AMS 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 3 = No service level prediction available. | Shall be included for each target ABS when Mode=0b00 |
| O | Physical_Carrier_Index | 8 | Physical carrier index of the recommended Target ABS | ShouldShall be included for each target ABS which is multi-carrier ABS when Mode = 0b00 and target ABS is not included in AAI_NBR-ADV message or is multi-carrier ABS |
| O | Channel bandwidth | 8 | Channel bandwidth of recommended target ABS in units of 125 KHz. | Shall be included for each target ABS when Mode=0b00 and target ABS is not included in AAI_NBR-ADV message |
| O | CP length | 2 | CP length of the recommended target ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included for each target ABS when Mode=0b00 and target ABS is not included in AAI_NBR-ADV message |
| O | Carrier Preassignment Indication | 1 | 1: information of pre-assigned secondary carriers by the target ABS is included in this message; 0: no pre-assigned secondary carrier | Shall be included for an AMS with multi-carrier capability when Mode = 0b00 |
| O | N_Preassigned_Carriers | 3 | Number of pre-assigned secondary carriers at the target ABS. A value of 0 means no secondary is pre-assigned. | Shall be included when Carrier Preassignment Indication = 1 |
| O | Pre-assigned secondary carrier information | TBD | Carrier information of each pre-assigned secondary carrier, which includes: Carrier Status Bitmap: indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done; Physical carrier index of pre-assigned secondary carriers; Logical carrier index, which is assigned implicitly in the order of Carrier information of each pre-assigned secondary carriers. | Shall be included when N_Preassigned_Carriers > 0 |
| O | LZone Preambile Index | 7 | LZone Preamble Index for AMS to switch from MZone to LZone | Shall be included when Mode = 0b01. |
| O | Pre allocated Basic CID | 16 | Used by the AMS to derive its own primary CID and transport CID in target YBS. | May be included when HO to a LZone or a YBS |
| O | SFH delta information | Variable | Delta encoding with reference to the serving ABS. | May be included for each target ABS when Mode = 0b00 and SFH delta information is not included in the AAI_NBR-ADV message for the target ABS |
| O | S-SFH change count | 4 | S-SFH change count of the reference for the included SFH delta information.. | Shall be included for each target ABS when SFH delta information is included |

------------------------------------------------------  Text End  ---------------------------------------------------------------