| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the AAI_RNG-ACK Messages (16.2.3.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Kelvin Chou MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D5) |  |
| Abstract | Clarification on the AAI_RNG-ACK Message |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the AAI_RNG-ACK Messages (16.2.3.3)

Kelvin Chou

MediaTek Inc.

Text Proposal 1

In P802.16m/D5, Section 16.2.3.3, Page 85, modify Table 682 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 682—AAI_RNG-ACK message Field Descriptions

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Unicast indication |  | 1 | 0 : for broadcast 1: for unicast | N/A |
| O | Frame identifier |  | 4 | Frame which contains the ranging opportunities to which this message refers. The frame identifier is produced by concatenating the following two values: 1.The 2 least significant bits of the superframe number 2.The frame index within the superframe (ranging from 0b00 to 0b11). |  |
| O | RNG-ACK Bitmap |  | 2Variable, | The size of this bitmap is decided by the number of ranging slots in the referenced frame, including the ranging slots allocated by the Broadcast Assignment A-MAP IE. Each bit indicates the decoding status of the corresponding ranging opportunity. 0b0: No ranging code is detected; 0b1: At least one code is detected. | It shall be included when Unicast indication =0 |
| O | Number of received codes |  | 6 | The number of ranging code indices detected in this corresponding ranging opportunity. | It shall be included when Unicast indication =0 |
| O | Ranging responses[0..32] | Code Index | 6 | Code index received in this ranging opportunity. | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 |
|  |  | Ranging Status | 2 | Used to indicate whether UL messages are received within acceptable limits by ABS. 0b01 = continue 0b10 = abort 0b11 = success | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 Or It shall be included for the specific user when Unicast indication =1 |
|  |  | Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 Or It shall be included for the specific user when Unicast indication =1. In this case, Bit #2 is always set to 0. |
|  |  | Timing offset adjustment | 10 | Time offset adjustment | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and and API bit #0 = 1 Or It shall be included for the specific user when Unicast indication =1 and API bit #0 = 1 |
|  |  | Power level adjustment | 3 | Power level adjustment | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and API bit #1 = 1 Or It shall be included for the specific user when Unicast indication =1 and API bit #1 = 1 |
|  |  | Frequency offset adjustment | 8 | Frequency offset adjustment | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and API bit #2 = 1 |

------------------------------------------------------  Text End  ---------------------------------------------------------------