| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up of MAC Signaling Header Definitions (16.2.2.1.3) |  |
| Date Submitted | 2009-04-29 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a on P802.16m/D5 |  |
| Abstract | The contribution proposes a clean-up of MAC Signaling Header definitions based on the accepted MAC signaling header format changes in Letter Ballot#31 (16.2.2.1.3). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up of MAC Signaling Header Definitions (16.2.2.1.3)

Lei Wang

InterDigital Communications, LLC

- Introduction

In Working Group Letter Ballot #31, some MAC signaling header format changes were accepted such that the MAC signaling header is now of the format as shown in Table 655. However, the existing MAC signaling headers have not been changed accordingly due to the time constraint in the 802.16 Session #66.

This contribution proposes a clean-up of the MAC signaling header definitions, based on the revised MAC signaling header format.

- Suggested changes in the 802.16m/D5

The following is the proposed change in the 802.16m/D5. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: page 48, line 35, Change Table 657 as follows:

Table 657—BR with STID Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| BR with STID () { |  |  |
| Flow ID | 4 | Flow identifier. Set to 0001. |
| Type | 4 | MAC signaling header type = 0b0000 |
| Length | 3 | Indicates the length of the signaling header in bytes. Set to 0b110. |
| Reserved | 1 | Reserved. This field shall be filled by 0 |
| BR Size | 19 | Aggregated bandwidth request size in bytes |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| STID | 12 | STID of the AMS which requests UL bandwidth. |
| Reserved | 5 1 | Reserved. This field shall be filled by 0 |
| } |  |  |

Suggested Remedy #2: line 1, page 49

Change Table 658 as follows:

Table 658—BR without STID Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| BR without STID header () { |  |  |
| Flow ID | 4 | Flow identifier. Set to 0001. |
| Type | 4 | MAC signaling header type = 0b0001 |
| Length | 3 | Indicates the length of the signaling header in bytes. Set to 0b101. |
| Reserved | 1 | Reserved. This field shall be filled by 0 |
| BR type | 1 | Indicates whether the requested bandwidth is incremental or aggregate: 0: aggregate 1: incremental |
| BR Size | 19 | Aggregated bandwidth request size in bytes |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| Reserved | 16 4 | Reserved. This field shall be filled by 0 |
| } |  |  |

Suggested Remedy #3: line 1, page 50

Change Table 659 as follows:

Table 659—Service Specific BR without STID Scheduling Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Service Specific Scheduling Control Header () { |  |  |
| Flow ID | 4 | Flow identifier. Set to 0001. |
| Type | 4 | MAC signaling header type = 0b0010 |
| Length | 3 | Indicates the length of the signaling header in bytes. Set to 0b110. |
| Reserved | 1 | Reserved. This field shall be filled by 0 |
| BR type | 1 | Indicates whether the requested bandwidth is incremental or aggregate: 0: aggregate 1: incremental |
| BR Size | 19 | Aggregated bandwidth request size in bytes |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| If (scheduling type == aGP Service) { |  |  |
| QoS parameter change indicator | 1 | QoS parameter change indicator 0: no change in QoS parameter 1: having changes in QoS parameter |
| SCID change indicator | 1 |  |
| If (QoS parameter change indicator ==1) { |  |  |
| Running Grant Polling Interval (GPI) | 6 | Indicating new GPI (frames) to use for future allocation. |
| } else { |  |  |
| QoS parameter set switch | 1 | 0: primary QoS parameter set 1: secondary QoS parameter set |
| } |  |  |
| If (SCID change indicator == 1) |  |  |
| { |  |  |
| SCID | 4 |  |
| } |  |  |
| } |  |  |
| Else if (scheduling type == BE) { |  |  |
| Minimum grant delay | 6 | Indicating minimum delay (frames) of the requested grant |
| } |  |  |
| Padding | variable | This field shall be filled by ‘0’ so that the total length of this header is equal to 6 bytes. |
| } |  |  |

Suggested Remedy #4: line 25, page 51

Change Table 660 as follows:

Table 660—Sleep Control Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Sleep Control Header Format { |  | — |
| FID | 4 | Flow Identifier. Set to 0001. |
| Type | 4 | MAC signaling header type = 0b0011 |
| Length | 3 | Indicates the length of the signaling header in bytes. Set to 0b011. |
| Reserved | 1 | Reserved. This field shall be filled by 0 |
| SCH sub-type | 3 | 0b000 = Listening Window control 0b001 = Resume Sleep Cycle Indication 0b010 = Sleep cycle configuration switch 0b011 = Sleep Cycle control 0b100 = Multi-Carrier Listening Window control 0b101~0b111 = reserved |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e., acknowledgement) to the request |
| If (SCH sub-type == Listening Window Control) { | — | — |
| Listening Window End or Extension | 1 | 0 = Listening Window End Indication 1 = Listening Window Extension Indication |
| if(Listening Window End or Extension==1){ | — | — |
| Last frame of Extended Listening Window | 7 | LSB of frame sequence. Indicate the frame that extended listening window is terminated. |
| } | — | — |
| } else if( SCH sub-type == Resume Sleep Cycle Indication) { | — | — |
| Scheduled Sleep Cycle Interruption included | 1 | 0 = no scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication 1 = scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication |
| if (Scheduled Sleep Cycle Interruption included == 1) { | — | — |
| Start Frame Offset for Scheduled Sleep Cycle Interruption | 7 | Number of frames in the future from the frame containing this SCH at which the scheduled Sleep Cycle interruption will occur. Frame offset is value of this field plus one (i.e., range is 1 to 128). |
| } | — | — |
| } | — | — |
| else if (SCH sub-type == Sleep cycle configuration switch){ | — | — |
| SCID | 4 | SCID corresponds to the new sleep cycle setting to be switched |
| Start Frame Offset for new sleep cycle configuration | 3 | Least Significant 3 bits of Frame Number in which sleep cycle setting is to be applied. |
| } | — | — |
| else if (SCH sub-type == Sleep Cycle control) { |  |  |
| Next Sleep Cycle Flag (NSCF) | 2 | 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved |
| if (NSCF == 0b10) |  |  |
| { |  |  |
| New Initial Sleep Cycle | 6 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. |
| } |  |  |
| } |  |  |
| else if (SCH sub-type == Multi- Carrier Listening Window control) { |  |  |
| Target Carrier Index Bitmap | 4 | If nth bit is set to 1, it indicates that DL data transmission on the secondary carrier of which logical carrier index is equal to (n+1) ends. |
| } |  |  |
| Padding | variable | For byte alignment |
| } | — | — |

Suggested Remedy #5: line 22, page 53

Change Table 661 as follows:

Table 661—AMS battery Report header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AMS battery Level Report header () { | - | - |
| Flow ID | 4 | Flow identifier. This field indicates MAC signaling header. Set to 0001. |
| Type | 4 | MAC signaling header type = 0b0100 |
| Length | 3 | Indicates the length of the signaling header in bytes. Set to 0b011. |
| Reserved | 1 | Reserved. This field shall be filled by 0 |
| AMS Batter Status | 1 | 0b0: The AMS is plugged into a power source 0b1: The AMS is not plugged into a power source |
| Battery Level Indication | 1 | 0b0: Detailed battery level report is not included. 0b1: Detailed battery level report is included. |
| If (Batter Level Indication =1) { |  |  |
| AMS Battery Level | 3 | 0b000: Battery level is > 75% and <= 100 % 0b001: Battery level is > 50% and <= 75 % 0b010: Battery level is > 25% and <= 50 % 0b011: Battery level is > 5 % and <= 25 % 0b100: Battery level is below 5 % 0b101 – 0b111: Reserved |
| Reserved | 35 7 | Shall be filled by 0 |
| } |  |  |
| else { |  |  |
| Reserved | 38 10 | Shall be filled by 0 |
| } |  |  |
| } |  |  |

Suggested Remedy #6: line 7, page 54

Change Table 662 as follows:

Table 661—Uplink Power Status Report Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Uplink Power Status Report Header () { |  |  |
| Flow ID | 4 | Flow identifier. Set to 0001. |
| Type | 4 | MAC signaling header type = 0b0101 |
| Length | 3 | Indicates the length of the signaling header in bytes. Set to 0b100. |
| Reserved | 1 | Reserved. This field shall be filled by 0 |
| ULPC Parameters Updating Indicator | 1 | Indicates whether the AMS has updated its ULPC parameters from AAI_SCD: 0: No ULPC parameters changed comparing to previous reporting. 1: The confirmation of new ULPC parameters from AAI_SCD applied. |
| txPowerPsdBase | 8 | txPowerPsdBase ( PSD(base) ) is coded using 8 bits in 0.5 dBm steps ranging from -74 dBm (coded 0x00) to 53.5 dBm (coded 0xFF). |
| txSirDownlink | 10 | txSirDownlink (SIRDL) is coded using 10 bits in 1/16 dB steps ranging from -12dB (coded 0x000) to 51.9375 dB (coded 0x3ff). |
| Reserved | 21 1 | Shall be filled by 0 |
| } |  |  |

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”