| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up AAI_Global-Config MAC control message (16.2.3.55) |  |
| Date Submitted | 2010-05-11 |  |
| Source(s) | Youngsoo Yuk, Eunjong Lee, Kiseon Ryu and Jin-Sam Kwak | youngsoo.yuk@lge.com |
| Re: | IEEE 802.16-10/0032, IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | This contribution propose text related on AAI_Global-Config MAC control message |  |
| Purpose | For TGm discussion and adoption into P802.16m/D6 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up AAI_Global-Config MAC control message (16.2.3.55)

Youngsoo Yuk, Eunjong Lee, Kiseon Ryu and Jin-Sam Kwak

LG Electronics

- Introduction

The proposed modifications in this contribution are as follows:

- The current MCI (Multicarrier Configuration Index) has limited flexibility even though it can reduce the overhead. We propose to alternative method to extend the possible multicarrier configuration.

- In addition, with the fixed sixe of subcarrier spacing (10.9375kHz) and frequency raster (200 or 250kHz), the possible values of the frequency shift for multicarrier alignment are limited to following 7 cases when the equations (178, 179) are applied.  {0, ±1.5625, ±3.125, ±4.6875}kHz.  In addition, for other OFDMA numerology, no shift is required.

- The physical carrier index of the current carrier is broadcast via AAI_MC-ADV message. Thus, the field is redundant to transmit via the AAI_Global-Config Message. (Actually, the field was required to omit center frequency information of the carrier group where the current carrier is included. However, the gain is not large enough (just 2 bits), thus that field is not required to be transmitted even for AAI_MC-ADV.)

- There is no place to inform which carrier is partially configured carrier. Partially configured carrier is defined only for FDD, we propose to include this information in this message in case of FDD carrier.

- Finally in order to maintain consistency in naming of MAC control message, I’d like to change the name of AAI_Global-Config into AAI_Global-CFG.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Modify the following text on Page #209, Line #17 in P802.16m/D5]

16.2.3.53 AAI_Global-Config (global carrier configuration) Message

The AAI_Global- Config message provides the carrier information for all available carriers in the network. The ABS transmits the AAI_Global- Config message to an AMS after transmitting the AAI_REG-RSP message.

Table 758 – AAI_Global- Config MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| MAC Control Message Type | 8 |  |
| Number of Carrier Groups | 4 | Groups of contiguous carriers In case of FDD, only DL carrier groups are included. |
| For (i=0; i< Number of Carrier Groups; i++){ |  |  |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Multi-Carrier Configuration Index Across the Network | 6 | Index associated to Table 770 803 |
| Start Frequency Assignment Index | 8 | Frequency Assignment Index of the first carrier in the i-th carrier group |
| Number of Carriers | 6 |  |
| For(j=0;j<Number of Carriers; j++) { |  |  |
| Physical Carrier Index | 6 | Index of the physical carrier |
| If(Duplexing_Mode==1){ |  |  |
| Carrier Type | 1 | Indicates the type of this physical carrier. 0b0: Fully configured carrier 0b1: Partially configured carrier |
| } |  |  |
| Reference Carrier Indicator | 1 | 0b0: Non-reference carrier 0b1: Reference carrier of current carrier group |
| } |  |  |
| } |  |  |
| Physical Carrier Index of current carrier | 6 | The carrier that broadcasts this message; the physical carrier index refers to AAI_Global-Config message |

--------------------------------------------------------End of the Text-----------------------------------------------------------