| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Trigger Action clarification for MC operation (16.8.2) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Xiangying Yang Etemad Kamran Intel | Xiangying.yang@intel.com |
| Re: | Commets on P802.16m/D5 for recirculation #31a |  |
| Abstract | The contribution provides the proposed text of SA-Preamble partitioning for ABS type to be included in IEEE 802.16m amendment document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Trigger Action clarification for MC operation (16.8.2)

Xiangying Yang, Etemad Kamran

Intel

- Introduction

Currently the trigger/Action defined in the handover section assumes single carrier mode operation. The notions of “serving BS” and “target BS” are to be clarified in the MC operation.

- References

[1] IEEE 802.16m-09/1902r3, “Proposed Text Proposal of SA-Preamble Partition in IEEE 802.16m/D1”

[2] IEEE 802.16m-07/0034r2, “802.16m System Requirements”

[3] IEEE 802.16m/D4, “Advanced Air Interface (Draft)”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

Modify P802.16m/D5 as following

-------------------------------------------------------  Text Start  --------------------------------------------------------------

[Note for Editor: Page 304, line 33]

16.2.8.2.9 Handover

The multicarrier handover (MCHO) is defined as the handover procedure which involves multiple radio carriers. An AMS with multicarrier capability may follow the single-carrier handover procedure per section 16.2.6. It may also decide to perform MCHO procedure as defined in this section.

For a MS operating in MC mode, it shall use its Primary Carrier as reference for serving ABS measurement and follow the trigger/action defined for its Primary Carrier obtained by AAI_SCD-ADV broadcast on its Primary Carrier for mobility related operations.

[Note for Editor: Page 311, line 12]

16.2.8.2.11 Carrier management

16.2.8.2.11.1 Secondary Carrier Activation/De-activation

The activation or deactivation of secondary carrier(s) is decided by the ABS based on QoS requirement, load condition of carriers, channel quality from CQI for activated carrier or scan report for deactivated carrier, and other factors. The trigger/action definition associated with the relevant carrier is obtained from AAI_SCD-ADV broadcast on the AMS’s Primary Carrier. The ABS activates and/or deactivates secondary carrier with the AAI_CM-CMD MAC control message. The ABS sends the AAI_CM-CMD MAC control message on the primary carrier and includes the following information:

[Note for Editor: Page 312, line 1]

16.2.8.2.11.2 Primary Carrier Change

The AMS may perform scanning on other assigned carriers which are not serving the AMS in an unsolicited manner or by the instruction of the ABS. The AMS reports the scanning results back to the serving ABS, which may be used by the ABS to determine the carrier for the AMS to switch to. The scanning operation shall follow the trigger/action defined for its Primary Carrier in AAI_SCD-ADV, while using the Primary Carrier as reference for “serving ABS” measurement and using the target carrier for scanning as reference for “neighbor ABS” measurement. In this case, if the target carrier is not currently serving the AMS, the AMS may perform synchronization with the target carrier if required.

-------------------------------------------------------  Text End  ----------------------------------------------------------------