| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to Channel quality measurements (16.3.14) |  |
| Date Submitted | 2010-05-12 |  |
| Source(s) | Huaning Niu, Yi Hsuan, Hujun Yin, Yang-seok Choi Intel Corp Daniel Cohn, Ariel Doubchak Alvarion Jeongho Park, Jiyun Seol Samsung | Huaning.niu@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D5 LB31a |  |
| Abstract | Modifications are proposed for channel quality measurements (16.3.14) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 5) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to Channel quality measurements (16.3.14)

Huaning Niu, Yi Hsuan, Hujun Yin, Yang-seok Choi

Intel Corp

Daniel Cohn and Ariel Doubchak

Alvarion

Jeongho Park, Jiyun Seol

Samsung

# Introduction

RSSI and CINR measurement in 802.16m is defined to be the same as 802.16-2009. This definition is not appropriate due to

- Link adaptation in 16m is based on CQI, which is the effective CINR derived based on different MIMO scheme. The physical CINR and RSSI is mainly used for FFR response, HO and scan

- The physical CINR and RSSI are measured using PA-preamble, SA-preamble, not on the pilots since pilots are precoded and power boosted.

- 16m support FFR which makes interference power calculation different from 16e.

- The SIRDL for OL UL power control is not defined in 802.16-2009

In this contribution, we propose the 16m physical CINR, RSSI and SIR definition. Also downlink NI definition is provided. How AMS reports these measurements is controlled by various MAC messages and is not discussed in this contribution ([2][3]).

- References

[1] Draft Amendment, P802.16m/D5, Apr. 2010.

[2] C80216m-10_0081.

[3] C80216m-10_0082.

# Text proposal for inclusion in the 802.16m amendment

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

-------------------------------------------------- Text Change ------------------------------------------------

- [ Recommended AWD Text Proposal (Section 16.3.14, page 766, line49) ]

16.3.14 Channel quality measurements

16.3.14.1 Introduction

RSSI and CINR measurements and associated statistics can help characterizing signal quality as well as assisting BS in selection/assignment of radio resources to the mobile stations according to their channel conditions. As channel behavior is time-variant, both mean and variance are required to be defined.

RSSI, CINR and SIR measurements and associated statistics may be used for characterizing signal quality, assisting ABS in selection/assignment of frequency partition, HO, scan and UL power control. As channel behavior is time variant, both mean and variance are required to be defined.

16.3.14.2 RSSI mean for handover and scan reports

The method for calculation of the RSSI and its mean and variance in IEEE 802.16m shall be the same as that described in 8.4.12.2 of IEEE Std 802.16-2009.

When a collection of RSSI measurements is mandated by the ABS, an AMS shall obtain an RSSI measurement from the DL SA-preamble. From a succession of RSSI measurements, the AMS shall derive and update estimates of the mean of the RSSI, and report them through a proper message (e.g., AAI_HO-REQ or AAI_SCN-REP message).

RSSI mean shall be reported in units of decibels relative to 1 mW , following the quantization requirement in each message.

The method used to estimate the RSSI of a single measurement is left to individual implementation.

The  mean RSSI statistics (in dBm), derived from a multiplicity of single measurement, shall be updated using Equation (x)

μRSSI[k]=R[0]dBk=0(1−αavg)n+1μRSSI[k−1]+(1−(1−αavg)n+1)R[k]dBk>0          dBm           (x)

where

k is the time index for the measurement; and n is the number of consecutive frames in which no measurement is made.

R[k]dB is the RSSI in dBm measured during message k, and αavgis an averaging parameter specified by the ABS per the specific usage case.

16.3.14.3 CINR mean and variance

The method for calculation of the CINR and its mean and variance in IEEE 802.16m shall be the same as that described in 8.4.12.3 of IEEE Std 802.16-2009.

When CINR measurements are mandated by the ABS, an AMS shall obtain a CINR measurement from the SA-preamble for HO, scan and AAI_FFR-REP. From a succession of these measurements, the AMS shall derive and update estimates of the mean and variance of the CINR, and report them through a proper message (e.g., AAI_FFR-REP, AAI_HO-REQ, AAI_SCN-REP message).

Mean and variance statistics for CINR shall be reported in units of dB, following the quantization requirement in each message.

The method used to estimate the CINR of a single measurement is left to individual implementation.

The estimated CINR may be formulated without loss of generality as follows.

CINRk=j=1NrCINR[k]j=j=1NrPsignalkjPinterference[k]j+Pnoise[k]j

Where CINR[k] is the CINR for measurement k,

CINR[k]j is CINR for measurement k at jth received antenna,

Psignal[k]j is the signal power for measurement k at jth received antenna excluding SA-preamble boosting,

Pinterference[k]jis the interference power for measurement k at jth received antenna excluding SA-preamble boosting.

Pnoise[k]j is the noise power for measurement k at jth received antenna.

Nr is the number of receive antennas at AMS

The estimation of the noise power that could be measured from the interference-free region is MS implementation specific.

The estimation of the interference power is described in the following equation.

Pinterference[k]j=PSeg0⋅Pinterference[k]j+PSeg1Pinterference[k]j+PSeg2Pinterference[k]j           (y)

where Tsis a set of measurement indices derived from the SA-preamble tone indices of segment s; Pinterference[k]jis the interference power level of segment s at jth received antenna calculated from SA-preamble measurement. PSegi is the linear power boosting or deboosting level for segment i. For example, in reuse 1 deployment, PSeg0=PSeg1=PSeg2=1. For segment 0 of reuse 3 deployment, PSeg0=1 and PSeg1=PSeg2=0. For the power boosting reuse 3 partition of segment 0 in FFR deployment, PSeg0=FP1Power, PSeg1=FP2Power, PSeg2=FP3Power where FP1_Power is the linear power boosting level and FP2_Power and FP3_Power are linear power deboosting levels.

The measured signal power for boosted reuse-3 FP shall be multiplied byPSeg0=FP1Power to obtain Psignal[k]j.

The mean CINR statistics (in decibels) shall be derived from a multiplicity of single measurement using Equation (xx).

μCINR[k]=CINR[0]dBk=0(1−αavg)n+1μCINR[k−1]+(1−(1−αavg)n+1)CINR[k]dBk>0                       (xx)

whereCINR[k]dB=10log10(CINR[k]) and αavgis an averaging parameter specified by the ABS as per the specific usage case; and n is the number of consecutive frames in which no measurement is made. To obtain the variance, the expectation-squared statistic shall be updated using Equation (xxx)

xCINR2[k]=|CINR[0]dB|2k=0(1−αavg)n+1xCINR2[k−1]+(1−(1−αavg)n+1)|CINR[k]dB|2k>0                     (xxx)

and

σCINR2[k]=xCINR2[k]−μCINR2[k]

16.3.14.3 SIR mean

When SIR measurements are mandated by the ABS, an AMS shall obtain a SIR measurement from the SA-preamble for UL OL power control. From a succession of these measurements, the AMS shall derive and update estimates of the SIR mean, and report them through Uplink Power Status Report Header.

Mean statistics for SIR shall be reported in units of dB, following the quantization requirement in Uplink Power Status Report Header.

The method used to estimate the SIR of a single measurement is left to individual implementation.

The estimated SIR may be formulated without loss of generality as follows.

SIRInstk=j=1NrSIR[k]j=j=1NrPsignalkjPinterference[k]j

Where SIR[k] is the SIR for measurement k,

SIR[k]j is SIR for measurement k at jth received antenna,

Psignal[k]j is the signal power for measurement k at jth received antenna excluding SA-preamble boosting,

Pinterference[k]jis the interference power for measurement k at jth received antenna excluding SA-preamble boosting as described in equation (y).

Nr is the number of receive antennas at AMS

The measured signal power for boosted reuse-3 FP shall be multiplied byPSeg0to obtainPsignal[k]j.

The mean SIR statistics (in decibels) shall be derived from a multiplicity of single measurement using Equation (xxxx).

SIRDL[k]dB=SIRInst[0]dBk=0(1−αavg)n+1SIRDL[k−1]dB+(1−(1−αavg)n+1)SIRInst[k]dBk>0                   (xxxx)

whereSIRInst[k]dB=10log10(SIRInst[k]) and αavgis an averaging parameter specified by the ABS as per the specific usage case; and n is the number of consecutive frames in which no measurement is made.

16.3.14.4 Downlink Noise and Interference Level Mean and Variance

For downlink noise and interference (DL NI) level report through FFR-REP which responds to FFR-CMD instructed by ABS, an AMS shall obtain DL NI measurement from the SA-preamble for downlink FFR operation. From a succession of these measurements, the AMS shall derive and update estimates of the DL NI mean and variance, and report them through FFR-REP message.

The method used to estimate the DL NI of a single measurement is left to individual implementation.

The estimated DL NI may be formulated without loss of generality as follows.

NIk=1Nrj=1NrNI[k]j=1Nrj=1NrPinterference[k]j+Pnoise[k]j

Where NI[k] is the NI for measurement k,

NI[k]j is NI for measurement k at jth received antenna,

Pinterference[k]jis the interference power for measurement k at jth received antenna excluding SA-preamble boosting as described in equation (y).

Pnoise[k]j is the noise power for measurement k at jth received antenna.

Nr is the number of receive antennas at AMS

The estimation of the noise power that could be measured from the interference-free region is MS implementation specific.

The mean NI statistics (in decibels) shall be derived from a multiplicity of single measurement using Equation (zz).

μNI[k]=NI[0]dBk=0(1−αavg)n+1μNI[k−1]+(1−(1−αavg)n+1)NI[k]dBk>0                       (zz)

whereNI[k]dB=10log10(NI[k]) and αavgis an averaging parameter specified by the ABS as per the specific usage case; and n is the number of consecutive frames in which no measurement is made. To obtain the variance, the expectation-squared statistic shall be updated using Equation (zzz)

xNI2[k]=|NI[0]dB|2k=0(1−αavg)n+1xNI2[k−1]+(1−(1−αavg)n+1)|NI[k]dB|2k>0                     (zzz)

and

σNI2[k]=xNI2[k]−μNI2[k]