| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Correction on MAEH format (P802.16m/D5-16.2.2.2.5) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Lei Zhang Yang Liu Jerry Chow ZTE Corporation | zhang.lei13@zte.com.cn |
| Re: | Category: LB31a / Area: Chapter 16.2.2.2.5 (MAC Control ACK Extended Header) |  |
| Abstract | This contribution proposes changes to the Section 16.2.2.2.5 of IEEE P802.16m/D5 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Correction on MAEH format (P802.16m/D5-16.2.2.2.5)

Lei Zhang, Yang Liu, Jerry Chow

ZTE Corporation

- Introduction

There are some errors found in MAEH format, this proposal is used to correct all the errors in the format of MAEH.

- Proposed text

======================== Start of Proposed Text ========================

[Modify Table 670 of P802.16m/D5 as follows]

- 16.2.2.2.5 MAC Control ACK Extended Header (MAEH)

Table 670—MAEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MAEH() { |  |  |
| Type | 4 | MAEH Type |
| MAEH sub-type | 1 | Bit #0: Indicates reception of a control message Bit #1: Indicates reception of a BR without STID Service Specific Scheduling Control header |
| If (Bit #0 of MAEH sub-type == 10) { |  |  |
| Control Connection Channel ID (CCC ID) | 1 | Control Connection Channel ID (CCC ID) that the MAC control message is received. |
| ACK_SN | 8 | SN retrieved from the MCEH of the MAC PDU with the Polling bit set to 1 |
| } |  |  |
| Reserved | Variable | For byte alignment |
| } |  |  |

======================== End of Proposed Text ========================

- References

[1] IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, April 2010.