| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Correction on MAEH format (P802.16m/D5-16.2.2.2.5) |  |
| Date Submitted | 2010-05-05 |  |
| Source(s) | Lei Zhang Yang Liu Jerry Chow Hongyun Qu Nan Li Kaiying Lv ZTE Corporation | zhang.lei13@zte.com.cn |
| Re: | Category: LB31a / Area: Chapter 16.2.2.2.5 (MAC Control ACK Extended Header) |  |
| Abstract | This contribution proposes changes to the Section 16.2.2.2.5 of IEEE P802.16m/D5 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Correction on MAEH format (P802.16m/D5-16.2.2.2.5)

Lei Zhang, Yang Liu, Jerry Chow, Hongyun Qu, Nan Li, Kaiying Lv

ZTE Corporation

- Introduction

In D5, some function of signaling header and extended header seems duplicated (e.g. SCH, SCEH and MAEH could be used to carry short control signaling). Actually, based on previous discussion, people prefer to use extended headers only for the control info that is relevant to the MAC PDU construction and payload decoding. For this reason, this contribution proposed to change MAEH as signaling header.

- Proposed text

======================== Start of Proposed Text ========================

Suggested Remedy #1:

[Modify Table 656 of P802.16m/D5 as follows]

Table 656—Type field encodings for MAC signaling header type

| Type field (4 bits) | MAC Signaling Header Type |
| --- | --- |
| 0000 | BR with STID |
| 0001 | BR without STID |
| 0010 | Service Specific Scheduling Control Header |
| 0011 | Sleep Control |
| 0100 | AMS Battery Level Report |
| 0101 | Uplink Power Status Report |
| 0110 | MAC Control ACK |
| 0111-1111 | Reserved |

[Modify section 16.2.2.2.5 (from line 12 on page 61) of P802.16m/D5 as follows]

- 16.2.2.2.5 16.2.2.1.3.7 MAC Control ACK Extended Header (MCAEH)

The MCAEH format is defined in Table xxx. This header may be used by ABS and AMS to indicate the reception of a specific, previously received MAC control message, or Service Specific Scheduling Control BR without STID header (see 16.2.2.1.3.2). When an ABS or AMS receives a MAC control message or MAC control message fragment with the Polling bit set to 1 in the MCEH, the ABS or AMS shall transmit an MCAEH or AAI_MSG-ACK message as an acknowledgement after receiving the complete message with the SN of the MAC control message PDU or the SN of the last received fragment if fragmented. The ABS shall pre-allocate the resource for transmission of this MCAH or AAI_MSG-ACK message without AMS's request when the polling bit in the MCEH of correponding message is set to 1.

Table xxx—MCAEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MAEH MAC control ACK header format() { |  |  |
| FlowID | 4 | Flow Identifier. Set to 0001 |
| Type | 4 | MAEH MAC signaling header Type = 0110 |
| Length | 3 | Indicates the length of the signaling header in bytes. Set to 0b011. |
| MCAEH sub-type | 1 | Bit #0: Indicates reception of a control message Bit #1: Indicates reception of a BR without STID Service Specific Scheduling Control header |
| If (Bit #0 of MCAEH sub-type == 10) { |  |  |
| Control Connection Channel ID (CCC ID) | 1 | Control Connection Channel ID (CCC ID) that the MAC control message is received. |
| ACK_SN | 8 | SN retrieved from the MCEH of the MAC PDU with the Polling bit set to 1 |
| } |  |  |
| Reserved | Variable | For byte alignment |
| } |  |  |

Suggested Remedy #2:

Change all ‘MAEH’ in P802.16m/D5 to ‘MCAH’

======================== End of Proposed Text ========================

- References

[1] IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, April 2010.