| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handling of on going DSx Transactions Interrupted by HO (Section 16.2.6.3.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Stavros Tzavidas Motorola | stavros.tzavidas@motorola.com |
| Re: | P802.16m, Letter Ballot 31a |  |
| Abstract | The concept of network configuration and of Legacy ASN-GW support was introduced in the last meeting and some functionality was made dependent on the “Network Configuration” bit in S-SFH. However the impact on idle mode was not adequately addressed. |  |
| Purpose | Adoption of proposed text by P802.16m TG. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Handling of on going DSx Transactions Interrupted by HO (Section 16.2.6.3.3)

Stavros Tzavidas

Motorola

# Introduction

Contribution 802.16m-10/264r3 was adopted in the last meeting and it introduced the concept of a change count for each flow. This concept is used when DSx transactions are interrupted by a HO, at which point the AMS goes to the T-ABS and reports the status of each flow by reporting the associated change count in the RNG-REQ message during network entry.

However still issues remain with the implementation and with how it deals with the actual problem.

- The description of “FID change count” is unclear. FID Change count is only described as part of DSx message description (e.g. Table 740) and it specifies that the FID Change count is assigned by the sender and if a new transaction is started then the change count is incremented by one. Pages later in the description of AAI_RNG-REQ (section 16.2.3.1) in Table 680 we are told that the AMS shall include a list of FIDs which the AMS currently has, and Change Count of each FID. What is probably implied here is that each side maintains one count per flow but the current explanation could easily mean that there is one global change count that is incremented every time a new transaction is started (for any flow). The FID change count needs to be clearly defined.

- It is not clear how the reporting of the change count can inform the target ABS about the status of the unfinished transaction. When there is a mismatch between the change count that the AMS reports for an FID and the value that the T-ABS has available internally, then we know that some transactions were totally missed, but if the change count match we only know that both sides have seen the same number of transactions start. We don’t know if they are completed. The problem exists when one side believes a transaction is complete while the other things it is still on-going.

- When missmatch is detected, the prescribed action is uncecessarily severe (i.e. delete!). The point is to figure out which transaction is missing and to repeat it, not to delete the whole flow!

- This solution may not work in a situation where the backbone network is still in legacy (16e) mode. The solution requires passing more info per flow (i.e. the change count) from the S-ABS to the T-ABS. A legacy ASN-GW has no notion of "change count", i.e. this is not part of the context we can transfer from the S-ABS to the T-ABS. In fact the ASN-GW may need to be aware of such a concept because it IS affected by DSx transactions.

For all the above reasons we propose that it is better to terminate all DSx transactions at the serving abs and never involve the T-ABS. This can be done in the HO-CMD message, and has the advantage that it prevents possible missmatches instead of simply detecting them and also that it does not involve the ASN-GW in any way.

# Proposed Solution

We can adopt the following simple rules:

- The S-ABS shall not initiate any DSx transactions after it decides to initiate a HO and send AAI_HO-CMD to the AMS.

- The AMS shall not initiate any DSx transactions after it receives AAI_HO-CMD from the S-ABS

- Both sides should ignore any DSx messages that are received after the AAI_HO-CMD is transmitted (at the ABS side) or received (at the AMS side).

- The AAI_HO-CMD message shall be used to convey final information and complete/terminate any pending DSx transactions.

This can be accomplished by simply including in AAI_HO-CMD the same information that is normally

included in DSx-RSP and/or DSx-ACK, i.e the pair (Transaction identifying information, Confirmation Code) that indicates the status of the pending transaction.

In the following the term “Transaction ID” refers to some information that can be used to identify a particular transaction. For example one possibility is Transaction ID = (FID, Change Count).

More specifically, depending on which side initiated the transaction and which messages have been received by the ABS at the time it transmits AAI_HO-CMD, we can distinguish the following cases:

| DSx transaction initiated by the ABS (ABS sent DSx-REQ) |  |
| --- | --- |
| If… | The DSx transaction shall be considered… |
| DSx-REQ sent but DSx-RSP not received | Failed The AAI_HO-CMD shall contain (Transaction ID, Confirmation Code) for this transaction with Confirmation Code set to 1. |
| DSx-RSP received | Complete. The AAI_HO-CMD shall contain (Transaction ID, Confirmation Code) for this transaction. This information may be repeated if the ACK has already been transmitted. |
| DSx transactions initiated by the AMS (AMS sent DSx-REQ) |  |
| If… | The DSx transaction shall be considered… |
| DSx-REQ not received by the ABS | Failed. The ABS is obviously unaware of this transaction and does not include the transaction ID in the AAI_HO-CMD. |
| DSx-REQ received but DSx-RSP not yet transmitted. | Complete. The AAI_HO-CMD shall contain (Transaction ID, Confirmation Code) that would normally be included in DSx-RSP. DSx-ACK is not required. |
| DSx-RSP transmitted but DSx-ACK not received yet. | Complete. The AAI_HO-CMD shall contain (Transaction ID, Confirmation Code) that was included in DSx-RSP. DSx-ACK is not required. |

# Proposed Text

[Modify the text in section 16.2.6.3.3 “HO Preparation” on page 276, line 31 as indicated:]

16.2.6.3.3 HO Preparation

During HO preparation phase, the serving ABS communicates with target ABS(s) selected for HO. The AAI_HO-CMD message should not include an ABS with cell bar bit =1. A Femto ABS shall follow the handover procedure described in section 16.4.8. The target ABS may obtain AMS information from the serving ABS via backbone network for HO optimization.

[text ommited]

The AAI_HO-CMD message indicates if the static and/or dynamic context and its components of the AMS are available at the target ABS.

All on-going DSx transaction during HO shall be handled as specified in section 16.2.6.3.3.1.  cancelled, and shall be re-started after HO completion. After an ABS receives the AAI_HO-REQ message from an AMS, the ABS shall not send any DSx message to the AMS until HO completion. After an ABS sends the AAI_HO-CMD message to an AMS, the ABS shall not send any DSx message to the AMS until HO completion.

[Insert the following new sub-clause in section 16.2.6.3.3 “HO Preparation” on page 276:]

16.2.6.3.3.1 Handling of DSx transactions during HO

The serving ABS shall not initiate any DSx transactions after sending the AAI_HO-CMD message. The AMS shall not initiate any DSx transactions after receiving the AAI_HO-CMD message. The ABS shall ignore any DSx messages received after sending the AAI_HO-CMD message. The AMS shall ignore any DSx messages received after receiving the AAI_HO-CMD message.

The Serving ABS shall include in AAI_HO-CMD a set of (FID, Change Count, Confirmation Code) for each pending DSx transaction. The Confirmation Code shall be set according to the rules in table XXX.

The Serving ABS may also include in AAI_HO-CMD a set of (FID, Change Count, Confirmation Code) for DSx transactions that were recently completed, to ensure reception synchronization with the AMS.

Table XXX – Rules for setting Confirmation Code value in AAI_HO-CMD

| DSx transaction initiated by the ABS (ABS sent DSx-REQ) |  |
| --- | --- |
| If… | The DSx transaction shall be considered… |
| DSx-REQ sent but DSx-RSP not received | Failed The AAI_HO-CMD shall contain (FID, Change Count, Confirmation Code) for this transaction with Confirmation Code set to 1. |
| DSx-RSP received | Complete. The AAI_HO-CMD shall contain (FID, Change Count, Confirmation Code) for this transaction. This information may be repeated if the ACK has already been transmitted. |
| DSx transactions initiated by the AMS (AMS sent DSx-REQ) |  |
| If… | The DSx transaction shall be considered… |
| DSx-REQ not received by the ABS | Failed. The ABS is unaware of this transaction and does not include the transaction ID in the AAI_HO-CMD. |
| DSx-REQ received but DSx-RSP not yet transmitted. | Complete. The AAI_HO-CMD shall contain (FID, Change Count, Confirmation Code) with Confirmation Code set to the value that would normally be included in DSx-RSP. DSx-ACK is not required. |
| DSx-RSP transmitted but DSx-ACK not received yet. | Complete. The AAI_HO-CMD shall contain (FID, Change Count, Confirmation Code) with Confirmation Code set to the value that was included in DSx-RSP. DSx-ACK is not required. |

Upon receiving the AAI_HO-CMD message the AMS shall decide on the status of pending DSx transactions according to the following rules:

- If the FID and Change Count of a pending DSx transaction is not included in AAI_HO-CMD, the transaction shall be considered as failed.

- The AMS shall ignore any information contained in AAI_HO-CMD for transactions that have been completed (DSx-ACK received prior to receiving AAI_HO-CMD).

- If the FID and Change Count of a pending DSx transaction is included in the AAI_HO-CMD, the Confirmation Code included in AAI_HO-CMD is considered final and no further control messages shall be required.

[Modify the text in Table 680-“AAI_RNG-REQ message Field Descriptions” on page 79 as indicated:]

Table 680—AAI_RNG-REQ message Field Descriptions

| M/O | Attributes / Array of Attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | CSGID(s)[TBD] |  | TBD | One or more identifiers for one or more CSG(s) | Sent by AMS to aid quick CSG membership detection by Femto ABS as well as to aid ABS reselection |
| O | CMAC Tuple | PMK SN | 4 | The sequence number of PMK used to derive the current AK. | It shall be included when the AMS is attempting to perform Network Reentry, Secure Location Update, or HO, or a reentry if the AMS has a CMAC tuple necessary to expedite security authentication. |
|  |  | CMAC_PN | 24 | Current CMAC_PN_U for uplink or current CMAC_PN_D for downlink. |  |
|  |  | CMAC value | 64 | The CMAC value derived from the whole message. |  |
| O | List of FIDs, and Change Count of each FID |  |  | List of FIDs which the AMS currently has, and Change Count of each FID perEachFID SEQUENCE { fid INTEGER (0 .. 15), fidchangecount INTEGER (0 .. 15) } | Shall be included if Ranging Purpose Indication value is set to 0b000 and Paging Controller ID is absent, |

[Insert new text in Table 692-“AAI_HO-CMD parameters” on page 109 as indicated:]

Table 692—AAI_HO-CMD parameters

| M/O | Attributes / Array of Attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Pre-assigned secondary carrier information | TBD | Carrier information of each pre-assigned secondary carrier, which includes: Carrier Status Bitmap: indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done; Physical carrier index of pre-assigned secondary carriers; Logical carrier index, which is assigned implicitly in the order of Carrier information of each pre-assigned secondary carriers. | Shall be included when N_Preassigned_Carriers > 0 |
| O | LZone Preambile Index | 7 | LZone Preamble Index for AMS to switch from MZone to LZone | Shall be included when Mode = 0b01. |
| O | Pre allocated Basic CID | 16 | Used by the AMS to derive its own primary CID and transport CID in target legacy BS. | May be included when HO to a LZone or a legacy BS |
| O | SFH delta information | Variable | Delta encoding with reference to the serving ABS. | May be included for each target ABS when Mode = 0b00 and SFH delta information is not included in the AAI_NBR-ADV message for the target ABS |
| O | S-SFH change count | 4 | S-SFH change count of the reference for the included SFH delta information.. | Shall be included for each target ABS when SFH delta information is included |
| O | Pending DSx transaction status reports |  | List of (FID, Change Count, Confirmation Code) sets for each FID that has an on-going DSx transaction perDSxStatus SEQUENCE { fid INTEGER (0 .. 15), FidChangeCount INTEGER (0..15) Confirmation Code ENUMERATED {SUCCESS(0), ERROR(1)} } |  |