| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On the Structuring and Minimization of Capability Negotiation Parameters during Network Entry/Re-entry Sections: 16.2.15, 16.2.3 |  |
| Date Submitted | 2010-05-11 |  |
| Source(s) | Shaocheng Wang Sassan Ahmadi Muthaiah Venkatachalam Xiangying Yang Intel Corporation | E-mail: shaocheng.wang@intel.com; sassan.ahmadi@intel.com; muthaiah.venkatachalam@intel.com; xiangying.yang@intel.com |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “sections 16.2.15 and 16.2.3” |  |
| Abstract | This contribution proposes a new structure for capability negotiation parameters during network entry/re-entry to minimize the use of radio resources for transmission of unnecessary parameters that otherwise must be supported by default. We further categorize the parameters to be negotiated based on pre-authentication or post-authentication to ensure encryption and protection of those parameters that are directly pertained to the user or device identity and credentials. |  |
| Purpose | Adopt the proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

On the Structuring and Minimization of Capability Negotiation Parameters during Network Entry/Re-entry

Shaocheng Wang, Sassan Ahmadi, Muthaiah Venkatachalam, Xiangying Yang

Intel Corporation

- Introduction

This contribution proposes a new structure for capability negotiation parameters (i.e., AAI_SBC-REQ/RSP and AAI_REG-REQ/RSP) during network entry/re-entry to minimize the use of radio resources for transmission of unnecessary parameters that otherwise must be supported by default. We further propose categorization of the parameters to be negotiated based on pre-authentication or post-authentication to ensure encryption and protection of those parameters that are directly pertained to the user or device identity and credentials. The current parameters specified in Tables 684 and 686 of P802.16m/D5 are based on the reference system capability negotiation scheme where no default capabilities were defined such that MS and BS had to negotiate the very basic parameters in order to begin the normal operation following network entry/re-entry.

Unlike the legacy standard, IEEE 802.16m specifies a set of default configuration parameters known as Capability Class 0, where both MS and BS are required to support in order to perform normal operations in IEEE 802.16m networks and achieve the minimum performance. The features specified by the standard as mandatory and the default configuration parameters are not negotiable and must be excluded from capability negotiation. It must be noted that MAC control messages that are used for capability negotiations do consume airlink radio resource; therefore, unnecessary parameter exchange would degrade the overall system performance.

The remaining parameters including optional features and features for which different parameter values exist can be negotiated. Among those parameters, some have direct relationship with the user identity or credentials and must be protected by encryption. Those general parameters that are not related to the user identity and therefore are not confidential can be grouped into one bucket and can be negotiated via AAI_SBC-REQ/RSP messages. Other parameters that require user authentication and encryption shall be included in a different bucket and can be negotiated using AAI_REG-REQ/RSP. This would avoid unnecessary encryption of general parameters and can be used as a rationale for future expansion of the parameter sets.

This contribution proposes a structuring method for classifying the configuration parameters according to the following criteria:

      - Post-authentication parameter set: Those are parameters that are related to user identity and require privacy protection shall be negotiated during Registration process (AAI_REG-REQ/AAI_REG-RSP).

      - Pre-authentication parameter set: Those parameters that are irrelevant to user identity and do not require privacy protection shall be negotiated in Basic Capability Negotiation (AAI_SBC-REQ/ AAI_SBC-RSP).

      - Remove unnecessary negotiations if such information is available in DL PHY synchronization (i.e. PA-Preamble and SA-Preamble) or by decoding P-SFH/S-SFH and the parameters are not negotiable.

- Problem discussion

The following table is provides a summary of the existing capability parameters in P802.16m/D5 draft, and our suggestions in this proposal.

| Parameter | Status in P802.16m/D5 | Our Proposal |  |  |
| --- | --- | --- | --- | --- |
| UL sounding | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| AMS DL processing capability per sub-frame | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| AMS UL processing capability per sub-frame | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| Authorization policy support | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| Modulation scheme (DL/UL) | AAI_SBC-REQ/RSP | Remove from NE |  |  |
| Max. Number of streams for SU-MIMO in DL MIMO | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| DL HARQ buffering capability | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| UL HARQ buffering capability | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| DL MIMO mode | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| UL MIMO mode | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| DL FFR | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| Long TTI for UL | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| Long TTI for DL | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| Inter-RAT Operation Mode | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| Supported Inter-RAT type | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| MIH Capability Supported | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| feedback support for DL | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| Number of Tx antenna of MS | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| FFT size | AAI_SBC-REQ/RSP | AAI_SBC-REQ/RSP |  |  |
| AMS MAC address | AAI_REG-REQ | AAI_REG-REQ |  |  |
| Host-Configuration-Capability-Indicator IE | AAI_REG-REQ/RSP | AAI_REG-REQ/RSP |  |  |
| Requested-Host-Configurations IE | AAI_REG-REQ/RSP | AAI_REG-REQ/RSP |  |  |
| Supported-IP-Versions IE | AAI_REG-REQ/RSP | AAI_REG-REQ/RSP |  |  |
| Classification/PHS options and SDU encapsulation support | AAI_REG-REQ/RSP | AAI_REG-REQ/RSP |  |  |
| Maximum number of classification rules | AAI_REG-REQ/RSP | AAI_REG-REQ/RSP |  |  |
| PHS support | AAI_REG-REQ/RSP | AAI_REG-REQ/RSP |  |  |
| ROHC support | AAI_REG-REQ/RSP | AAI_REG-REQ/RSP |  |  |
| IPv4-Host-Address IE | AAI_REG-REQ/RSP | AAI_REG-REQ/RSP |  |  |
| IPv6-Home-Network-Prefix IE | AAI_REG-REQ/RSP | AAI_REG-REQ/RSP |  |  |
| Additional-Host-Configurations IE | AAI_REG-REQ/RSP | AAI_REG-REQ/RSP |  |  |
| ARQ_WINDOW_SIZE | AAI_REG-REQ/RSP | move to AAI_SBC-REQ |  |  |
| ARQ_BLOCK_LIFETIME | AAI_REG-REQ/RSP | move to AAI_SBC-REQ |  |  |
| ARQ_RX_PURGE_TIMEOUT | AAI_REG-REQ/RSP | move to AAI_SBC-REQ |  |  |
| ARQ_SN_MODULUS | AAI_REG-REQ/RSP | move to AAI_SBC-REQ |  |  |
| ARQ_SUB_BLOCK_SIZE | AAI_REG-REQ/RSP | move to AAI_SBC-REQ |  |  |
| MAXIMUM_ ARQ_BUFFER_SIZE | AAI_REG-REQ/RSP | move to AAI_SBC-REQ |  |  |
| Global carrier configuration change count | AAI_REG-REQ/RSP | AAI_REG-REQ/RSP |  |  |
| STID | AAI_REG-RSP | AAI_REG-RSP |  |  |
| CRID | AAI_REG-RSP | AAI_REG-RSP |  |  |
| Resource_Retain_Time | AAI_REG-RSP | Remove from NE |  |  |
| Redirection Info | AAI_REG-RSP | AAI_REG-RSP |  |  |
| Femto ABS LDM parameters: Start_frame_number | AAI_REG-RSP | AAI_REG-RSP |  |  |
| Femto ABS LDM parameters: Available_interval_length | AAI_REG-RSP | AAI_REG-RSP |  |  |
| Femto ABS LDM parameters: Unavailable_interval_length | AAI_REG-RSP | AAI_REG-RSP |  |  |
| Multicarrier capabilities | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| Capabilities for Zone switching mode | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| LBS Capabilities | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| DL PMI coordination capability | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| DL collaborative multi-BS MIMO capability | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| DL closed-loop multi-BS macro diversity capability | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| UL PMI combination capability | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| E-MBS capabilities | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| Persistent Allocation support | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| Group Resource Allocation support | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| Capabilities for Handover EBB mode | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| Co-located coexistence capability support | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| HO trigger metric | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| MAX Tx Power | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| Channel BW and Cyclic prefix | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| frame configuration to support legacy(5MHz) | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| frame configuration to support legacy(10MHz) | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| OFDMA system support | AAI_REG-REQ | move to AAI_SBC-REQ |  |  |
| 3-step BR | AAI_REG-REQ | Remove from NE |  |  |
| dynamic ranging for HO | AAI_REG-REQ | Remove from NE |  |  |
| AMS Scanning Capability | AAI_REG-REQ | Remove from NE |  |  |

- Proposed Changes

--------------------------------Start of the proposed text-----------------------------------------

In Figure 440 of section 16.2.15 page 362 and in the third step replace “Obtain P-SFH/S-SFH SP1” with “Obtain P-SFH/S-SFH SP1 and SP2”

--------------------------------End of the proposed text-----------------------------------------

--------------------------------Start of the proposed text-----------------------------------------

In section 16.2.3.4 under AAI_SBC-REQ replace Table 684 with following table (entries that are crossed out are existing parameters that we suggest to remove)

Table 684—AMS Capabilities to be transmitted to ABS

| Nego parameters |  | value |
| --- | --- | --- |
| Modulation Scheme |  | If Bit#0 =1, DL QPSK supports If Bit#1 =1, DL 16QAM supports If Bit#2 =1, DL 64 QAM supports If Bit#3 =1, UL QPSK supports If Bit#4 =1, UL 16QAM supports If Bit#5=1, UL 64 QAM supports |
| UL HARQ buffering capability |  | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. |
| DL HARQ buffering capability |  | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. |
| AMS DL processing capability per sub-frame |  | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| AMS UL processing capability per sub-frame |  | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| FFT size(2048/1024/512) |  | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports |
| Authorization policy support |  | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported |
| Inter-RAT Operation Mode |  | 0b00: single radio mode operation for inter RAT handover 0b01: multi radio mode operation for inter RAT handover 0b10 - 0b11 : reserved |
| Supported Inter-RAT type |  | 1 indicates support, 0 indicates not support: bit #0: 802.11 bit #1: GERAN(GSM/GPRS/EGPRS) bit #2: UTRAN bit #3: E-UTRAN bit #4: CDMA 2000 bit #5-7: Reserved, set to zero |
| MIH Capability Supported |  | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. |
| ARQ parameters | ARQ_SN_MODULUS | the number of unique SN values |
|  | ARQ_WINDOW_SIZE | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |
|  | ARQ_SUB_BLOCK_SIZE | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  | ARQ_BLOCK_LIFETIME | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  | ARQ_RX_PURGE_TIMEOUT | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  | MAXIMUM_ ARQ_BUFFER_SIZE | The maximum ARQ buffer size Depend on AMS's memory |
| Multicarrier capabilities |  | 0b00: No MC modes 0b01: Basic MC mode 0b10: Multicarrier Aggregation 0b11: Multicarrier Switching |
| Zone Switch Mode Support |  | Indicates whether the AMS has a capability to maintain its data communication with the ABS in LZone while performing network reentry in MZone 0b0: Not supported 0b1: Supported |
| Capability for supporting A-GPS method |  | 0b0: no support 0b1: support |
| Capabilities for interference mitigation support | DL PMI coordination capability | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable |
|  | DL collaborative multi-BS MIMO capability | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is DL collaborative multi-BS MIMO capable |
|  | DL closed-loop multi-BS macro diversity capability | 0: AMS is not DL closed-loop multi-BS macro diversity capable 1: AMS is DL closed-loop multi-BS macro diversity capable |
|  | UL PMI combination capability | 0: AMS is not UL PMI combination capable 1: AMS is UL PMI combination capable |
| E-MBS capabilities |  | If Bit#0 is set to 1, it indicates E-MBS in Serving ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |
| Channel BW and Cyclic prefix |  | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) |
| frame configuration to support legacy(5MHz) |  | If Bit#0 =1,it supports(TDD only) |
| frame configuration to support legacy(10MHz) |  | If Bit#0 =1,it supports(TDD only) |
| OFDMA system support |  | If Bit#0 =1,it supports |
| MAX Tx Power |  | Bit 0-7: Maximum transmitted power for QPSK. Bit 8-15: Maximum transmitted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. |
| dynamic ranging for HO |  | If Bit#0 =1,it supports |
| Persistent Allocation support |  | If Bit#0 =1,it supports |
| Group Resource Allocation support |  | If Bit#0 =1,it supports |
| Co-located coexistence capability support |  | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class |
| HO trigger metric |  | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports |
| Buffering support during seamless HO |  | 0b0: no support. Traffic associated with SAID 0x00 (null SA) and 0x02 (AES-CTR) shall not be exchanged in either DL or UL before ranging transaction completion. 0b1: support. |
| Capabilities for Handover EBB mode | EBB support | Indicates whether the AMS has a capability to support EBB operation during handover 0b0: Not supported 0b1: Supported |
|  | Minimal HO_Reentry_Interleaving_Interval | The minimal HO_Reentry_Interleaving_Interval measured in frames. For MC HO capable AMS, this value shall be 0. Shall be included only if EBB support is set to 1 |

--------------------------------End of the proposed text-----------------------------------------

--------------------------------Start of the proposed text-----------------------------------------

In section 16.2.3.5 under AAI_SBC-RSP replace Table 686 with following table (entries that are crossed out are existing parameters that we suggest to remove)

Table 686—AMS Capabilities to be confirmed by ABS

| Nego parameters |  | value |
| --- | --- | --- |
| FFT size(2048/1024/512) |  | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports |
| Authorization policy support |  | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported |
| Supported Inter-RAT type |  | 1 indicates support, 0 indicates not support: bit #0: 802.11 bit #1: GERAN(GSM/GPRS/EGPRS) bit #2: UTRAN bit #3: E-UTRAN bit #4: CDMA 2000 bit #5-7: Reserved, set to zero |
| MIH Capability Supported |  | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. |
| ARQ parameters | ARQ_SN_MODULUS | the number of unique SN values |
|  | ARQ_WINDOW_SIZE | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |
|  | ARQ_SUB_BLOCK_SIZE | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  | ARQ_BLOCK_LIFETIME | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  | ARQ_RX_PURGE_TIMEOUT | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  | MAXIMUM_ ARQ_BUFFER_SIZE | The maximum ARQ buffer size Depend on AMS's memory |
| Capabilities for interference mitigation support | DL PMI coordination capability | 0: ABS is not DL PMI coordination capable 1: ABS is DL PMI coordination capable |
|  | DL collaborative multi-BS MIMO capability | 0: ABS is not DL collaborative multi-BS MIMO capable 1: ABS is DL collaborative multi-BS MIMO capable |
|  | DL closed-loop multi-BS macro diversity capability | 0: ABS is not DL closed-loop multi-BS macro diversity capable 1: ABS is DL closed-loop multi-BS macro diversity capable |
|  | UL PMI combination capability | 0: ABS is not UL PMI combination capable 1: ABS is UL PMI combination capable |

--------------------------------End of the proposed text-----------------------------------------

--------------------------------Start of the proposed text-----------------------------------------

In section 16.2.3.7 under AAI_REG-REQ replace Table 688 with following table (entries that are crossed out are existing parameters that we suggest to remove)

Table 688—AAI_REG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| M | AMS MAC address |  |  | 48 | This is used to derive security keys |  |
| O | AMS capability negotiation parameters | ARQ parameters | ARQ_SN_MODULUS | 10 | the number of unique SN values |  |
|  |  |  | ARQ_WINDOW_SIZE | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |  |
|  |  |  | ARQ_SUB_BLOCK_SIZE | TBD | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
|  |  |  | ARQ_BLOCK_LIFETIME | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
|  |  |  | ARQ_RX_PURGE_TIMEOUT | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |  |
|  |  |  | MAXIMUM_ ARQ_BUFFER_SIZE | TBD | The maximum ARQ buffer size Depend on AMS's memory |  |
| O |  | Multicarrier capabilities |  | 2 | 0b00: No MC modes 0b01: Basic MC mode 0b10: Multicarrier Aggregation 0b11: Multicarrier Switching |  |
| O |  | Capabilities for Zone switching mode | Zone Switch Mode Support |  | Indicates whether the AMS has a capability to maintain its data communication with the ABS in LZone while performing network reentry in MZone 0b0: Not supported 0b1: Supported |  |
|  |  |  | Reserved |  |  |  |
| O |  | LBS Capabilities | Capability for supporting A-GPS method | 1 | 0b0: no support 0b1: support |  |
| O |  | Capability for supporting A-GPS method |  | 1 | 0b0: no support 0b1: support |  |
| O |  | Capabilities for interference mitigation support | DL PMI coordination capability | 1 | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable |  |
| O |  |  | DL collaborative multi-BS MIMO capability | 1 | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is DL collaborative multi-BS MIMO capable |  |
| O |  |  | DL closed-loop multi-BS macro diversity capability | 1 | 0: AMS is not DL closed-loop multi-BS macro diversity capable 1: AMS is DL closed-loop multi-BS macro diversity capable |  |
| O |  |  | UL PMI combination capability | 1 | 0: AMS is not UL PMI combination capable 1: AMS is UL PMI combination capable |  |
| O |  | E-MBS capabilities |  | 3 | If Bit#0 is set to 1, it indicates E-MBS in Serving ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| O |  | Channel BW and Cyclic prefix |  | 15 | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) |  |
| O |  | frame configuration to support legacy(5MHz) |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | frame configuration to support legacy(10MHz) |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | OFDMA system support |  | 1 | If Bit#0 =1,it supports |  |
| O |  | MAX Tx Power |  | 24 | Bit 0-7: Maximum transmitted power for QPSK. Bit 8-15: Maximum transmitted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. |  |
| O |  | 3-step BR |  | 1 | If Bit#0 =1,it supports |  |
| O |  | dynamic ranging for HO |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Persistent Allocation support |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Group Resource Allocation support |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Co-located coexistence capability support |  | 1 | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class |  |
| O |  | HO trigger metric |  | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports |  |
| O | Capabilities for Handover EBB mode | EBB support |  | 1 | Indicates whether the AMS has a capability to support EBB operation during handover 0b0: Not supported 0b1: Supported |  |
| O |  | Minimal HO_Reentry_Interleaving_Interval |  |  | The minimal HO_Reentry_Interleaving_Interval measured in frames. For MC HO capable AMS, this value shall be 0. Shall be included only if EBB support is set to 1 |  |
| O | Capabilities for interference mitigation support | DL PMI coordination capability |  | 1 | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable |  |
| O |  | DL collaborative multi-BS MIMO capability |  | 1 | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is DL collaborative multi-BS MIMO capable |  |
| O |  | AMS Scanning Capability |  | 24 | This field indicates properties of the AMS that the ABS needs to know for scanning purposes: - Number of Center Frequencies AMS can scan during one scan iteration. - Maximum number of preambles per Center Frequency AMS can scan during one iteration. - Number of frames required to complete scanning of all preambles for a given frequency The length of the field is 3 Bytes, which are divided as follows: Bit# 0-4: Indicates the maximum number of preambles using the same center frequency that AMS can scan within one frame Bit# 5-7: Number of sub-frames the AMS requires to switch from the serving ABS to the first center frequency. This value is measured from the end of the last frame during which the AMS is connected to the S-ABS (and may have to receive/send data) until the beginning of the first frame in which a measurement of a neighboring ABS in a different center frequency can be performed. o 0b000 = 0 sub-frames o 0b111 = 7 sub-frames) Bits# 8-10: The number of sub-frames the AMS requires to switch between center frequencies. This is the time duration (in sub-frames) between a sub-frame in which a center frequency was scanned until the beginning of a sub-frame in which a new center frequency can be scanned. o 0b000 = 0 sub-frames o 0b111 = 7 sub-frames) Bits# 11-13: The number of sub-frames the AMS requires to switch from the last center frequency back to the serving ABS. This is the time duration (in sub-frames) between a frame in which a center frequency was scanned until the beginning of the first frame in which the AMS is available at the S-ABS and may receive/transmit data. o 0b000 = 0 sub-frames o 0b111 = 7 sub-frames) Bit# 14-15: Minimum supported scan duration o (0b000 = Indicates which classification/PHS options and SDU encapsulation the AMS supports no limit, 0b001= 1 sub-frame, 0b111= 7 sub-frames) Bit# 16-23: Maximum supported scanduration |  |
| O | Convergence sublayer capabilities | Classification/PHS options and SDU encapsulation support |  | TBD |  |  |
| O |  | Maximum number of classification rules |  | TBD | Maximum number of simultaneous admitted classification rules that a service flow supports. |  |
| O |  | ROHC support |  | 1 | 0: not support 1: support |  |
| O |  | PHS support |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS |  |
| O |  | Supported-IP-Versions IE |  | 2 | Indicates whether the AMS supports IPv4, IPv6 or both. 0b00: reserved 0b01: IPv4 support 0b10: IPv6 support 0b11: both IPv4 and IPv6 support |  |
| O | Host configuration capabilities and parameters | Host-Configuration-Capability-Indicator IE |  |  | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI_REG-RSP message. (One bit indicator) This shall be omitted if Requested-Host-Configurations IE is included in the message. |  |
| O |  | Requested-Host-Configurations IE |  |  | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI_REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. |  |
| O | Global carrier configuration change count |  |  |  | Indicates the AMS's last received value of Global carrier configuration change count of the network. If set to 0, it implies that AMS never received a Global carrier configuration information. |  |

--------------------------------End of the proposed text-----------------------------------------

--------------------------------Start of the proposed text-----------------------------------------

In section 16.2.3.8 under AAI_REG-RSP replace Table 689 with following table (entries that are crossed out are existing parameters that we suggest to remove)

Table 689—AAI_REG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes |  |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | STID |  |  |  | 48 | AMS identifier which the ABS assigns to the AMS in place of the temporary STID which has been transferred by AAI-RNG-RSP message. |  |
| M | CRID |  |  |  | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode |  |
| O | AMS capability negotiation parameters |  | ARQ parameters | ARQ_SN_MODULUS | 10 | the number of unique SN values |  |
|  |  |  |  | ARQ_WINDOW_SIZE | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |  |
|  |  |  |  | ARQ_SUB_BLOCK_SIZE | TBD | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
|  |  |  |  | ARQ_BLOCK_LIFETIME | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
|  |  |  |  | ARQ_RX_PURGE_TIMEOUT | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |  |
|  |  |  |  | MAXIMUM_ ARQ_BUFFER_SIZE | TBD | The maximum ARQ buffer size Depend on AMS's memory |  |
| O |  |  | Capabilities for interference mitigation support | DL PMI coordination capability | 1 | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable |  |
| O |  |  |  | DL collaborative multi-BS MIMO capability | 1 | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is DL collaborative multi-BS MIMO capable |  |
| O |  |  |  | DL closed-loop multi-BS macro diversity capability | 1 | 0: AMS is not DL closed-loop multi-BS macro diversity capable 1: AMS is DL closed-loop multi-BS macro diversity capable |  |
| O |  |  |  | UL PMI combination capability | 1 | 0: AMS is not UL PMI combination capable 1: AMS is UL PMI combination capable |  |
| O | Convergence sublayer capabilities |  | Classification/PHS options and SDU encapsulation support |  | TBD |  |  |
| O |  |  | Maximum number of classification rules |  | TBD | Maximum number of simultaneous admitted classification rules that a service flow supports. |  |
| O |  |  | ROHC support |  | 1 | 0: not support 1: support |  |
| O |  |  | PHS support |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS |  |
| O |  |  | Resource_Retain_Time |  | 16 | 0-65535: In units of 100 milliseconds Duration that the serving ABS shall retain the AMS context |  |
| O |  |  | Supported-IP-Versions IE |  | 2 | Indicates whether the AMS supports IPv4, IPv6 or both. 0b00: reserved 0b01: IPv4 support 0b10: IPv6 support 0b11: both IPv4 and IPv6 support |  |
| O | Host configuration capabilities and parameters |  | Host-Configuration-Capability-Indicator IE |  |  | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI_REG-RSP message. (One bit indicator) This shall be omitted if Requested-Host-Configurations IE is included in the message. |  |
| O |  |  | Requested-Host-Configurations IE |  |  | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI_REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. |  |
|  | Redirection Info | ABSID, preamble index and center frequency for one or more neighbor ABS |  |  |  | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry. |  |
| O | Global carrier configuration change count |  |  |  |  | Indicates the AMS's last received value of Global carrier configuration change count of the network. If set to 0, it implies that AMS never received a Global carrier configuration information. |  |

--------------------------------End of the proposed text-----------------------------------------