| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Correlation matrix updating with PMI |  |
| Date Submitted | 2010-4-30 |  |
| Source(s) | Bin Chen, Yang Tang Huawei Technologies Co., Ltd. | binchen@huawei.com ytang@huawei.com |
| Re: | Contribution in support of a comment on LB31a Section 16.3.7.2.5.5.1 Transformation codebook feedback mode |  |
| Abstract | The contribution proposes the text of transformation codebook to be included in the 802.16m AWD (P802.16m/D5). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD (P802.16m/D5). |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Correlation Matrix Updating with PMI

Bin Chen, Yang Tang

Huawei Technologies Co., Ltd.

- Introduction

In contribution C80216m-10_0318.doc[1], a correlation matrix calculation scheme and procedure for 8 Tx antennas is proposed. There will be performance gain to adopt this scheme, however there is some issues in that contribution. We would like to support the scheme with the issues corrected in this contribution, and provide the text amendment for 802.16m/D5[2].

We provide two optional schemes. The first one is same as [1] (just with some error correction), while the second one is to calculate the correlation matrix every m PMI report period. Since option 1 need very strong and complex calculation in AMS and ABS which will consume too much power especially in AMS, we prefer option 2.

- References

[1] IEEE 802.16m-08/0318, “Text Proposal for Transformation Codebook Refinement and Tracking on Section 16.3.7.2.5.5.1”

[2] IEEE 802.16m/D5, “802.16m Amendment Working Document (AWD)”

- Text proposal for inclusion in the 802.16m AWD/D4

Black: current text in D4

Blue: text to be added in D4

Insert the following text on page 632, line 38, of section 16.3.7.2.5.5.1:

Option 1:

-------------------------------  Text Start  ---------------------------------------------------------------

The total number of bits of feedback is 6 bits for 2 transmit antennas, 28 bits for 4 transmit antennas, and 120 bits for 8 transmit antennas. The AMS and ABS shall use the same transformation based on the correlation matrix fed back by the AMS.

In the case that the ABS has eight transmit antennas, the AMS and the ABS shall further follow the procedure described below, in order to utilize PMI (preferred Matrix Index) information reported by the MS, in addition to the information provided by the wideband correlation matrix R, achieving more efficient transmit codebook. The enhanced correlation matrix is indicated as Rt which shall take the place of R in the equation (249).

- Reset of the estimate of Rt:

When the AMS transmits the wideband correlation matrix R to the ABS, it shall set Rt = R. The ABS shall do likewise, i.e., upon receiving a wideband correlation matrix R reported by the AMS, it shall set Rt = R.

- Update of Rt:

Between two long-term feedback reports of the wideband correlation matrix R, both the AMS and the ABS shall simultaneously track and update/refine the correlation matrix Rt. Rt shall be updated each short reporting period of Best-M subbands PMI, which are as defined in the Feedback Allocation A-MAP IE. The update is performed using the equations below. In these equations, the index n is in units of the short reporting period of PMI, and Rt [n] represents the update of Rt after receiving the PMI information for the nth short reporting period. n = 0 corresponds to the reset event as defined in Step 1 above. The update equation is as

Rt[n]=αRt[n−1]+(1−α)(s=1Mvs[n−1]vsH[n−1])M,if a PMI for subband s reported at time instant n

--- Equation xxxx

where

- α is the forgetting factor, which is designed to track the mobility of mobile channel

- vs[n−1]is a codeword of the transformed codebook at the time instant n, where s is indicated by the PMI of subband s at time instant n.

-------------------------------  Text End  -----------------------------------------------------------------