| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Updating Broadcast Messages (16.2.3.11) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Sungcheol Chang, Soojung Jung, Eunkyung Kim, Jaesun Cha, Hyun Lee, Chulsik Yoon ETRI | E-mail: scchang@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “P802.16m/D5,” in response to the IEEE 802.16 Working Group Letter Ballot Recirculation #31a: Announcement, IEEE 802.16-10/0032 |  |
| Abstract | This provides the text modification for updating broadcast messages |  |
| Purpose | To be discussed and adapted by TGm for 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Updating Broadcast Messages (16.2.3.11)

Sungcheol Chang, Soojung Jung, Eunkyung Kim, Jaesun Cha, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

An AMS shall keep cell configuration information up to date on system configurations broadcast from a cell. The cell configuration is broadcast in the format of AAI_SCD message and SFH and the neighbor cells configuration is done in AAI_NBR-ADV message. These configurations need to be identified with change counts for ABS to notify AMS of their versions. Also an ABS shall notify AMSs of activation time when the cell configuration is applied.

This contribution provides text modifications for synchronization of cell configurations between AMS and ABS as followings:

- AAI_SCD message shall broadcast the applied version of system configuration. A “SCD applying offset” field is introduced because 16m/D5 has already applying offset concept for S-SFH version indication.

- When ABS detects that a neighbor ABS updates its cell configuration and an AMS need to be updated with it for handover, AAI_HO-CMD message carries the updated cell configuration in S-SFH for each target ABS.

Figure 1 shows that SCD applying offset indicates an active system configuration. The SCD applying offset field indicates the applied version of AAI_SCD message associated with either the current SCD change count or the previous SCD change count. When the SCD applying offset is 0, AAI_SCD message with the current SCD change count is applied. Otherwise, AAI_SCD message with the previous SCD change count is applied.

Figure 1 SCD Change Count and SCD applying offset

For handover preparation, an AMS may fail to synchronize neighbor cell configuration information to a target cell , as shown in Figure 2, when the target cell updates S-SFH. If ABS detects its mismatch during AMS initiated handover, updated SFH information of the target cell is carried by AAI_HO-CMD message. If the AMS synchronizes with cell configuration of target cell, the network re-entry procedure will be successful.

Figure 2 Change Counts in NBR-ADV message and S-SFH

# Text Proposal for the IEEE 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

--------------Start of Proposed Text---------------------------------------------------------------------

## 16.2.3.11 AAI_HO-CMD

## [Modify the text of the AAI_HO-CMD message as follows.]

Table 692—AAI_HO-CMD parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| …… | …… | …… | …… | …… |
| O | SFH delta information | Variable | Delta encoding with reference to the serving ABS. | May be included for each target ABS when Mode = 0b00 and SFH delta information is not included in the AAI_NBR-ADV message for the target ABS or when Mode = 0b00 and serving ABS updates S-SFH information of target ABS. |
| O | S-SFH change count | 4 | S-SFH change count of the reference for the included SFH delta information.. | Shall be included for each target ABS when SFH delta information is included |
| O | S-SFH activation superframe number | 12 | Superframe number when S-SFH information of target ABS associated with the S-SFH change count is used for applying the system configuration parameters | May be included for each target ABS when Mode = 0b00 and serving ABS updates S-SFH information of target ABS. |

## 16.2.3.30 AAI_SCD

## [Modify the text of the AAI_SCD message as follows.]

Table 717—AAI_SCD Message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Change Configuration ChangeCount | 4 | The value is increased whenever the contents of this message is changed The value rolls over from 0 to 15 | N.A. |
| M | SCD applying offset | 1 | Indicate the associated SCD change count in the next superframe after AAI_SCD message is decoded 0b0: Use AAI_SCD message associated with the current SCD change count 0b1: Use AAI_SCD message associated with (the current SCD change count -1) modulo 16 | N.A. |
| …… | …… | …… | …… | …… |

--------------End of Proposed Text ---------------------------------------------------------------------

# References

- IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” April 2009