| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the PGID_Info message for determination of paging carrier index (16.2.8.2.10.2) |  |
| Date Submitted | 2010-05-12 |  |
| Source(s) | Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI Shantidev Mohanty Intel Yuqin Chen, Lei Zhang ZTE Corporation Hyunjeong Kang Samsung | E-mail: sjjung@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31a:Announcement |  |
| Abstract | Proposed text to modify the PGID_Info message. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the PGID_Info message for determination of paging carrier index (16.2.8.10.2)

Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

Shantidev Mohanty

Intel

Yuqin Chen, Lei Zhang

ZTE Corporation

Hyunjeong Kang

Samsung

# 1. Introduction

According to the section 16.2.8.2.10.2, an AMS determines the paging carrier index using the DID and N. The value of N can be the number of carrier per PGID in an ABS that are used for transmitting paging message. Therefore, an idle AMS shall acquire the value of N and carrier index(s) used to support PGs at the preferred ABS.

We propose that an ABS shall provide the value of N per PG and paging group of carriers using the PGID_Info message.

# 2. Text Proposal

------------------------------------------------------------Start of the Text--------------------------------------------------------

[Modify Table 710. PGID_Info message as follows]

Table 710 PGID_Info message

| M/O | Attributes/Array of attributes | Size(bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Num_PGIDs | 2 | Indicate the number of PGID included in PGID_Info message 1..4 | N.A |
| M | PGArr [0…3] |  |  |  |
| M | A)PGID | 16 | Indicate paging group identifier where the BS belongs | N.A |
| M | B) m | 2 | Time domain hash parameter(1~4) used to determine the frame number of a superframe for paging message transmission of an idle mode AMS. Indicate m in the equation Npaging frame = AMS's deregistration identifier mod m to determine a predefined paging frame for the MS 1..4. | N.A |
| O | C)Paging carrier indication bitmap | Number of carriers in AAI_Global-Config L | This bitmap is used to specify if carriers are a paging carrier in the ABS or not. The size of paging carrier indication bitmap (L) equals to the number of carriers in the AAI_Global_Config message. The physical carrier index of each bit in bitmap corresponds to the ascending order of physical carrier index of the carriers in the AAI_Global_Config message. The value N used to determine the paging carrier index is equal to the number of bits set to 1. | Present only when an ABS supports multiple carrier operation and when PG_uniformity flag=0 |
| O | PG_uniformity flag | 1 | 1: All PGs have same paging indication bitmap. 0: Otherwise | Present only when an ABS supports multiple carrier operation and belongs to multiple paging groups |
| O | Paging carrier indication bitmap | L | This bitmap is used to specify if carriers are a paging carrier in the ABS or not. The size of paging carrier indication bitmap(L) equals to the number of carriers in the AAI_Global_Config message. The physical carrier index of each bit in bitmap corresponds to the ascending order of physical carrier index of the carriers in the AAI_Global_Config message. The value N used to determine the paging carrier index is equal to the number of bits set to 1. | Present only when an ABS supports multiple carrier operation and belongs to multiple paging groups with PG_uniformity flag=1 |

-------------------------------------------------------------End of the Text--------------------------------------------------------

# References

- IEEE P802.16m/D5, “Advanced Air Interface (Draft),” April. 2010.