| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the AAI_HO-CMD/IND message for MCHO supporting (19.2.8.2.9.2.2) |  |
| Date Submitted | 2010-05-10 |  |
| Source(s) | Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | E-mail: sjjung@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31a:Announcement |  |
| Abstract | Proposed text to modify the AAI_HO- CMD/IND message. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the AAI_HO-CMD/IND message for MCHO supporting (19.2.8.2.9.2.2)

Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# 1. Introduction

According to the section 16.2.8.2.9.2.2, an AMS with multicarrier capability could use the original primary carrier or another carrier for network entry. In this case, serving ABS needs to know which carrier (transceiver) of AMS is unavailable during network reentry. Therefore, AMS and ABS may align with each other for proper scheduling. but, there is no specific description about this information in AAI_HO-CMD or AAI_HO-IND message in current draft.  It is proposed to add some parameters to the existing AAI_HO-CMD/IND message for supporting multicarrier handover.

# 2. Text Proposal

------------------------------------------------------------Start of the Text--------------------------------------------------------

[Add the following row to table 690. AAI_HO-IND parameters]

| M/O | Attributes/Array of attributes | Size(bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Logical carrier index at serving | 3 | Recommended carrier index of the AMS to be used for network reentry to target ABS | May be included when AMS’s multicarrier capability=0b10 and HO Reentry Mode = 1 |

[Add the following row to table 692. AAI_HO-CMD parameters]

| M/o | Attributes/Array of attributes | Size(bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Logical carrier index at serving | 3 | Recommended carrier index of the AMS to be used for network reentry to target ABS | May be included when AMS’s multicarrier capability= 0b10 and HO Reentry Mode = 1 |

-------------------------------------------------------------End of the Text--------------------------------------------------------

# References

- IEEE P802.16m/D5, “Advanced Air Interface (Draft),” April. 2010.