| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | System Configuration Information Update during Sleep Mode in IEEE 802.16m (16.2.17) |  |
| Date Submitted | 2010-05-11 |  |
| Source(s) | Hyunjeong Kang Samsung Electronics Ming-Hung Tao ITRI Maruti Gupta, Elad Levy, Aran Bergman, Shantidev Mohanty Intel Corporation | E-mail: shantidev.mohanty@intel.com |
| Re: |  |  |
| Abstract | This contribution proposes methods using which AMS can have up-to-date system information in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 31a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

System Configuration Information Update during Sleep Mode in IEEE 802.16m (16.2.17)

Maruti Gupta, Elad Levy, Aran Bergman, Shantidev Mohanty

- Intel Corporation

- Introduction

Although current D3 specifies that 'AMS shall ensure that it has up-to-date system information for proper operation.", there is no specific mechanism defined to achieve this.

This proposal proposes methods using which AMS in sleep mode can ensure that it has up-to-date system information.

- Text proposal for inclusion in the P802.16m/D5

========================== Start of Proposed Text ==============================

[Editor’s Note: Modify the text in line 39 in page 371 in “16.2.17 Sleep Mode”]

The synchronization and system configuration information acquisition and verification may be done by AMS waking up at the some Super Frame Header just prior to the frame in which its listening window is located to ensure that the Super Frame number and the S-SFH Change Count are as expected.

[Editor’s Note: Add the following text after line sentence “AMS shall ensure that it has up-to-date system information for proper operation.” in line 39 in page 371 in “16.2.17 Sleep Mode”]

To ensure that the AMS has up-to-date system information, the following scenarios may occur during the sleep window of the sleep mode AMS .

Case 1: If there is no possible change on any S-SFH SP IE (i.e. S-SFH SP1 IE, SP2 IE and SP3 IE) during AMS’s sleep window (i.e., the sleep window terminates before the SFH where the S-SFH SP IE may change), then the AMS determines it does not have to decode this S-SFH SP IE during its sleep window.

Case 2: If there is only one S-SFH SP IE change possibility during the sleep window, then the AMS performs operations defined in Section 16.2.24 to receive this S-SFH SP IE during the sleep window at SFH just before the sleep window termination where this SP IE is transmitted.

Case 3: If there are multiple S-SFH SP IEs change possibility during the sleep window, then the AMS performs operations defined in Section 16.2.24 for each of these S-SFH SP IEs during the sleep window at SFHs just before the sleep window termination where these SP IEs are transmitted.

========================== End of Proposed Text ==============================