| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Methods to Determine Paging Parameters in the Legacy Operation Mode of IEEE 802.16m (16.2.18) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Shantidev Mohanty, Muthaiah Venkatachalam Intel Corporation | E-mail: shantidev.mohanty@intel.com |
| Re: |  |  |
| Abstract | This contribution proposes methods using which AMS in idle mode can determine paging parameters in the Legacy operation mode of IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 31a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Methods to Determine Paging Parameters in the Legacy Operation Mode of IEEE 802.16m (16.2.18)

Shantidev Mohanty, Muthaiah Venkatachalam

- Intel Corporation

- Introduction

In the 16m legacy mode operation defined in IEEE 802.16m standard, a 16m base station is attached to a legacy 16e network. In this case, the legacy network considers all the terminals as legacy terminals and hence uses the legacy protocols for various operations such as idle mode, paging etc.

In legacy networks based on IEEE 802.16e standard, the paging cycles and paging offsets for idle mode MSs are represented in terms of number of frames. On the other hand, in IEEE 802.16m these parameters are represented in terms of number of super-frames. It may be noted that one super-frame consists of four frames. Duration of each frame in IEEE 802.16e and IEEE 802.16m is 5 ms. Thus, the duration of super-frame = 4 * 5 = 20ms.

In legacy mode of operation the network entity responsible for idle mode operation of MSs , e.g., Paging Controller, assign the paging cycle and paging offset that are represented in terms of frames. However, the MS is attached to a base station that uses IEEE 802.16m specifications. Thus, the MS is aware about paging cycle and paging offset to be defined in terms of number of super-frames. Therefore there is a need for mechanisms using which the MS can determine its paging operational parameters, i.e., its paging listening interval in IEEE 802.16m legacy mode operation using the IEEE 802.16e paging parameters that it receives from the BS.

- Text proposal for inclusion in the P802.16m/D5

========================== Start of Proposed Text ==============================

[Editor’s Note: Add the following text after line 45 in page 387 in “16.2.18.2.2 Operation during paging unavailable interval”]

In Legacy mode operation when an 16m AMS is attached to a ABS that is connected to a legacy network, the paging parameters received by this 16m AMS belongs to legacy network. The AMS uses these legacy idle mode parameters (paging cycle and paging offset) to determine the corresponding parameters in the advanced system. Then it uses these advanced system paging parameters (paging cycle and paging offset) to determine the time that is remaining until the next regular paging listening interval for the preferred ABS. The calculated time until the next regular paging listening interval shall be the paging unavailable interval.

========================== End of Proposed Text ==============================