| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on target carrier information to Support FDM deployment of 16e/16m coexistence |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Yuqin Chen, Feng Xie, Lei Zhang, Yang Liu, Nan Li ZTE Corporation | chen.yuqin@zte.com.cn |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31a:Announcement |  |
| Abstract | This contribution proposes target carrier information to support FDM deployment of 16e/16m coexistence. |  |
| Purpose | To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on target carrier information to Support FDM deployment of 16e/16m coexistence

Yuqin Chen, Feng Xie, Lei Zhang, Yang Liu, Nan Li

ZTE Corporation

# 1. Introduction

To support backward compatibility, 16m proposed mixed mode ABS with TDM Lzone and Mzone. And all other functions, like HO and zone switch, were designed based on it. However, for operators, 16e and 16m could be also supported by different carriers. In this sense, some modifications are required accordingly.

We propose to add one new field “target carrier index” in current Zone Switch TLV to support the deployment mentioned above.

# 2. Text Proposal

------------------------------------------------------------Start of the Text-------------------------------------------

Suggested Remedy 1:

16.2.6.4 Handover between WirelessMAN-OFDMA Advanced and Reference Systems

16.2.6.4.1 Handover from WirelessMAN-OFDMA Reference to Advanced System

…

16.2.6.4.1.2.1 Zone Switch based Handover Procedure

…

[P286, L60: Insert one field in Zone Switch TLV]

The Zone Switch TLV may also include the following:

- Temporary STID for being used in Mzone

-Ranging initiation deadline: Valid time for Temporary STID. Shall be included if Temporary STID is included.

  Target carrier information of LZone: FA index of target LZone.

Suggested Remedy 2:

[P114 Table 693: Insert one row in Table 693-AAI_NBR-ADV parameters]

Table 693-AAI_NBR-ADV parameters

| M/O | Attributes / Array of attributes | Size(bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| O | NBR-ADV offset | 4 | The frame number offset the MOB-ADV is sent from the start point of frame counting. Unit: Frame. | It shall be included when the existence of neighbor R1 BS or Lzone of ABS are indicated in AAI_NBR-ADV. |
| O | NBR-ADV interval | 4 | The interval of MOB_NBR-ADV is sent in Lzone. Unit: Frame. | It shall be included when the existence of neighbor R1 BS or LZone of ABS are indicated in AAI_NBR-ADV. |
| O | Target carrier information of LZone | FA index | FA index of Lzone | It shall be included when LZone exists in another carrier different from the one of MZone. |

Suggested Remedy 3:

[P106 L18 Table692-AAI_HO-CMD parameters]

Table 692-AAI_HO-CMD parameters

| M/O | Attributes / Array of attributes | Size(bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| O | FA index | 8 |  | Shall be included for each target ABS when Mode=0b00 and for target LZone when LZone exists on a carrier different from the one of MZone. |
| … |  |  |  |  |